Reprinted by permission of Intel Corporation

Copyright 1989 Intel Corporation

INTEL

Table of Contents

CHAPTER 1

Introduction

1.1  Introduction to MCS BASIC-52 ........................        1

1.2  Getting Started .....................................        2

1.3  Getting Started - What Happens After Reset ..........        2

1.4  Definition of Terms .................................        4

1.5  What's the difference between Version 1.0 and Version 1.1    9

CHAPTER 2

Description of Commands

2.1  RUN .................................................       13

2.2  CONT ................................................       14

2.3  LIST ................................................       15

2.4  LIST # ..............................................       16

2.5  LIST @ ..............................................       17

2.6  NEW .................................................       18

2.7  NULL ................................................       19

CHAPTER 3

Description of Statements

3.1  RAM and ROM .........................................       21

3.2  XFER ................................................       22

3.3  PROG ................................................       23

3.4  PROG1 and PROG2 .....................................       24

3.5  FPROG, FPROG1, and FPROG2 ...........................       25

3.6  PROG3, PROG4, FPROG3, and FPROG4 (Version 1.1 only) .       26

3.7  PROG5, PROG6, FPROG5, and FPROG6 (Version 1.1 only0 .       27

CHAPTER 4

Description of Statements

4.1  BAUD ................................................       28

4.2  CALL ................................................       29

4.3  CLEAR ...............................................       30

4.4  CLEARS and CLEARI ...................................       31

4.5  CLOCK1 and CLOCK0 ...................................       32

4.6  DATA - READ - RESTORE ...............................       33

4.7  DIM .................................................       35

4.8  DO - UNTIL ..........................................       36

4.9  DO - WHILE ..........................................       37

4.10 END .................................................       38

4.11 FOR - TO - STEP - NEXT ..............................       39

4.12 GOSUB - RETURN ......................................       41

4.13 GOTO ................................................       43

1

4.14 ON GOTO - ON GOSUB ..................................       44

4.15 IF -THEN -ELSE ......................................       45

4.16 INPUT ...............................................       47

4.17 LET .................................................       49

4.18 ONERR ...............................................       50

4.19 ONEXT1 ..............................................       51

4.20 ONTIME ..............................................       52

4.21 PRINT ...............................................       54

4.22 PRINT# ..............................................       57

4.23 PH0., PH1., PH0.#, PH1.#  ...........................       58

4.24 PRINT@, PH0.@, PH1.@ (Version 1.1 Only) .............       59

4.25 PUSH ................................................       60

4.26 POP .................................................       61

4.27 PWM .................................................       62

4.28 REM .................................................       63

4.29 RETI ................................................       64

4.30 STOP ................................................       65

4.31 STRING ..............................................       66

4.32 UI1 AND UI0 .........................................       67

4.33 UO1 and UO0 .........................................       68

4.34 IDLE (Version 1.1 only) .............................       69

4.35 RROM (Version 1.1 only) .............................       70

4.36 LD@ and ST@ (Version 1.1 only) ......................       71

4.37 PGM (Version 1.1 only) ..............................       72

CHAPTER 5

Description of Arithmetic/Logical Operators and Expressions

5.1  Dual Operand (DYADIC) Operators .....................       74

5.2  Unary Operators .....................................       76

5.2.1  General Purpose ...................................       76

5.2.2  Log Functions .....................................       78

5.2.3  Trig Functions ....................................       78

5.3  Understanding Precedence of Operators ...............       80

5.4  How Relational Expressions Work .....................       81

CHAPTER 6

Description of String Operators

6.1  What are Strings? ...................................       82

6.2  The ASC Operator ....................................       83

6.3  The CHR Operator ....................................       85

CHAPTER 7

Special Operators

7.1  Special Function Operators ..........................       86

7.2  Examples of Manipulating Special Function Operators .       94

7.3  System Control Values ...............................       95

CHAPTER 8

2

Error Messages, Bells, Whistles, and Anomalies

8.1  Error Messages ......................................       96

8.2  Disabling Control-C .................................      100

8.3  Implementing "Fake DMA" .............................      101

8.4  Run Trap Option (Version 1.1 only) ..................      102

8.5  Anomalies ...........................................      103

CHAPTER 9

Assembly Language Linkage

9.1  Overview ............................................      104

9.2  General Purposes Routines ...........................      106

9.3  Unary Operators .....................................      113

9.4  Special Operators ...................................      115

9.5  Dual Operand Operators ..............................      118

9.6  Added Link Routines to Version 1.1 ..................      122

9.7  Interrupts ..........................................      129

9.8  I/O Resource Allocation .............................      131

CHAPTER 10

System Configuration

10.1  Memory/Hardware Configuration .......................      132

10.2  EPROM Programming Configuration/Timing ..............      135

10.3  Serial Port Implementation ..........................      136

CHAPTER 11

Reset Options (Version 1.1 only) .........................      145

CHAPTER 12

Command/Statement Extensions (Version 1.1 only) ..........      153

CHAPTER 13

Mapping User Code Memory (Version 1.1 only) ..............      159

APPENDIX A

1.1  Memory Usage (Version 1.0 and version 1.1) ...........      162

1.2  Using the PWM Statement ..............................      170

1.3  Baud Rates and Crystals ..............................      174

1.4  Quick Reference ......................................      176

1.5  Instruction Set Summary ..............................      183

1.6  Floating Point Format ................................      184

1.7  Storage Allocation ...................................      185

1.8  Format of an MCS BASIC-52 program ....................      188

1.9  Answers to a Few Questions ...........................      190

1.10 pin-out List .........................................      192

1.11 8052AH Special Function Registers ....................      193

1.12 References ...........................................      199

3

APPENDIX B

Instruction Set Summary ..................................      200

INDEX  .....................................................      213

4

1CHAPTER 1

INTRODUCTION

1.1  INTRODUCTION TO MCS BASIC-52

Welcome to MCS BASIC-52.   This program functions as a BASIC interpreter

occupying  8K  of  ROM in INTEL's 8052AH microcontroller.   MCS BASIC-52

provides most of the features of "standard" BASIC,  plus many additional

features  that  apply to control environments and to the architecture of

the 8052AH.

The design goal of MCS BASIC-52 was to develop a software  program  that

would make it easy for a hardware/software designer to interact with the

8052  device;  but,  at the same time not limit the designer to the slow

and sometimes awkward constructs of BASIC.   This program is not a "toy"

like many of the so called TINY BASICS.   It is a powerful software tool

that can significantly reduce the design time of  many  projects.    MCS

BASIC-52  is  ideal for so called embedded systems,  where terminals are

not attached to system,  but the systems controls and manipulates equip-

ment and data.

MCS BASIC-52 offers many unique hardware and software features,  includ-

ing the ability to store and execute the user program out of  an  EPROM,

the  ability  to  process  interrupts  within  the constructs of a BASIC

program, plus an accurate real time clock.  In addition,  the arithmetic

routines and I/O routines contained in MCS BASIC-52 can be accessed with

assembly  language CALL routines.  This feature can be used to eliminate

the need for the user to write these  sometimes  difficult  and  tedious

programs.

All  of  the above are covered in this document.   This is NOT a "How to

Write Basic Programs" manual.  Many excellent texts on this subject have

been produced.  Your local computer store can recommend many such texts.

The descriptions of may of the statements in this manual involve  rather

detailed discussions that relate to interfacing MCS BASIC-52 to assembly

language programs.  If the user is not interested in using assembly  lan-

guage  with  MCS  BASIC-52 these discussions maybe ignored.   If you are

only interested in programming the MCS BASIC-52 device in BASIC, you can

treat all statements the same way they would be in  any  standard  BASIC

interpreter.

In  reading this manual,  you will find that information may be repeated

two or three times.  This is not an accident.   Years of experience have

proven  that one of the most frustrating experiences one encounters with

manuals is trying to find a particular piece  of  information  that  the

reader knows is in the manual, but can't remember where.

--------------------

1.

1

1.2 Getting Started

If you are like most engineers,  technicians,  hobbyist and humans,  and

don't like to read manuals,  this section is for you.   The  purpose  of

this  section  is  to get you off on the right foot.   If you are in the

High Anxiety Mode and you just want to see if the darn chip works,  wire

the  device  in  the  minimum hardware configuration as suggested in the

Hardware Configuration chapter of this manual,  apply power,  and  watch

what  happens.   NOTHING!   That's because after power is applied to the

MCS                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                favor,  read this section and touch the space bar before  you

call your local Intel Field Applications Engineer.  We received a number

of  questions  asking  how  the  AUTO-BAUD search routine worked.   As a

result this routine is listed in Chapter 11 of this manual.

1.3 WHAT HAPPENS AFTER RESET?

After RESET, MCS BASIC-52:

1) Clears the INTERNAL 8052AH memory

2) Initializes the internal registers and pointers

3) Tests, clears, and sizes the EXTERNAL memory

BASIC then assigns the top of EXTERNAL RANDOM ACCESS MEMORY to the  SYS-

TEM  CONTROL  VALUE-mtop and uses this number as the random number seed.

BASIC assigns the default crystal value, 11.0592 Mhz, to the SYSTEM CON-

TROL VALUE-XTAL and uses this default value to calculate all time depend-

ent functions,  such as the EPROM programming timer  and  the  interrupt

driven REAL TIME CLOCK.   Finally, BASIC checks external memory location

8000H to see if the baud rate information is stored.   If the baud  rate

is  stored,  MCS  BASIC-52  initializes  the  baud  rate  generator (the

8052AH's SPECIAL FUNCTION  REGISTER-T2CON)  with  this  information  and

signs on.   If it isn't stored, BASIC interrogates the serial port input

and waits for a space character to be typed.   This sounds like  a  lot,

but on the 8052AH, it doesn't take much time.

2

1.3 WHAT HAPPENS AFTER RESET?

MCS BASIC-52 initializes the 8052AH's Special Function Registers,  TMOD,

TCON, and T2CON with the following values:

TCON - 244 (OF4H)

TMOD -  16 (10H)

T2CON - 52 (34H)

After Reset, the console device should display the following:

*MCS-51(tm) BASIC Vx.x*

READY

To see if everything is OK after Reset, type the following:

>PRINT XTAL, TMOD, TCON, T2CON

(BASIC should respond)

11059200 16 244 52

If it does,  everything is working properly.   If it does not make  sure

that the external memory,  the serial port,  and the oscillator are con-

nected and working.  Hardware debug begins here.

In the Appendix of this manual is a QUICK REFERENCE GUIDE.   It provides

a short description of all of the COMMANDS and STATEMENTS implemented in

MCS BASIC-52.  You might want to use this section to gain a quick under-

standing  of  the  MCS BASIC-52 software package.   Those of you who are

familiar with the BASIC language will notice that most of the STATEMENTS

and COMMANDS used in MCS BASIC-52 are  "standard,"  so  getting  started

should not be a problem.

3

1.4 DEFINITION OF TERMS:

COMMANDS:

MCS  BASIC-52 operates in two modes,  the COMMAND or direct mode and the

interpreter or RUN mode.

MCS BASIC-52 Commands can only be entered when the processor is  in  the

COMMAND or direct mode.

MCS  BASIC-52  takes  immediate action after a command has been entered.

This document will use the terms RUN MODE and COMMAND MODE to  refer  to

the two different modes of operation.

STATEMENTS:

A BASIC program is comprised of statements.  Every statement begins with

a line number, followed by the statement body, and terminated with a Car-

riage Return (cr), or a colon (:) in the case of multiple statements per

line.   Some statements can be executed in the COMMAND MODE, others can-

not.   The DESCRIPTION OF STATEMENTS section of  this  manual  describes

whether  a  statement can be executed in the COMMAND mode or only in the

RUN mode.

There are three general types of statements  in  MCS  BASIC-52:  ASSIGN-

MENTS,  INPUT/OUTPUT, and CONTROL.  The DESCRIPTION OF STATEMENT section

of this manual explains what type is associated with each statement.

-    EVERY line in a program must have a statement line  number  ranging

between 0 and 65535 inclusive.

-    Statement numbers are used by BASIC to order the program statements

sequentially.

-    In any program, a statement number can be used only once.

-    Statements  need  not be entered in numerical order,  because BASIC

will automatically order them in ascending order.

-    A statement may contain no more than 72 characters in  Version  1.0

and no more than 79 in Version 1.1.

-    Blanks  (spaces)  are  ignored by BASIC and BASIC automatically in-

serts blanks during LIST.

-    More than one statement can be put on a line,  if  separated  by  a

colon (:), but only one statement number is allowed per line.

4

FORMAT STATEMENTS:

Format statements may only be used within the PRINT STATEMENT.  The for-

mat statements include TAB([expr]), SPC([expr]), USING(special symbols),

and CR (carriage return with no line feed).  Details of the format state-

ments  are provided in the description of the PRINT STATEMENT section of

this manual.

5

1.4 DEFINITION OF TERMS

DATA FORMAT:

The range of numbers that can be represented in MCS BASIC-52 is:

+             +

- IE - 127 to - .99999999E + 127.

There are eight digits of significance in MCS BASIC-52.  Numbers are in-

ternally rounded to fit this precision.   Numbers maybe entered and dis-

played in four formats: integer,  decimal, hexadecimal, and exponential.

Example:  129,  34.98,  0A6EH,  1.23456+3

INTEGERS:

In MCS BASIC-52,  integers are numbers that ranges from 0  to  65535  or

0ffffh.    All  integers can be entered in either decimal or hexadecimal

format and all hexadecimal numbers must begin with a valid  digit  (e.g.

the  number  A000H must be entered 0A000H).   When an operator,  such as

.AND. requires an integer,  MCS BASIC-52 will truncate the fraction por-

tion of number so it will fit the integer format.  All line numbers used

by MCS BASIC-52 are integers.   This document will refer to integers and

line numbers, respectively in the following manner:

[integer] - [In num]

NOTE - Throughout this document the brackets [] are used only  to  indi-

cate an integer,  constant,  etc.   They are NOT entered when typing the

actual number or variable.

CONSTANTS:

A constant is a real number that  ranges  from  +-  1  E  -  127  to  +-

.99999999E+127.   A constant,  of course, can be an integer.  This docu-

ment will refer to constants in the following manner:

[const]

OPERATORS:

An operator performs a pre-defined operation on  variables  and/or  con-

stants.    Operators  require  either one or two operands.   Typical two

operand or dyadic operators include ADD (+), SUBTRACT (-), MULTIPLY (*),

and DIVIDE (/).   Operators that require  only  one  operand  are  often

referred  to as UNARY OPERATORS.   Some typical UNARY OPERATORS are SIN,

COS, and ABS.

6

1.4 DEFINITION OF TERMS

VARIABLES:

In version 1.0 of MCS BASIC-52 a variable could be defined as  either  a

letter,  (i.e.A,X,I),  a letter followed by a number, (i.e. Q1,T7,L3), a

letter followed by a ONE DIMENSIONED  expression,  (i.e.  J(4),  G(A+6),

I(10*SIN(X))), or a letter followed by a number followed by a ONE DIMEN-

SIONED expression (i.e.  A1(8),  P7 (DBY(9)),  W8(A+B).   In Version 1.1

variables can be defined in the same manner as in Version  1.0,  however

variables may also contain up to 8 letters or numbers including the  un-

derline character.  This permits the user to use a more descriptive name

for a given variable.  Examples of valid variables in Version 1.1 of MCS

BASIC-52 are as follows:

FRED  VOLTAGE1  I_I1  ARRAY(ELE_1)

When  using  the expanded variable names available in Version 1.1 of MCS

BASIC-52 it is important to note that 1) It takes longer for MCS  BASIC-

52  to process these expanded variable names and 2) The user may not use

any keyword as part of variable name (i.e.  the variables TABLE and DIET

could  not  be used because TAB and IE are reserved words).   BAD SYNTAX

ERRORS will be generated if the user attempts to define  a variable that

contains a reserved word.

Variables that include a ONE DIMENSIONED  expression  (expr)  are  often

referred to as DIMENSIONED or ARRAYED variables. Variables that only in-

volve  a  letter  or  a letter and a number are called SCALAR variables.

The details concerning DIMENSIONED variables are covered in the descrip-

tion of the STATEMENT ROUTINE DIM.   This document will refer  to  VARI-

ABLES as:

[var].

MCS BASIC-52 allocates variables in a "static" manner.   That means each

time a variable is used,  BASIC allocates a portion of memory (8  bytes)

specifically for that variable.  This memory cannot be de-allocated on a

variable by variable basis.   That means if you execute a statement like

Q=3, later on you cannot tell BASIC that the variable Q no longer exists

so, please "free up" the 8 bytes of memory that belong to Q.  Sorry,  it

doesn't work this way.   The only way the user can clear the memory that

is allocated to variables is to execute a CLEAR STATEMENT.   This State-

ment "frees" all memory allocated to variables.

IMPORTANT NOTE:

Relative  to  a  dimensioned variable,  it takes MCS BASIC-52 a lot less

time to find a scalar variable.   That's because there is no  expression

to evaluate in a scalar variable.   So,  if you want to make program run

as fast as possible ,  use dimensioned variables only when you have  to.

Use scalars for intermediate variables,  then assign the final result to

a dimensioned variable.

7

EXPRESSIONS:

An expression is a logical mathematical formula that involves  OPERATORS

(both unary and dyadic),  CONSTANTS,  and VARIABLES.  Expressions can be

simple   or   quite   complex,   i.e.   12*EXP(A)/100,    H(1)+55,    or

(SIN(A)*SIN(A)+COS(A))/2.    A  "stand alone" variable [var] or constant

[const] is also considered an EXPRESSION.   This document will refer  to

EXPRESSION as:

[expr].

1.4 DEFINITION OF TERMS

RELATIONAL EXPRESSIONS:

Relational expressions involve the operators EQUAL (=),  NOT EQUAL (<>),

GREATER THAN (>), LESS THAN (<),  GREATER THAN OR EQUAL TO (>=) and LESS

THAN OR EQUAL TO (<=).   They are used in control statements to "test" a

condition (i.e. IF A < 100 THEN ...).  Relational expressions ALWAYS RE-

QUIRE TWO OPERANDS.   This document will refer to RELATIONAL EXPRESSIONS

as:

[rel expr].

SPECIAL FUNCTION OPERATORS:

Virtually all of the special function registers on the 8052AH can be ac-

cessed  by  using  the  special function operators.   The exceptions are

PORTS 0,  2 and 3 and and non-I/O associated registers such as  ACC,  B,

and PSW.   Other SPECIAL FUNCTION OPERATORS are XTAL and TIME.   Details

of the SPECIAL FUNCTION OPERATORS are covered  in  the  section  SPECIAL

FUNCTION OPERATORS.

SYSTEM CONTROL VALUES:

The system control values include the following:  LEN (which returns the

length of the program), FREE (which designates how many bytes of RAM are

not  used  that  are  allocated  to BASIC),  and MTOP (which is the last

memory location that is assigned to BASIC).   Details of the system con-

trol values are covered in the section SYSTEM CONTROL VALUES.

1.4 DEFINITION OF TERMS

STACK STRUCTURE:

8

MCS  BASIC-52  reserves  the  first 512 bytes of EXTERNAL DATA MEMORY to

implement two "software" stacks.   These are the control stack  and  the

arithmetic  stack or ARGUMENT STACK.   Understanding how the stacks work

in MCS BASIC-52 is NOT NECESSARY if the user wishes only to  program  in

BASIC.   However,  understanding the stack structure is necessary if the

user wishes to link MCS BASIC-52 to ASSEMBLY  language  routines.    The

details  of how to link to assembly language are covered in the ASSEMBLY

LANGUAGE LINKAGE section of this manual.

CONTROL STACK - The control stack occupies locations  96  (60H)  through

254 (0FEH) in external ram memory.  This memory is used to store all in-

formation  associated with loop control (i.e.  DO-WHILE,  DO-UNTIL,  and

FOR-NEXT) and basic subroutines (GOSUB).   The stack is  initialized  to

254 (0FEH) and "grows down."

ARGUMENT  STACK  -  The  ARGUMENT  STACK  occupies  locations 301 (12DH)

through 510 (1FEH) in external ram memory.   This stack stores all  con-

stants  that  MCS BASIC-52 is currently using.   Operations such as ADD,

SUBTRACT,  MULTIPLY,  and DIVIDE always operate on the first two numbers

on the ARGUMENT STACK and return the result to the ARGUMENT STACK.   The

argument stack is initialized to 510 (1FEH) and  "grows  down"  as  more

values  are  placed  on the ARGUMENT STACK.   Each floating point number

placed on the ARGUMENT STACK requires 6 BYTES of storage.

INTERNAL STACK - The stack  pointer  on  the  8052AH  (SPECIAL  FUNCTION

REGISTER,  SP)  is initialized to 77 (4DH).   The 8052AH's stack pointer

"grows up" as values are placed on the stack.   In MCS BASIC-52 the user

has  the  option  of  placing the 8052AH's STACK POINTER anywhere (above

location 77) in internal memory.   The details on how  to  do  this  are

covered in the ASSEMBLY LANGUAGE LINKAGE section of this manual.

LINE EDITOR:

MCS  BASIC-52 contains a minimum level line editor.   Once a line is en-

tered the user may not change  the  line  without  re-typing  the  line.

However,  it  is  possible  to  delete characters while a line is in the

process of being entered.   This is done by entering a RUBOUT or  DELETE

character (7FH).  The RUBOUT character will cause the last character en-

tered to be erased from the text input buffer.  Additionally, a control-

D will cause the entire line to be erased.  In Version 1.1 of MCS BASIC-

52,  Control-Q  (X-ON) and Control S (X-OFF) recognition have been added

to the serial port.   The user cautioned  not  to  accidentally  type  a

Control-S  when  entering  information  because the MCS BASIC-52 will no

longer respond to the console device.   Control-Q is used to  bring  the

console device back to life after Control-S is typed.

NOTE-In this document a carriage return is indicated by the symbol (cr).

The carriage return is the RETURN key on most keyboards.

1.5 WHAT'S THE DIFFERENCE BETWEEN V1.0 AND V1.1

9

Thanks  to feedback from many of the users of MCS BASIC-52,  a number of

changes and additions have been made to  Version  1.1.    All  of  these

changes and additions were made to enhance the usefulness of the product

and  yet retain 100% compatibility,  well almost 100% compatibility with

the original version.  To make things simple, all of the changes will be

mentioned here and reference will be provided as to where the reader  of

this manual may obtain more information about the change or addition.

The  only  change that has been made to V1.1 that is not compatible with

V1.0 is with the IF_THEN_ELSE STATEMENT when used with  multiple  state-

ments per line.   In V1.0,  the following two examples would function in

the same manner.

EXAMPLE 1:

10 IF A=B THEN C=A :  A=A/2  :  GOTO 100

20 PRINT A

EXAMPLE 2:

10 IF A=B THEN C=A

12 A=A/2

14 GOTO 100

20 PRINT A

They function in the same manner because V1.0 treats the  delimiter  (:)

exactly the same as a carriage return (cr) in every case.  However, V1.1

executes the remainder of the line if and only if the test A=B proves to

be true.   This means in EXAMPLE 1 IF A did equal B, V1.1 would then set

C=A, then set A=A/2, then execute line 100.  If A did not equal B,  V1.1

would  then PRINT A and ignore the statements C=A:A=A/2:GOTO 100.   V1.1

will execute EXAMPLE 2 exactly the same way as V1.0.   This same logical

interpretation holds true for the ELSE statement as well.   This example

dictates a simple rule for maintaining  IF_THEN_ELSE  compatibility  be-

tween  the  two  versions.    IF  THE  DELIMITER  (:)  IS NOT USED IN AN

IF_THEN_ELSE STATEMENT,  V1.0 AND V1.1 WILL TREAT THE STATEMENTS IN  THE

SAME MANNER!!

This  change  was  made because most users of MCS BASIC-52 felt that the

V1.1 interpretation of this statement was more useful because fewer GOTO

statements need be employed in a typical program.

Additionally, V1.1 accepts inputs in either lower or upper case, whereas

V1.0 converted lower case to upper case.   V1.1  will  however,  convert

keywords  from lower case to upper case during the LISTing of a program.

Finally,  MCS BASIC-52 V1.1 runs between 2% and 10%  faster  than  V1.0.

Typically, this should not cause any problems.

As far as the user is concerned, these are the only changes that may af-

fect the operation of a typical program.  Now, on to the additions.

10

1.5 WHAT'S THE DIFFERENCE BETWEEN V1.0 AND V1.1

ADDITIONS TO MCS BASIC-52 V1.1:

- X-ON (control Q) and X-OFF (control S) have been added.   These permit

the user to "stop" (control S) and start  (control  Q)  the  display  of

characters  during  a  LIST  or  PRINT.     This  feature  also  permits

synchronization with external I/O (input/output)  devices.    The  X-OFF

(control  S)  functions  on a line by line basis,  not on a character by

character basis.

- Five new statements have been added.   These include IDLE,  LD@,  ST@,

PGM, and RROM.  Details of these statements are listed under the DESCRIP-

TION OF STATEMENTS section of this manual.

- Six new RESET options have been provided.  They permit the user to as-

sign the top of memory (MTOP) during reset,  and allow the user to write

specific RESET  programs  in  assembly  language.    Additionally,  they

provide  an  option  where  the memory WILL NOT be cleared during RESET.

More information on the  specific  RESET  OPTIONS  is  detailed  in  the

DESCRIPTIONS OF EPROM FILE COMMANDS under PROG1,  PROG2,  PROG3,  PROG4,

PROG5, and PROG6 COMMANDS and in Chapter 11 of this manual.

- The Timing of the EPROM programming algorithm has  been  significantly

relaxed  between  the various strobes required for the EPROM programming

function.   This relaxed timing permits the user to program devices such

as  the 8751H and the 8748/9 using the EPROM programming capabilities of

the MCS BASIC-52 device.   Details of the timing changes are in  Chapter

10 of this manual.

- During EPROM programming, the INTO/DMA REQUEST pin of the MCS BASIC-52

device is treated as a ready input pin.  This allows for a simple direct

connection  to  EEPROM  devices  such  as  the 2817A.   For normal EPROM

programming,  INTO must be  kept  high  or  the  programming  hangs  up.

Details  concerning  the  use  EEPROMS  with the MCS BASIC-52 device are

provided in Chapter 10 of this manual.

- A RUN TRAP option has been provided.  This option traps the MCS BASIC-

52 interpreter in the program RUN mode and will not permit the  user  to

exit  this  mode.   Details of this option are covered in Chapter 8.4 of

this manual.

- A user STATEMENT/COMMAND expansion option has been provided.  This per-

mits the user to easily add new or custom STATEMENTS and COMMANDS to MCS

BASIC-52.   Details of this option are covered in  Chapter  12  of  this

manual.

1.5 WHAT'S THE DIFFERENCE BETWEEN V1.0 AND V1.1

11

ADDITIONS TO MCS BASIC-52 V1.1:

-  A  number  of  new  assembly  language user OP BYTES have been added.

These permit the user to make better use of the STATEMENT/COMMAND expan-

sion option previously described.   Details of these new  OP  BYTES  are

presented in Chapter 9.6 of this manual.

-  The  length of the input buffer has been increased from 72 characters

to 79 characters and the ERROR: LINE TOO LONG has been eliminated.   In-

stead,  when  the cursor reaches the 79th position a bell character will

be echoed everytime the user attempts to enter another character.

-  A new variation on the PRINT (including PH0.  and PH1.) and the  LIST

statements have been added.  This new option is evoked with an @ charac-

ter  (EXAMPLE:PRINT  @  or LIST @ and permits the user to write specific

output drivers for these statements and commands.   When the @ PRINT  or

LIST is evoked,  MCS BASIC-52 CALLS external code memory location 403CH.

The user must put the specific output driver in  this  location.    More

details  of  this option is in Description of Statements section of this

manual.

-  The control stack has been made more "forgiving."   This  means  that

the  user  can  execute a GOSUB to a subroutine that contains a FOR-NEXT

loop and return from the  subroutine  without  completing  the  FOR-NEXT

loop.    Version  1.0  would  yield  a  C-STACK  ERROR  under these cir-

cumstances, V1.1 yields no error.

- The question mark character?   is interpreted  as  a  PRINT  statement

(EXAMPLE:(PRINT 10+20 is the same as ? 10+20).  The symbols P. remains a

shorthand notation for PRINT just as in V1.0.

- The FOR-NEXT statement can be executed in the direct mode.   This lets

the user write short routines in the DIRECT MODE to,  for example,  dis-

play a region of memory (EXAMPLE:FOR I=200H to 210H: PHO.XBY(I):NEXT I)

- Variables can be up to 8 characters in length, however, only the first

character, the last character, and the total number of characters are of

significance.    This  lets  the user better describe variables that are

used in a program.   Chapter 1.4 details the limitations on the expanded

variables in Version 1.1.

1.5  WHAT'S THE DIFFERENCE BETWEEN V1.0 AND V1.1

ADDITIONS TO MCS BASIC-52 V1.1:

-  The  CALL  statement  vectors to locations 4100H through 41FFH if the

CALL integer is between 0 and 7FH inclusive.   This means  that  CALL  0

will vector to location 4100H, CALL 1 to location 4102H, CALL 2 to loca-

tion 4104H, etc.  This permits the user to easily generate assembly lan-

12

guage  CALL  tables  by  using  simple integers with the CALL statement.

Anyway,  CALL 0 through CALL 1FFFH was not too useful because these num-

bers vectored into the MCS BASIC-52 ROM.

-  The  error  message  anomaly  for an invalid line number on a GOTO or

GOSUB STATEMENT has been eliminated on V1.1 of MCS BASIC-52.   The  cor-

rect line number is now processed and displayed by the error processor.

-  The  FOR-TO-{STEP}-NEXT statement can be executed in the COMMAND MODE

in version 1.1 of MCS BASIC-52.   Additionally,  the NEXT statement does

not  require  a variable in version 1.1.   Details of these features are

covered in the Description of Statements section of this manual.

_ The REM statement can be executed in the COMMAND MODE.  If the user is

employing some type of UPLOAD/DOWNLOAD routine  with  a  computer,  this

lets  the  user insert REM statements,  without line numbers in the text

and not download them to the MCS BASIC-52 device.   This helps  to  con-

serve memory.

- Version 1.1 is also a little less "crashable" than version 1.0.   This

is due to a more extensive "type checking" on control transfer  routines

(i.e. GOTO, GOSUB).

13

CHAPTER 2

Description of Commands

2.1 DESCRIPTION OF COMMANDS

COMMAND: RUN(cr)

ACTION TAKEN:

After  RUN(cr)  is typed all variables are set equal to zero,  all BASIC

evoked interrupts are cleared and  program  execution  begins  with  the

first line number of the selected program.  the RUN command and the GOTO

statement  are  the  only way the user can place the MCS BASIC-52 inter-

preter into the RUN mode from the COMMAND mode.  Program execution maybe

terminated at any time by typing a control-C on the console device.

VARIATIONS:

Unlike some Basic interpreters that allow a line number  to  follow  the

RUN  command (i.e.,RUN 100),  MCS BASIC-52 does not permit such a varia-

tion on the RUN command.   Execution always begins with the  first  line

number.    To  obtain the same functionality as the RUN[1n num] command,

use the GOTO[1n num] statement in the direct mode.  SEE STATEMENT GOTO.

EXAMPLE:

>10 FOR I=1 TO 3

>20 PRINT I

>30 NEXT I

>RUN

1

2

3

READY

>

14

2.2 DESCRIPTION OF COMMANDS:

COMMAND: CONT(cr)

ACTION TAKEN:

If a program is stopped by typing a control-C on the console  device  or

by  execution  of  a  STOP  statement,  you  can resume execution of the

program by typing CONT(cr).  Between the stopping and the re-starting of

the program you may display the values of variables or change the values

of variables.  However,  you may NOT CONTinue if the program is modified

during the STOP or after an error.

VARIATIONS:

None.

EXAMPLE:

>10 FOR I=1 to 10000

>20 PRINT I

>30 NEXT I

>RUN

1

2

3

4

5    -(TYPE CONTROL-C ON CONSOLE)

STOP - IN LINE 20

READY

>PRINT I

6

>I=10

>CONT

10

11

12

15

2.3 DESCRIPTIONS OF COMMANDS:

COMMAND: LIST(cr)

ACTION TAKEN:

The  LIST(cr)  command  prints the program to the console device.   Note

that the list command "formats" the program in an easy to  read  number.

Spaces  are  inserted  after the line number and before and after state-

ments.  This feature is designed to aid in the debugging of MCS BASIC-52

programs.   The "listing" of a program may be terminated at  anytime  by

typing a control-C on the console device.

VARIATIONS:

Two variations of the LIST COMMAND are possible with MCS BASIC-52.  They

are:

LIST [1n num] (cr) and

LIST [1N NUM] - [1N NUM] (cr)

The first variation causes the program to be printed from the designated

line  number (integer) to the end of the program.   The second variation

causes the program to be printed from the first line number (integer) to

the second line number (integer).   NOTE -the two line numbers  MUST  BE

SEPARATED BY A DASH -.

EXAMPLE:

READY

>LIST

10 PRINT "LOOP PROGRAM"

20 FOR I=1 TO 3

30 PRINT I

40 NEXT I

50 END

READY

>LIST 30

30 PRINT I

40 NEXT I

50 END

READY

>LIST 20-40

20 FOR I=1 TO 3

30 PRINT I

40 NEXT I

16

2.4 DESCRIPTIONS OF COMMANDS

COMMAND:LIST#(cr)

ACTION TAKEN:

The  LIST#(cr)command  prints the program to the LIST device.   The BAUD

rate to this device must be  initialized  by  the  STATEMENT-BAUD[expr].

All  comments that apply to the LIST command apply to the LIST# command.

The LIST#(cr) command is included to  permit  the  user  to  make  "hard

copies"  of a program.   The output to the list device is on P1.7 of the

MCS BASIC-52 device.

17

2.5 DESCRIPTION OF COMMANDS:

COMMAND: LIST@(cr) (VERSION 1.1 ONLY)

ACTION TAKEN:

The LIST @ command does the same thing as the LIST command  except  that

the  output  is directed to a user defined output driver.   This command

assumes that the user has placed an assembly language output routine  in

external code memory location 403CH.  To enable the @ driver routine the

user  must  SET BIT 27H (39D) in the internal memory of the MCS BASIC-52

device.   BIT 27H (39D) is BIT 7 of internal memory location 24H  (36D).

This BIT can be set by the BASIC statement DBY(24H)=DBY(24H).  OR.80H or

by a user supplied assembly language routine.   If the user evokes the @

driver routine and this bit is not set,  the output will be directed  to

the console driver.   The only reason this BIT must be set to enable the

@ driver is that it adds a certain degree of  protection  from  acciden-

tally  typing  LIST@  when  no  assembly  language  routine exist.   The

philosophy here is that if the user sets the bit,  the user provides the

driver or else!!!

When  MCS  BASIC-52  calls  the  user  output driver routine at location

403CH,  the byte to output is in the accumulator and R5 of register bank

0  (RB0).   The user may modify the accumulator (A) and the data pointer

(DPTR) in the assembly language output routine, but cannot modify any of

the registers in RB0.   This is intended to make it real  easy  for  the

user  to  implement a parallel or serial output driver without having to

do a PUSH or a POP.

18

2.6 DESCRIPTION OF COMMANDS

COMMAND:NEW(cr)

ACTION TAKEN:

When NEW (cr) is entered,  MCS BASIC-52 deletes the program that is cur-

rently stored in RAM memory.   In addition,  all variables are set equal

to ZERO,  all strings and all BASIC evoked interrupts are cleared.   The

REAL TIME CLOCK, string allocation, and the internal stack pointer value

(location 3EH) are NOT effected.  In general, NEW (cr) is used simply to

erase a program and all variables.

19

2.7  DESCRIPTION OF COMMANDS

COMMAND: NULL [integer](cr)

ACTION TAKEN:

The NULL[integer] (cr) command determines how many NULL characters (00H)

MCS BASIC-52 will output after a carriage return.   After initialization

NULL = 0.   The NULL command was more important back in the days when  a

"pure"  mechanical printer was the most common I/0 device.   Most modern

printers contain some kind of RAM buffer that virtually  eliminates  the

need to output NULL characters after a carriage return.  NOTE - the NULL

count  used by MCS BASIC-52 is stored in internal RAM location 21 (15H).

The NULL value can be changed  dynamically  in  a  program  by  using  a

DBY(21)=[expr] statement.  The [expr] can be any value between 0 and 255

(0FFH) inclusive.

VARIATIONS:

None.

20

CHAPTER 3

Description of EPROM File Commands

DESCRIPTION OF EPROM FILE COMMANDS

One  of  the unique and powerful features of MCS BASIC-52 is that it has

the ability to execute and SAVE programs in an  EPROM.MCS  BASIC-52  ac-

tually  generates all of the timing signals needed to program most EPROM

devices.   Saving programs in EPROMS is a much more attractive and RELI-

ABLE alternative relative to cassette tape, especially in control and/or

noisy environments.

The hardware needed to permit MCS BASIC-52 to program an EPROM device is

minimal,  typically only one NAND gate, three or four transistors, and a

few resistors are all that is required.  Details of the hardware require-

ments are in the EPROM PROGRAMMING section of this manual.

MCS BASIC-52 can save more than one program in an EPROM.   In  fact,  it

can save as many programs as the size of the EPROM memory permits.   The

programs are stored sequentially in the EPROM and  any  program  can  be

retrieved and executed.  This sequential storing of programs is referred

to as the EPROM FILE.   The following commands permit the user to gener-

ate and manipulate the EPROM FILE.

21

3.1 DESCRIPTION OF EPROM FILE COMMANDS

COMMANDS: RAM(cr) and ROM [integer] (cr)

ACTION TAKEN:

These two commands tell the MCS BASIC-52 interpreter whether  to  select

the  current  program  (the current program is the one that will be dis-

played during a LIST command and executed when RUN is typed) out of  RAM

or EPROM.  The RAM address is assumed to be 512 (200H) and the EPROM ad-

dress begins at 32, 784 (8010H)

RAM

When  RAM(cr)  is  entered MCS BASIC-52 selects the current program from

RAM MEMORY.   This is usually considered the "normal" mode of  operation

and is the mode that most users interact with the command interpreter.

ROM

When  ROM  [integer](cr)  is  entered  MCS  BASIC-52 selects the current

program out of EPROM memory.   If no integer is typed after the ROM com-

mand  (i.e.ROM(cr)) MCS BASIC-52 defaults to ROM 1.   Since the programs

are store sequentially in EPROM the integer following  the  ROM  command

selects which program the user wants to run or list.   If you attempt to

select a program that does not exist (i.e.  you type in ROM 8 and only 6

programs  are  stored  in the EPROM) the message ERROR:PROM MODE will be

displayed.

MCS BASIC-52 does not transfer the program from EPROM to  RAM  when  the

ROM  mode is selected.   So,  you cannot EDIT a program in the ROM mode.

If you attempt to a program in the ROM mode, by typing in a line number,

the message ERROR:PROM MODE will be displayed.  The following command to

be described, XFER,  permits one to transfer a program from EPROM to RAM

fro editing purposes.

Since the ROM command does NOT transfer a program to RAM, it is possible

to have different programs in ROM and RAM simultaneously.   The user can

"flip" back and forth between the two modes at any time.   Another added

benefit  of  NOT  transferring  a  program to RAM is that all of the RAM

memory can be used for variable storage if  the  PROGRAM  is  stored  in

EPROM.    The  SYSTEM CONTROL VALUES - MTOP and FREE always refer to RAM

not EPROM.

VARIATIONS:

None.

22

3.2 DESCRIPTION OF EPROM FILE COMMANDS

COMMAND:XFER(cr)

ACTION TAKEN:

The XFER (transfer) command transfers the current  selected  program  in

EPROM to RAM and then selects the RAM mode.   If XFER is typed while MCS

BASIC-52 is in the RAM mode,  the program stored in RAM  is  transferred

back into RAM and the RAM mode selected.  The net result is that nothing

happens  except  that  a  few  milli-seconds of CPU time is used to do a

wasted move.  After the XFER command is executed,  the user may edit the

program in the same manner any RAM program may be edited.

VARIATIONS:

None.

23

3.3 DESCRIPTION OF EPROM FILE COMMANDS

COMMAND:PROG(cr)

ACTION TAKEN:

The  PROG  COMMAND programs the resident EPROM with the current selected

program.   The current selected program may  reside  in  either  RAM  or

EPROM.  This command assumes that the hardware is configured in the man-

ner described in the EPROM PROGRAMMING section of this manual.

After PROG (cr) is typed,  MCS BASIC-52 displays the number in the EPROM

FILE the program will occupy.

EXAMPLE:

>LIST

10      FOR I=1 TO 10

20      PRINT I

30      NEXT I

READY

>PROG

12

READY

>ROM 12

READY

>LIST

10      FOR I=1 TO 10

20      PRINT I

30      NEXT I

READY

>

In this example,  the program just placed  in  the  EPROM  is  the  12th

program stored.

VARIATIONS:

None.

24

3.4 DESCRIPTION OF EPROM FILE COMMANDS

COMMANDS: PROG1(cr) and PROG2(cr)

ACTION TAKEN:

PROG1

Normally,  after  power is applied to the MCS BASIC-52 device,  the user

MUST type a "space" character to initialize the  8052AH's  serial  port.

As a convenience, MCS BASIC-52 contains a PROG1 COMMAND.  What this com-

mand  does is program the resident EPROM with the BAUD RATE information.

So,  the next time the MCS BASIC-52 device is "powered up," i.e.  RESET,

the  chip will read this information and initialize the serial port with

the stored baud rate.  The "sign on" message will be sent to the console

immediately after the MCS BASIC-52 device completes its reset  sequence.

The  "space" character no longer needs to be typed.   of course,  if the

BAUD rate on  the  console  device  is  changed  a  new  EPROM  must  be

programmed to make MCS BASIC-52 compatible with the new console.

PROG2

The PROG2 command does everything the PROG1 command does, but instead of

"signing on" and entering the COMMAND MODE,  the MCS BASIC-52 device im-

mediately begins executing the first  program  stored  in  the  resident

EPROM.

THIS IS AN IMPORTANT FEATURE!!

By  using the PROG2 command it is possible to RUN a program from a RESET

condition and NEVER connect the MCS BASIC-52 chip to a console.   In es-

sence,  saving  PROG2  information is equivalent to typing a ROM 1,  RUN

command sequence.   This is ideal for control applications,  where it is

not always possible to have terminal present.  In addition, this feature

permits  the user to write a special initialization sequence in BASIC or

ASSEMBLY LANGUAGE and generate a custom "sign on" message  for  specific

applications.

25

3.5 DESCRIPTION OF EPROM FILE COMMANDS

COMMANDS: FPROG1(cr), FPROG1(cr), AND FPROG2(cr)

ACTION TAKEN:

FPROG(cr),  FPROG1(cr),  and  FPROG2(cr)  do  exactly  the same thing as

PROG(cr), PROG1(cr),  and PROG2(cr) respectively,  except that the algo-

rithm   used   to   perform   the  programming  function  is  the  INTEL

"INTELLIGENT" fast programming algorithm.   The user MUST provide a  way

to increase VCC to the EPROM to 6 volts.

26

3.6 DESCRIPTION OF EPROM FILE COMMANDS

COMMANDS:  PROG3(cr),  PROG4(cr),  FPROG3(cr),  FPROG4(cr)  (VERSION 1.1

ONLY)

ACTION TAKEN:

PROG3

The PROG3 COMMAND functions the same way as the PROG1 COMMAND previously

described, except that PROG3 also saves the system control value,  MTOP,

when  it  is  evoked.   During a RESET or power-up sequence MCS BASIC-52

will only clear the external data memory up to the MTOP value  that  was

saved  when  the  PROG3  COMMAND  was evoked.   This permits the user to

"protect" regions of memory from being cleared during a RESET or  power-

up condition.   In typical use,  the PROG3 COMMAND assumes that the user

is saving some critical information if some type to be de                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                 algorithm

is used to program the EPROM.

VARIATIONS:

None.

27

3.7 DESCRIPTION OF EPROM FILE COMMANDS

COMMANDS:  PROG5(cr),  PROG6(cr),  FPROG5(cr),  FPROG6(cr)  (VERSION 1.1

ONLY)

ACTION TAKEN:

PROG5 & FPROG5

The PROG5 command saves both the baud rate information and the MTOP  in-

formation,  just like the PROG3 command previously described.   However,

during a RESET or power-up condition the MCS  BASIC-52  device  examines

external data memory location 5FH (95 decimal).   If the user has placed

the value 0A5H (165 decimal) in this location.  This permits the user to

"save" programs in external memory, providing some type of battery back-

up scheme has been employed.

Normally,  when using the PROG5 command to establish the RESET or power-

up condition,  the MCS BASIC-52 device will enter the command mode after

RESET or power-up.   However,  if the user wishes to execute the program

stored  in  external memory,  the character 34H (52 decimal) needs to be

placed in external memory location 5EH (94 decimal).   Placing a 34H  in

location  5EH causes MCS BASIC-52 to enter the "RUN TRAP MODE."  Details

of this mode are presented in chapter 8 of this manual.

PROG6 & FPROG6

Does the same thing as PROG5, but CALLS external program memory location

4039H during a RESET or power-up sequence.   This option  also  requires

the  user  to  put the character 0A5H in external memory location 5FH to

insure that external RAM will not be cleared during RESET  or  power-up.

The  user must put an assembly language initialization routine in exter-

nal code memory location 4039H or else this RESET mode will crash.  When

the user returns from the customized assembly  language  RESET  routine,

three options exist:

OPTION 1 FOR PROG6

If  the  CARRY  BIT is CLEARED (CARRY=0) upon return from the user RESET

routine MCS BASIC-52 will enter the auto-baud rate determining  routine.

The  user must then type a space character (20H) on the terminal to com-

plete the RESET routine and produce a RESET message on the terminal.

28

OPTION 2 FOR PROG6

If the CARRY BIT is SET (CARRY = 1) and BIT  0  of  the  ACCUMULATOR  is

CLEARED (ACC. 0 = 0) MCS BASIC-52 will produce the standard sign-on mes-

sage  upon  return from the user supplied RESET routine.   The baud rate

will be the one that was saved when the PROG6 option was used.

OPTION 3 FOR PROG6

If the CARRY BIT is SET (CARRY = 1) and BIT 0 of the ACCUMULATOR is  SET

(ACC. 0 = 1), MCS BASIC-52 will executed the first program

stored  by  the user in EPROM (starting address of the program is 8010H)

upon return from the user supplied RESET routine.

29

CHAPTER 4

Description of Statements

4.1 DESCRIPTION OF STATEMENTS

STATEMENT: BAUD [expr]

MODE: COMMAND AND/OR RUN

TYPE: CONTROL

The BAUD [expr] statement is used to set the baud rate for the  software

line  printer  port  resident on the MCS BASIC-52 device.   In order for

this STATEMENT to properly calculate the baud rate, the crystal (special

function operator - XTAL)  must  be  correctly  assigned  (e.g.  XTAL  =

9000000).    MCS  BASIC-52  assumes a crystal value of 11.0592 MHz if no

XTAL value is assigned.   The software line printer port is P1.7 on  the

8052AH device.  The main purpose of the software line printer port is to

let  the  user  make  a "hard copy" of the program listings and/or data.

The COMMAND LIST# and  the  STATEMENT  PRINT#  directs  outputs  to  the

software  line  printer  port.   If the BAUD [expr] STATEMENT is not ex-

ecuted before a LIST# or PRINT# command/statement is entered, the output

of the software line printer port will be at about 1 BAUD  and  it  will

take A LONG TIME to output something.  You may even think that BASIC has

crashed,  but it hasn't.   It's just outputting at a VERY SLOW rate.  So

be sure to assign a BAUD rate to the software printer port BEFORE  using

LIST# or PRINT#.  The maximum baud rate that can be assigned by the BAUD

statement depends on the crystal.   in general, 4800 is reasonable maxi-

mum baud rate,  however the user may want to experiment  with  different

rates.   The software serial transmits 8 data bits, 1 start bit, and two

stop bits.  no parity is transmitted.

EXAMPLE:

BAUD 1200

Will cause the line printer port to output data at 1200 BAUD.

VARIATIONS:

None.

30

4.2 DESCRIPTION OF STATEMENTS

STATEMENT: CALL [integer]

MODE: COMMAND AND/OR RUN

TYPE: CONTROL

The CALL [integer] STATEMENT  is  used  to  call  an  assembly  language

program.   The integer following CALL is the address where the user must

provide the assembly language routine.  To return to BASIC the user must

execute an assembly language RET instruction.   Examples of how  to  use

the  CALL  [integer]  instruction  are  given  in  the ASSEMBLY LANGUAGE

LINKAGE section of this manual.

EXAMPLE:

CALL 9000H

Will cause the 8052AH to execute the assembly language program beginning

at location 9000H (i.e. the program counter will be loaded with 9000H).

VARIATIONS: (VERSION 1.1 ONLY)

If the integer following the CALL statement is between 0 and 127  (7FH),

Version 1.1 of MCS BASIC-52 will multiply the user integer by two,  then

add 4100H and vector to that location.  This means that CALL 0 will call

location 4100H, CALL 1 will call 4102H, CALL 2 - 4104H and so on.   This

permits  the  user  to  generate  a  simple  table  of assembly language

routines without having to enter 4 digit hex  integers  after  the  CALL

statement from the user supplied RESET routine.

31

4.3 DESCRIPTION OF STATEMENTS

STATEMENT: CLEAR

MODE:  COMMAND AND/OR RUN

TYPE: CONTROL

The  CLEAR  STATEMENT sets all variables equal to 0 and resets all BASIC

evoked interrupts and stacks.  This means that after the CLEAR statement

is executed an ONEX1 or ONTIME statement must  be  executed  before  MCS

BASIC-52   will   acknowledge   interrupts.   ERROR   trapping  via  the

ONERR[integer] STATEMENT is executed.  The CLEAR STATEMENT does not  af-

fect the real time clock that is enabled by the CLOCK1 STATEMENT.  CLEAR

does not also reset the memory that has been allocated for  STRINGS,  so

it is NOT necessary to enter the STRING [expr],  [expr] STATEMENT to re-

allocate memory for strings after the CLEAR STATEMENT  is  executed.  In

general, CLEAR is simply used to "erase" all variables.

VARIATIONS:

None.

32

4.4 DESCRIPTION OF STATEMENTS

STATEMENTS: CLEARI (clear interrupts)

CLEARS (clear stacks)

MODE: COMMAND AND/OR RUN

TYPE: CONTROL

CLEARI

The CLEARI STATEMENT clears all of the BASIC evoked interrupts. Specifi-

cally,  the  ONTIME  and  ONEX1 interrupts are DISABLED after the CLEARI

STATEMENT is executed.  This is accomplished by clearing bits 2 and 3 of

the  8052AH's  special function register,  IE and by clearing the status

bits that determine whether MCS BASIC-52  or  the  user  is  controlling

these  interrupts.  The  real  time clock which is enabled by the CLOCK1

STATEMENT is not affected by CLEARI. This statement can be used to selec-

tively DISABLE interrupts during specific sections of  the  users  BASIC

program.  The  ONTIME  and/  or  ONEX1 STATEMENTS MUST BE EXECUTED AGAIN

before the specific interrupts will be enabled.

CLEARS

The CLEARS statement RESETS all of MCS BASIC-52's  STACKS.  The  CONTROL

and  ARGUMENT  STACKS are set to their initialization value,  254 (OFEH)

and 510 (1FEH) respectively.  The INTERNAL  STACK  (the  8052AH's  STACK

POINTER,  SPECIAL FUNCTION REGISTER-SP) is loaded with the value that is

in INTERNAL RAM location 62 (3EH). This statement can be used to "purge"

the stack should an error occur in a subroutine. In addition, this state-

ment can be used to provide a "special" exit from a FOR-NEXT,  DO-WHILE,

or DO-UNTIL loop.

EXAMPLE OF CLEARS:

>10 PRINT "MULTIPLICATION TEST, YOU HAVE 5 SECONDS"

>20 FOR I = 2 TO 9

>30 N = INT(RND*10) : A = N*I

>40 PRINT "WHAT IS ", N, "*", I, "?": CLOCK1

>50 TIME = 0 : ONTIME 5,200 : INPUT R:IF R<>A THEN 100

>60 PRINT "THAT'S RIGHT": TIME=0: NEXT I

>70 PRINT "YOU DID IT, GOOD JOB":END

>100 PRINT "WRONG, TRY AGAIN": GOTO 50

>200 REM WASTE CONTROL STACK, TOO MUCH TIME

>210 CLEARS: PRINT "YOU TOOK TOO LONG": GOTO 10

NOTE:  When the CLEARS and CLEARI STATEMENTS are LISTED they will appear

as CLEARS and CLEAR I respectively.  Don't be alarmed,  that is the  way

it's supposed to work.

4.5 DESCRIPTIONS OF STATEMENTS

33

STATEMENTS: CLOCK1 and CLOCK0

MODE: COMMAND AND/OR RUN

TYPE: CONTROL

CLOCK1

The CLOCK1 STATEMENT enables the REAL TIME CLOCK feature the resident on

the  MCS  BASIC-52 device.  The special function operator TIME is incre-

mented once every 5 milliseconds after the CLOCK1 STATEMENT has been ex-

ecuted.  The CLOCK1 STATEMENT uses TIMER/COUNTER 0 in the 13-bit mode to

generate  an interrupt once every 5 milliseconds.  Because of this,  the

special function operator TIME has a resolution of 5 milliseconds.

MCS BASIC-52  automatically  calculates  the  proper  reload  value  for

TIMER/COUNTER 0 after the crystal value has been assigned,  MCS BASIC-52

assumes a value of 11.0592  MHz.  The  special  function  operator  TIME

counts from 0 to 65535.995 seconds.  After reaching a count of 65535.995

seconds TIME overflows back to a count of zero. Because the CLOCK1 STATE-

MENT uses the interrupts associated with  TIMER/COUNTER  0  (the  CLOCK1

statements  sets bits 7 and 2 in the 8052AH's special function register,

IE), the user may not use this interrupt in an assembly language routine

if the CLOCK1 STATEMENT is executed in BASIC.  The interrupts associated

with  the  CLOCK1  STATEMENT cause MCS BASIC-52 programs to run at about

99.6% of normal speed.  That means that the interrupt handling  for  the

REAL  TIME  CLOCK feature only consumes about .4% of the total CPU time.

This very small interrupt overhead is attributed to the  very  fast  and

effective interrupt handling of the 8052AH device.

CLOCK0

The  CLOCK0 (zero) STATEMENT disables or "turns off" the REAL TIME CLOCK

feature.   This statement clears bit 2 in the 8052AH's special  function

ARIATIONS:

None.

4.6 DESCRIPTIONS OF STATEMENTS

STATEMENTS: DATA-READ-RESTORE

MODE: RUN

34

TYPE: ASSIGNMENT

DATA

DATA specifies expressions that may be retrieved by a READ STATEMENT. If

multiple expressions per line are used,  they MUST  be  separated  by  a

comma.

READ

READ  retrieves the expressions that are specified in the DATA STATEMENT

and assigns the value of the expression to  the  variable  in  the  READ

STATEMENT  .  The  READ STATEMENT MUST ALWAYS be followed by one or more

variables. If more than one variable follows a READ STATEMENT, they MUST

be separated by a comma.

RESTORE

RESTORE "resets" the internal read pointer back to the beginning of  the

data so that it may be read again.

EXAMPLE:

>10 FOR I=1 TO 3

>20 READ A,B

>30 PRINT A,B

>40 NEXT I

>50 RESTORE

>60 READ A,B

>70 PRINT A,B

>80 DATA 10,20,10/2,20/2,SIN(PI),COS(PI)

>RUN

10  20

5  10

0 -1

10  20

VARIATIONS:

None.

4.6 DESCRIPTION OF STATEMENTS

Explanation of the previous example:

35

Everytime  a  READ STATEMENT is encountered the next consecutive expres-

sion in the DATA STATEMENT is evaluated and assigned to the variable  in

the  READ  STATEMENT.  DATA  STATEMENTS  may be placed anywhere within a

program,  they will NOT be executed nor will they cause an error .  DATA

STATEMENTS  are  considered  to be chained together and appear to be one

BIG DATA STATEMENT.If at anytime all the DATA has been read and  another

READ  STATEMENT  is  executed then the program is terminated and message

ERROR: NO DATA-IN LINE XX is printed to the console device.

36

4.7 DESCRIPTIONS OF STATEMENTS

STATEMENT: DIM

MODE: COMMAND AND/OR RUN

TYPE: ASSIGNMENT

DIM reserves storage for matrices.  The storage area is first assumed to

be  zero.  Matrices  in MCS BASIC-52 may have only ONE DIMENSION and the

size of the dimensioned array MAY NOT exceed 254 elements.  Once a vari-

able  is  dimensioned in a program it may not be re-dimensioned.  An at-

tempt to re-dimension an array will caused an ARRAY SIZE  ERROR.  If  an

arrayed variable is used that has NOT been dimensioned by the DIM STATE-

MENT, BASIC will assign a default value of 10 to the array size. All ar-

rays  are  set equal to zero when the RUN COMMAND,  NEW COMMAND,  or the

CLEAR STATEMENT is executed. The number of bytes is allocated for an ar-

ray is 6 times the (array size plus 1). So,  the array A (100) would re-

quire  606  bytes  of storage.  Memory size usually limits the size of a

dimensioned array.

VARIATIONS:

More than one variable can be dimensioned by  a  single  DIM  STATEMENT,

i.e., DIM A(10), B(15), A1(20)

EXAMPLE:

DEFAULT ERROR ON ATTEMPT TO RE-DIMENSION ARRAY

>10 A(5)=10    -BASIC ASSIGNS DEFAULT OF 10 TO ARRAY SIZE HERE

>20 DIM A(5)   -ARRAY CANNOT BE RE-DIMENSIONED

>RUN

ERROR:   ARRAY SIZE - IN LINE 20

20   DIM A(5)

---------X

37

4.8 DESCRIPTION OF STATEMENTS

STATEMENTS: DO-UNTIL [rel expr]

MODE: RUN

TYPE: CONTROL

The  DO-UNTIL  [rel expr] instruction provides a means of 'loop control"

within an MCS BASIC-52 program.  All statements between the DO  and  the

UNTIL [rel expr] will be executed until the relational expression follow-

ing the UNTIL statements is TRUE. DO-UNTIL loops may be nested.

EXAMPLES:

SIMPLE DO-UNTIL           NESTED DO-UNTIL

>10 A=0                   >10 DO : A=A+1 : DO : B=B+1

>20 DO                    >20 PRINT A,B,A*B

>30 A=A+1                 >30 UNTIL B=3

>40 PRINT A               >40 B=0

>50 UNTIL A=4             >50 UNTIL A=3

>60 PRINT "DONE"          >RUN

>RUN

1 1 1

1                         1 2 2

2                         1 3 3

3                         2 1 2

4                         2 2 4

DONE                       2 3 6

3 1 3

READY                      3 2 6

>                          3 3 9

READY

>

VARIATIONS:

None.

38

4.9 DESCRIPTION OF STATEMENTS

STATEMENTS:DO-WHILE [rel expr]

MODE: RUN

TYPE: CONTROL

The  DO-WHILE  [rel expr] instruction provides a means of "loop control"

within an MCS BASIC-52 program.  This operation  of  this  statement  is

similar  to  the  DO-UNTIL [rel expr] except that all statements between

the DO and the WHILE [rel expr] will be executed as long  as  the  rela-

tional  expression  following the WHILE statement is true.  DO-WHILE and

DO-UNTIL statements can be nested.

EXAMPLES:

SIMPLE DO-WHILE          NESTED DO-WHILE - DO-UNTIL

>10 DO                    >10 DO : A=A+1 : B=B+1

>20 A=A+1                 >20 PRINT A,B,A*B

>30 PRINT A               >30 WHILE B<>3

>40 WHILE A<4             >40 B=0

>50 PRINT "DONE"          >50 UNTIL A=3

>RUN                      >RUN

1                         1 1 1

2                         1 2 2

3                         1 3 3

4                         2 1 2

DONE                       2 2 4

2 3 6

READY                      3 1 3

>                          3 2 6

3 3 9

READY

>

VARIATIONS:

None.

39

4.10 DESCRIPTION OF STATEMENTS

STATEMENT: END

MODE: RUN

TYPE: CONTROL

The END STATEMENT terminates program execution.  The  continue  command,

CONT  will  not operate if the END STATEMENT is used to terminate execu-

tion (i.e., a CAN'T CONTINUE ERROR will be printed to the console).  The

last  statement  in an MCS BASIC-52 program will automatically terminate

program execution if no END STATEMENT is used.

EXAMPLES:

LAST STATEMENT TERMINATION         END STATEMENT TERMINATION

>10 FOR I= 1 TO 4                  >10 FOR I=1 TO 4

>20 PRINT I                        >20 GOSUB 100

>30 NEXT I                         >30 NEXT I

>40 RUN                            >40 END

>100 PRINT I

1                               >110 RETURN

2                               >RUN

3

4                                1

2

READY                             3

>                                 4

READY

>

VARIATIONS:

None.

40

4.11 DESCRIPTION OF STATEMENTS

STATEMENTS: FOR-TO-{STEP}-NEXT

MODE: RUN VERSION 1.0 (COMMAND AND/OR RUN in Version 1.1)

TYPE: CONTROL

The FOR-TO-{STEP}-NEXT STATEMENTS are used to set up and control loops.

EXAMPLE:

10 FOR A=B TO C STEP D

20 PRINT A

30 NEXT A

If B=0, C=10, and D=2, the PRINT STATEMENT at line 20 will be executed 6

times. The values of "A" that will be printed are 0,2,4,6,8,10. "A" rep-

resents the name of the index or loop counter.  The value of "B" is  the

starting value of the index,  the value of "C" is the limit value of the

index, and the value of "D" is the increment value defaults to 1, there-

fore, STEP is an optional statement. The NEXT STATEMENT causes the value

of "D" to be added to the index. The index is then compared to the value

of "C", the limit. If the index is less than or equal to the limit, con-

trol will be transferred back to the statement after the FOR  STATEMENT.

Stepping "backwards" (i.e., FOR I = 100 TO 1 STEP-1) is permitted in MCS

BASIC-52. Unlike some BASICS, the index MAY NOT be omitted from the NEXT

STATEMENT in MCS BASIC-52 (i.e.,  the NEXT statement MUST always be fol-

lowed by the appropriate variable).

EXAMPLES:

>10 FOR I=1 TO 4         >10 FOR I=0 TO 8 STEP 2

>20 PRINT I              >20 PRINT I

>30 NEXT I               >30 NEXT I

>RUN                     >RUN

1                        0

2                        2

3                        4

4                        6

8

READY

>                        READY

>

4.11 DESCRIPTION OF STATEMENTS

41

In Version 1.1 of MCS BASIC-52 it is possible  to  execute  the  FOR-TO-

{STEP}-NEXT  statement  in the Command Mode.  This makes it possible for

the user to do things like display regions of memory by writing a  short

program  like FOR I=512 TO 560: PH0.  XBY(I),: NEXT I.  It may also have

other uses, but they haven't been thought of.

Also Version 1.1 allows the NEXT statement to be used without a variable

following the statement. This means that programs like:

EXAMPLE:

10 FOR I = 1 TO 100

20 PRINT I

30 NEXT

Are permitted in Version 1.1 of MCS BASIC-52.  The  variable  associated

with  the NEXT is always assumed to be the variable used in the last FOR

statement.

42

4.12 DESCRIPTION OF STATEMENTS

STATEMENTS: GOSUB[In num]-RETURN

MODE: RUN

TYPE: CONTROL

GOSUB

The GOSUB [In num] STATEMENT will cause MCS BASIC-52 to transfer control

of the program directly to the line  number  ([In  num])  following  the

GOSUB STATEMENT.  In addition, the GOSUB STATEMENT saves the location of

the STATEMENT following GOSUB on the control  stack  so  that  a  RETURN

STATEMENT can be performed to return control.

RETURN

This statement is used to "return" control back to the STATEMENT follow-

ing  the  most  recently executed GOSUB STATEMENT.  The GOSUB-RETURN se-

quence can be "nested" meaning that a subroutine  called  by  the  GOSUB

STATEMENT can call another subroutine with another GOSUB STATEMENT.

EXAMPLE:

SIMPLE SUBROUTINE           NESTED SUBROUTINES

>10 FOR I=1 TO 5             >10 FOR I=1 TO 3

>20 GOSUB 100                >20 GOSUB 100

>30 NEXT I                   >30 NEXT I

>100 PRINT I                 >40 END

>110 RETURN                  >100 PRINT I,

>RUN                         >110 GOSUB 200

>120 RETURN

1                           >200 PRINT I*I

2                           >210 RETURN

3                           >RUN

4

5                            1 1

2 4

READY                         3 9

>

READY

>

4.12 DESCRIPTION OF STATEMENTS

43

NOTE-  The Control Stack on Version 1.1 permits a graceful exit from in-

complete control loops, given the following example:

EXAMPLE:

.

.

50    GOSUB 1000

.

.

1000  FOR I = 1 TO 10

1010  IF X = I THEN 1040

1020  PRINT I*X

1030  NEXT I

1040  RETURN

Version 1.1 would permit the programmer  to  exit  the  subroutine  even

though the FOR-NEXT loop might not be allowed to complete if X did equal

I.  Version  1.0 of MCS BASIC-52 would yield a C-STACK error if the FOR-

NEXT loop was not allowed to complete before the  RETURN  statement  was

executed.

44

4.13 DESCRIPTION OF STATEMENTS

STATEMENT:GOTO [IN num]

MODE: COMMAND AND/OR RUN

TYPE: CONTROL

The  GOTO  [In  num}  STATEMENT  will  cause  BASIC  to transfer control

directly to the line number ([In num]) following the GOTO STATEMENT.

EXAMPLE:

50 GOTO 100

Will,  if line 100 exists,  cause execution of the program to resume  at

line  100.  If  line number 100 does not exist the message ERROR:INVALID

LINE NUMBER will be printed to the console device.

Unlike the RUN COMMAND the GOTO STATEMENT,  if executed in  the  COMMAND

MODE,  does not CLEAR the variable storage space or interrupts. However,

if the GOTO STATEMENT is executed in the COMMAND MODE after a  line  has

been edited,  MCS BASIC-52 will CLEAR the variable storage space and all

BASIC evoked interrupts.  This  is  a  necessity  because  the  variable

storage and the BASIC program reside in the same RAM memory.  So editing

a program can destroy variables.

NOTE-(Version 1.0 only)

Because of the way MCS BASIC-52's text  interpreter  processes  a  line,

when  an  INVALID LINE NUMBER ERROR occurs on the GOTO,GOSUB,ON GOTO,and

ON GOSUB STATEMENTS the line AFTER the GOTO or GOSUB STATEMENT  will  be

printed  out in the error message.  This may be confusing,  but it was a

trade-off between execution speed,code size,and  error  handling.  Error

handling lost.

EXAMPLE:

>10 GOTO                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                 E: CONTROL

The  value of the expression following the ON statement is the number in

the line list that control will be transferred to.

EXAMPLE:

10 ON Q GOTO 100,200,300

If Q was equal to 0,  control would be transferred to line  number  100.

If  Q  was equal to 1,  control would be transferred to line number 200.

If Q was equal to 2,  GOTO line 300,  etc.   All comments that apply  to

GOTO and GOSUB apply to the ON STATEMENT.   If Q is less than ZERO a BAD

ARGUMENT ERROR will be generated.   If Q is greater than the line number

list  following the GOTO or GOSUB STATEMENT,  a BAD SYNTAX ERROR will be

generated.   The ON STATEMENT provides "conditional  branching"  options

within the constructs of an MCS BASIC-52 program.

46

4.15 DESCRIPTIONS OF STATEMENTS

STATEMENTS: IF - THEN - ELSE

MODE: RUN

TYPE: CONTROL

The  IF  statement sets up a conditional test.   The generalized form of

the IF - THEN - statement is as follows:

[In num] IF [rel expr] THEN valid STATEMENT ELSE valid STATEMENT

A specific example is as follows:

>10 IF A=100 THEN A=0 ELSE A=A+1

Upon execution of line 10 IF A is equal to 100, THEN A would be assigned

a value of 0.   IF A does not equal 100,  A would be assigned a value of

A+1.    If  it  is desired to transfer control to different line numbers

using the IF statement, the GOTO statement may be omitted.   The follow-

ing examples would yield the same results:

>20 IF INT(A)< 10 THEN GOTO 100 ELSE GOTO 200

>20 IF INT(A)< 10 THEN 100 ELSE 200

Additionally, the THEN statement can be replaced by any valid MCS BASIC-

52 statement, as shown below:

>30 IF A<>10 THEN PRINT A ELSE 10

>30 IF A<> PRINT A ELSE 10

The ELSE statement may be omitted.   If it is,  control will pass to the

next statement.

EXAMPLE:

>20 IF A=10 THEN 40

>30 PRINT A

In this example, IF A equals 10 then control would be passed to line num-

ber 40.  Is A does not equal 10 line number 30 would be executed.

4.15 DESCRIPTIONS OF STATEMENT

COMMENTS ON IF-THEN-ELSE-

47

Version 1.1 is not compatible with V1.0 when the IF-THEN-ELSE  STATEMENT

is used with multiple statements per line.   In V1.0,  the following two

examples would function in the same manner.

EXAMPLE 1:

10 IF A=B THEN C=A : A=A/2 : GOTO 100

20 PRINT A

EXAMPLE 2:

10 IF A=B THEN C=A

12 A=A/2

14 GOTO 100

20 PRINT A

They function in the same manner because V1.0 treats the  delimiter  (:)

exactly the same as a carriage return (cr) in every case.  However, V1.1

executes the remainder of the line if and if only if the test A=B proves

to be true.   This means in EXAMPLE 1 IF A did equal B,  V1.1 would then

set C = A, then set A=A/2, then execute line 100.  If A did not equal B,

V1.1 would then PRINT A and ignore the statements C=A: A=A/2: GOTO  100.

V1.1  will  execute  EXAMPLE 2 exactly the same way as V1.0.   This same

logical interpretation holds true for the ELSE statement as well.   This

example  dictates  a  simple  rule  for  maintaining  IF-THEN-ELSE  com-

patibility between the two versions.   IF THE DELIMITER (:) IS NOT  USED

IN AN IF-THEN-ELSE STATEMENT, V1.0 AND V1.1 WILL TREAT THE STATEMENTS IN

THE SAME MANNER!!

This  change  was  made because most users of MCS BASIC-52 felt that the

V1.1 interpretation of this statement was more useful because fewer GOTO

statements need be employed in a typical program.

48

4.16 DESCRIPTION OF STATEMENTS

STATEMENTS: INPUT

MODE: RUN

TYPE: INPUT/OUTPUT

The INPUT statement allows users to enter data from the  console  during

program  execution.    One  or more variables maybe assigned data with a

single input statement.  The variables must be separated by a comma.

EXAMPLE:

INPUT A,B

Would cause the printing of a question mark (?) on the console device as

a prompt to the operator to input two numbers separated by a comma.   If

the  operator does not enter enough data,  then MCS BASIC-52 responds by

outputting the message TRY AGAIN to the console device.

EXAMPLE:

>10 INPUT A,B

>20 PRINT A,B

>RUN

?1

TRY AGAIN

?1, 2

1   2

READY

49

The INPUT statement may be written  so  that  a  descriptive  prompt  is

printed  to  tell  the user what to type.   The message to be printed is

placed in quotes after the INPUT statement.   If a comma appears  before

the first variable on the input list, the question mark prompt character

will not be displayed.

EXAMPLES:

>10 INPUT "ENTER A NUMBER "A   >10 INPUT "ENTER A NUMBER-",A

>20 PRINT SQR (A)              >20 PRINT SQR (A)

>RUN                           >RUN

ENTER A NUMBER                 ENTER A NUMBER-100

?100                            10

10

50

4.16 DESCRIPTION OF STATEMENTS

Strings can also be assigned with a INPUT statement.  Strings are always

terminated with a carriage return (cr).  So, if more than one string in-

put is requested with a single INPUT statement, MCS BASIC-52 will prompt

the user with a question mark.

EXAMPLES:

>10 STRING 110, 10         >10 STRING 110, 10

>20 INPUT "NAME: ",$(1)    >20 INPUT "NAMES: ", $(1), $(2)

>30 PRINT "HI ", $ (1)     >30 PRINT "HI ", $(1)," AND ", $(2)

>RUN                       >RUN

NAME:  SUSAN               NAMES: BILL

HI  SUSAN                  ?ANN

HI BILL AND ANN

READY

READY

Additionally,  strings and variables can be assigned with a single INPUT

statement.

EXAMPLE:

>10 STRING 100,10

>20 INPUT "NAME(CR), AGE - ",$(1), A

>30 PRINT "HELLO ", $(1),", YOU ARE ",A, "YEARS OLD"

>RUN

NAME (CR), AGE - FRED

?15

HELLO FRED, YOU ARE 15 YEARS OLD

READY

>

51

4.17 DESCRIPTION OF STATEMENTS

STATEMENT: LET

MODE: COMMAND AND/OR RUN

TYPE: ASSIGNMENT

The LET statement is used to assign a variable to the value of an expres-

sion.  The generalized form of LET is:

LET [var] = [expr]

EXAMPLES:

LET A = 10*SIN(B)/100 or

LET A = A + 1

Note that the = sign used in the LET statement is not equality operator,

but rather a "replacement" operator and that  the  statement  should  be

read A is replaced by A plus one.  THE WORD LET IS ALWAYS OPTIONAL, i.e.

LET A = 2 is the same as A = 2

When  LET  is omitted the LET statement is called an IMPLIED LET.   This

document will use the word LET to refer to both the  LET  statement  and

the IMPLIED LET statement.

The LET statement is also used to assign the string variables, i.e.:

LET $(1)="THIS IS A STRING" or

LET $(2)=$(1)

Before Strings can be assigned the STRING [expr],  [expr] STATEMENT MUST

be executed, or else a MEMORY ALLOCATION ERROR will occur.

SPECIAL FUNCTION VALUES can also be assigned by the LET statement, i.e.:

LET IE = 82H or

LET XBYTE(2000H)=5AH or

LET DBYTE(25) =XBYTE(1000)

4.18 DESCRIPTION OF STATEMENTS

STATEMENT: ONERR[In num]

52

MODE: RUN

TYPE: CONTROL

The ONERR[In num] statement lets the programmer  handle  arithmetic  er-

rors,  should they occur,  during program execution.   Only ARITH. OVER-

FLOW, ARITH. UNDERFLOW,  DIVIDE BY ZERO,  and BAD ARGUMENT errors can be

"trapped" by the ONERR statement, all other errors are not.  If an arith-

metic error occurs after the ONERR statement is executed, the MCS BASIC-

52  interpreter  will  pass  control  to  the  line number following the

ONERR[1n num] statement.   The programmer can handle the error condition

in  any manner suitable to the particular application.   Typically,  the

ONERR[1n num] statement should be viewed as an easy way to handle errors

that occur when the user provides inappropriate data to an INPUT  state-

ment.

With the ONERR[1n num] statement, the programmer has the option of deter-

mining what type of error occurred.   This is done by examining external

memory location 257 (101H) after the error condition is  trapped.    The

error codes are as follows:

ERROR CODE = 10 - DIVIDE BY ZERO

ERROR CODE = 20 ARITH. OVERFLOW

ERROR CODE = 30 ARITH. UNDERFLOW

ERROR CODE = 40 - BAD ARGUMENT

This location maybe examined by using an XBY(257) statement.

53

4.19 DESCRIPTION OF STATEMENTS

STATEMENT: ONEX1 [1N num]

MODE: RUN

TYPE: CONTROL

The  ONEX1  [1n  num]  statement  lets the user handle interrupts on the

8052AH's INT1 pin with a BASIC program.   The line number following  the

ONEX1  statement  tells  the MCS BASIC-52 interpreter which line to pass

control to when an interrupts occurs.   In essence,  the ONEX1 statement

"forces"  a  GOSUB to the line number following the ONEX1 statement when

the INTI pin on the 8052AH is pulled low.  The programmer must execute a

RETI statement to exit from the ONEX1 interrupt routine.  If this is not

done all future interrupts on the INT1 pin will be "locked out" and  ig-

nored until a RETI is executed.

The  ONEX1  statement sets bits 7 and 2 of the 8052AH's interrupt enable

register IE.  Before an interrupt can be processed, the MCS BASIC-52 in-

terpreter must complete execution  of  the  statement  it  is  currently

processing.     Because  of  this,   interrupt  latency  can  vary  from

microseconds to tens of milliseconds.  The ONTIME [expr], [1n num] inter-

rupt has priority over the ONEX1 interrupt.   So,  the ONTIME  interrupt

can interrupt the ONEX1 interrupt routine.

54

4.20 DESCRIPTION OF STATEMENTS

STATEMENT: ONTIME [expr], [1n num]

MODE: RUN

TYPE: CONTROL

Since  MCS  BASIC-52  processes a line in the millisecond time frame and

the timer/counters on the 8052AH operate in the micro-second time frame,

there is an inherent incompatibility between the timer/counters  on  the

8052AH  and  MCS  BASIC-52.    To  help  solve this situation the ONTIME

[expr], [1n num] statement was devised.  What ONTIME does is generate an

interrupt everytime the SPECIAL FUNCTION OPERATOR, TIME,  is equal to or

greater  than the expression following the ONTIME statement.   Actually,

only the integer portion of TIME is compared to the integer  portion  of

the  expression.    The interrupt forces a GOSUB to the line number ([1n

num]) following the expression ([expr]) in the ONTIME statement.

Since the ONTIME statement uses the SPECIAL FUNCTION OPERATOR, TIME, the

CLOCK1 statement must be executed in order for ONTIME to  operate.    If

CLOCK1 is not executed the SPECIAL FUNCTION OPERATOR,  TIME,  will never

increment and not much will happen.

Since the ONTIME statement generates an interrupt when TIME  is  greater

than or equal to the expression following the ONTIME statement,  how can

periodic interrupts be generated?   That's easy,  the  ONTIME  statement

must be executed again in the interrupt routine:

EXAMPLE:

>10 TIME=0 : CLOCK1 : ONTIME 2,100 : DO

>20 WHILE TIME<10 : END

>100 PRINT "TIMER INTERRUPT AT -", TIME, "SECONDS"

>110 ONTIME TIME+2,100 : RETI

>RUN

TIMER INTERRUPT AT - 2.045 SECONDS

TIMER INTERRUPT AT - 4.045 SECONDS

TIMER INTERRUPT AT - 6.045 SECONDS

TIMER INTERRUPT AT - 8.045 SECONDS

TIMER INTERRUPT AT - 10.045 SECONDS

READY

You  may  wonder  why  the TIME that was printed out was 45 milliseconds

greater than the time that the interrupt was supposed to  be  generated.

That's  because  the  terminal  used in this example was running at 4800

BAUD and takes about 45 milliseconds to print the message  TIMER  INTER-

RUPT AT - " ".

4.20 DESCRIPTION OF STATEMENTS

55

If  the  programmer  does not want this delay,  a variable should be as-

signed to the SPECIAL FUNCTION OPERATOR,  TIME,  at the beginning of the

interrupt routine.

EXAMPLE:

>10 TIME=0 : CLOCK1: ONTIME 2,100: DO

>20 WHILE TIME<10 : END

>100 A=TIME

>110 PRINT "TIMER INTERRUPT AT -",A, "SECONDS"

>120 ONTIME A+2,100 : RETI

>RUN

TIMER INTERRUPT AT - 2 SECONDS

TIMER INTERRUPT AT - 4 SECONDS

TIMER INTERRUPT AT - 6 SECONDS

TIMER INTERRUPT AT - 8 SECONDS

TIMER INTERRUPT AT - 10 SECONDS

READY

Like  the  ONEX1 statement,  the ONTIME interrupt routine must be exited

with a RETI statement.   Failure to do this will "lock-out"  all  future

interrupts.

The ONTIME interrupt has priority over the ONEX1 interrupt.   This means

that the ONTIME interrupt can interrupt  the  ONEX1  interrupt  routine.

This  priority was established because time related functions in control

applications were viewed as critical routines.   If the  user  does  not

want  the  ONEX1  routine  to be interrupted by the ONTIME interrupt,  a

CLOCK0 or a CLEARI statement should be executed at the beginning of  the

ONEX1  routine.    The interrupts would have to be re-enabled before the

end of the ONEX1 routine.   The ONEX1 interrupt cannot interrupt an  ON-

TIME routine.

The ONTIME statement in MCS BASIC-52 is unique, relative to most BASICS.

This  powerful  statement eliminates the need for the user to "test" the

value of the TIME operator periodically throughout the BASIC program.

56

4.21 DESCRIPTION OF STATEMENTS

STATEMENT: PRINT or P.(? VERSION 1.1 ONLY)

MODE: COMMAND and/or RUN

TYPE: INPUT/OUTPUT

The PRINT statement directs  MCS  BASIC-52  to  output  to  the  console

device.  The value of expressions, strings, literal values, variables or

test  strings may be printed out.   The various forms may be combined in

the print list by separating them with commas.   If  the  list  is  ter-

minated  with  comma,  the carriage return/line feed will be suppressed.

P.  is a "shorthand" notation for PRINT.   In  Version  1.1  ?  is  also

"shorthand" notation for PRINT.

EXAMPLES:

>PRINT 10*10,3*3  >PRINT "MCS-51"      >PRINT 5, 1E3

100  9           MCS-51                5  1000

Values  are printed next to one another with two intervening blanks.   A

PRINT statement with no arguments causes a carriage return/line feed se-

quence to be sent to the console device.

SPECIAL PRINT FORMATTING STATEMENTS

TAB([expr])

The TAB ([expr]) function is used in the PRINT statement to  cause  data

to be printed out in exact locations on the output device.  TAB ([expr])

tells  MCS  BASIC-52  which position to begin printing the next value in

the print list.   If the  printhead  or  cursor  is  on  or  beyond  the

specified TAB position, MCS BASIC-52 will ignore the TAB function.

EXAMPLE:

>PRINT TAB (5), "X", TAB (10), "Y"

X     Y

57

SPC ([expr])

The  SPC  ([expr])  function is used in the PRINT statement to cause MCS

BASIC-52 to output the number of spaces in the SPC argument.

EXAMPLE:

>PRINT A,SPC(5),B

may be used to place an additional 5 spaces between the A and B over and

above the two that would normally be printed.

58

4.21 DESCRIPTION OF STATEMENTS

CR

The CR function is interesting and unique to MCS BASIC-52.   When CR  is

used  in a PRINT statement it will force a carriage return,  but no line

feed.   This can be used to create  one  line  on  CRT  device  that  is

repeatedly updated.

EXAMPLE:

>10 FOR I=1 to 1000

>20 PRINT I,CR,

>30 NEXT I

will  cause  the  output to remain only on one line.   No line feed will

ever be sent to the console device.

USING(special characters)

The USING function is used to tell MCS BASIC-52 what format  to  display

the  values that are printed.   MCS BASIC-52 "stores" the desired format

after the USING statement is executed.   So,  all  outputs  following  a

USING statement will be in the format evoked by the last USING statement

executed.    The USING statement need not be executed within every PRINT

statement unless the programmer wants to change the format.    U.  is  a

"shorthand" notation for USING.  The options for USING are as follows:

USING(Fx)  -  This  will  force  MCS  BASIC-52  to  output  all  numbers

using the floating point format.  The value of x determines how many sig-

nificant digits will be printed.   If x equals 0,  MCS BASIC-52 will not

output  any trailing zeros,  so the number of digits will vary depending

upon the number.  MCS BASIC-52 will always output at least 3 significant

digits even if x is 1 or 2.  The maximum value for x is 8.

59

EXAMPLE:

>10 PRINT USING(F3),1,2,3

>20 PRINT USING(F4),1,2,3

>30 PRINT USING(F5),1,2,3

>40 FOR I=10 TO 40 STEP 10

>50 PRINT I

>60 NEXT I

>RUN

1.00 E 0  2.00 E 0  3.00 E 0

1.000 E 0  2.000 E 0  3.000 E 0

1.0000 E O  2.0000 E 0  3.0000 E 0

1.0000 E+1

2.0000 E+1

3.0000 E+1

4.0000 E+1

READY

60

4.21 DESCRIPTIONS OF STATEMENTS

USING(#.#) - This will force MCS BASIC-52 to output all numbers using an

integer and/or fraction format.  The number of "#" 's before the decimal

point represents the number of significant integer digits that  will  be

printed  in  the fraction.   The decimal point may be omitted,  in which

case only integers will be printed.   USING may be abbreviated U.  USING

(###.###),  USING(######)  and  USING(######.##)  are  all  valid in MCS

BASIC-52.   The maximum number of "#" characters is 8.   If MCS BASIC-52

cannot output the value in the desired format (usually because the value

is  too  large)  a  question mark (?) will be printed to console device,

then BASIC will output the number in the FREE FORMAT described below.

EXAMPLE:

>10 PRINT USING(##.##),1,2,3

>20 FOR I=1 TO 120 STEP 20

>30 PRINT I

>40 NEXT I

>RUN

1.00    2.00    3.00

1.00

21.00

41.00

61.00

81.00

? 101

READY

NOTE: The USING(Fx) and the USING(#.#) formats will always  "align"  the

decimal  points  when  printing a number.   This feature makes displayed

columns of numbers easy to read.

USING(0) - This argument lets MCS BASIC-52 determine what format to use.

The rules are simple,  if the number is  between  +-99999999  and  +-.1,

BASIC will display integers and fractions.   If it is out of this range,

BASIC will use the USING(F0) format.   Leading and trailing  zeros  will

always  be  suppressed.    After  reset,  MCS  BASIC-52 is placed in the

USING(0) format.

61

4.22 DESCRIPTION OF STATEMENTS

STATEMENT: PRINT# or P.# (?# VERSION 1.1 ONLY)

MODE: COMMAND and/or RUN

TYPE: INPUT/OUTPUT

The PRINT#,  P.#,  and ?# (in Version 1.1 only) statement does the  same

thing as the PRINT, P. and ? (in Version 1.1 only) statement except that

the output is directed to the list device instead of the console device.

The  BAUD  rate  to  the  list  must  be  initialized by the STATEMENT -

BAUD[expr] before the PRINT#, P.#, or,  ?# statement is used.   All com-

ments that apply to the PRINT,  P.  or ?  statement apply to the PRINT#,

P.#, or ?  statement.   P.# and ?# (in Version 1.1 only) are "shorthand"

notations for PRINT#.

62

4.23 DESCRIPTION OF STATEMENTS

STATEMENTS: PH0., PH1., PH0.#, PH1.#

MODE: COMMAND and/or RUN

TYPE: INPUT/OUTPUT

The  PH0.  and PHI.  statements do the same thing as the PRINT statement

except that the values are printed out in a hexadecimal

format. The PH0. statement suppresses two leading zeros if the number to

be printed is less than 225 (0FFH). The PH1. statement always prints out

four hexadecimal digits.  The character "H" is always printed after  the

number when PH0. or PH1. is used to direct an output. The values printed

are  always  truncated integers.  If the number to printed is not within

the  range  of  valid  integer  (i.e.   between  0  and  65535  (0FFFFH)

inclusive),  MCS  BASIC-52 will default to the normal mode of print.  If

this happens no "H" will be printed out after the value.  Since integers

can  be  entered  in  either  decimal or hexadecimal form the statements

PRINT, PH0.,  and PH1.  can be used to perform to decimal to hexadecimal

and  hexadecimal  to decimal conversion.  All comments that apply to the

PRINT statement apply to the PH0. AND PH1.  statements.  PH0.# AND PH1.#

do the same thing as PH0.  and PH1. respectively, except that the output

is directed to the list device instead of the console device.

EXAMPLES:

>PH0. 2*2    >PH1. 2*2    >PRINT 99H    >PH0. 100

04H          0004H        153           64H

>PH0. 1000   >PH1. 1000   >P. 3E8H      >PH0. PI

3E8H         03E8H        1000          03H

63

4.24 DESCRIPTION OF STATEMENTS

STATEMENT: PRINT@, PH0.@, PH1.@ (VERSION 1.1 ONLY)

MODE: COMMAND AND/OR RUN

TYPE: INPUT/OUTPUT

The PRINT@ (P.@ OR ?@, PH0.@,  and PH1.@ statements do the same thing as

the PRINT (P.@ or ?@),  PH0., and PH1. statements except that the output

is directed to a user defined output  driver.  These  statements  assume

that the user has placed an assembly language output routine in external

code memory location 403CH. To enable the @ driver routine the user must

SET BIT 27H (39D) in the internal memory of the MCS BASIC-52 device. BIT

27H  (39D) is BIT 7 of internal memory location 24H (36D).  This BIT can

be set by the BASIC statement DBY(24H)=DBY(24H).OR. 80H or by a user sup-

plied assembly language routine. If the user evokes the @ driver routine

and this bit is not set,  the output will be  directed  to  the  console

driver.  The  only reason this BIT must be set to enable the @ driver is

that it adds a certain degree of  protection  from  accidentally  typing

LIST@  when  no assembly language routine exist.  The philosophy here is

that if the user sets the bit,the user provides the driver or else!!!

When MCS BASIC-52 calls the user output driver routine  at  location  at

430CH,  the byte to output is in the accumulator and R5 of register bank

0 (RBO).  The user may modify the accumulator (A) and the  data  pointer

(DPTR) in the assembly language output routine, but cannot modify any of

the registers in RBO. This is intended to make it real easy for the user

to  implement  a parallel or serial output driver without having to do a

PUSH or a POP.

64

4.25 DESCRIPTION OF STATEMENTS

STATEMENT: PUSH[expr]

MODE: COMMAND AND / OR RUN

TYPE: ASSIGNMENT

The arithmetic expression,  or expressions following the PUSH  statement

are  evaluated  and  then sequentially placed on MCS BASIC-52's ARGUMENT

STACK.  This statement,  in conjunction with the POP statement provide a

simple means of passing parameters to assembly language routine. In addi-

tion,  the  PUSH  and  POP  statements can be used to pass parameters to

BASIC subroutines and to "SWAP" variables.  The last value  PUSHED  onto

the  ARGUMENT  STACK  will  be  the  first value POPPED off the ARGUMENT

STACK.

VARIATIONS:

More than one expression can be pushed onto the ARGUMENT  stack  with  a

single  PUSH statement.  The expressions are simply followed by a comma:

PUSH[expr],[expr],........[expr].  The last value PUSHED onto the  ARGU-

MENT  STACK  will  be the last expression [expr] encountered in the PUSH

STATEMENT.

EXAMPLES:

SWAPPING         SUBROUTINE

VARIABLES         PASSING

>10 A=10         >10 PUSH 1,3,2

>20 B=20         >20 GOSUB 100

>30 PRINT A,B    >30 POP R1,R2

>40 PUSH A,B     >40 PRINT R1,R2

>50 POP A,B      >50 END

>60 PRINT A,B    >100 REM QUADRATIC A=2, B=3, C=1 IN EXAMPLE

>RUN             >110 POP A,B,C,

>120 PUSH  (-B+SQR (B*B-4*A*C) )/(2*A)

10  20           >130 PUSH  (-B-SQR (B*B-4*A*C) )/(2*A)

20  10           >140 RETURN

>RUN

READY

>                -1 - 5

READY

>

65

4.26 DESCRIPTION OF STATEMENTS

STATEMENT: POP[var]

MODE: COMMAND AND / OR RUN

TYPE: ASSIGNMENT

The top of the ARGUMENT STACK is assigned to the variable following  the

POP statement and the ARGUMENT STACK is "POPPED"(i.e. incremented by 6).

Values can be placed on the stack by either the PUSH statement or by as-

sembly language CALLS. NOTE-If a POP statement is executed and no number

is on the ARGUMENT STACK, an A-STACK ERROR will occur.

VARIATIONS:

More  than  one  variable  can  be  popped off the ARGUMENT stack with a

single POP statement. The variables are simply followed by a comma (i.e.

POP [var],[var],........[var]).

EXAMPLES:

see PUSH statement.

COMMENT:

The PUSH and POP statements are unique to MCS BASIC-52.  These  powerful

statements  can  be used to "get around" the GLOBAL variable problems so

often encountered in BASIC PROGRAMS.  This  problem  arises  because  in

BASIC  the  "main"  program and all subroutines used by the main program

are required to use the same variable names (i.e. GLOBAL VARIABLES).  It

is not always convenient to use the same variables in a subroutine as in

the  main program and you often see programs re-assign a number of vari-

ables (i.e.  A=Q) before a GOSUB STATEMENT  is  executed.  If  the  user

reserves  some  variable  names JUST for subroutines (i.e.  S1,  S2) and

passes variables on the stack as shown in the previous example, you will

avoid any GLOBAL variable problems in MCS BASIC-52.

66

4.27 DESCRIPTION OF STATEMENTS

STATEMENT: PWM [expr], [expr], [expr]

MODE: COMMAND and/or RUN

TYPE: INPUT/OUTPUT

PWM stands for PULSE WIDTH MODULATION.  What it does is generate a  user

defined pulse sequence on P1.2 (bit 2 of I/O PORT 1) of the MCS BASIC-52

device.  The first expressions following the PWM statement is the number

of clock cycles the pulse will remain high.  A clock cycle is  equal  to

12/XTAL,  which is 1.085 microseconds at 11.0592 MHz. The second expres-

sion is the number of clock cycles the pulse will  remain  low  and  the

third  expression  is the total number of cycles the user wishes to out-

put.  All expressions in the PWM statement must be valid integers  (i.e.

between 0 and 65535 (0FFFFH) inclusive).  Additionally,  the the minimum

value for the first two expressions in the PWM statement is 25.

The PWM statement can be used to create "audible" feedback in a  system.

In a addition, just for fun, the programmer can play music using the PWM

system. More details about using the PWM statement are in the appendix.

EXAMPLE:

>PWM  100, 100, 1000

At  11.0592  MHz  would generate 1000 cycles of a square wave that has a

period of 217 microseconds (4608 Hz) on P1.2.

67

4.28 DESCRIPTION OF STATEMENTS

STATEMENTS: REM

MODE: RUN (Version 1.0) COMMAND AND/OR RUN (Version 1.1)

TYPE: CONTROL-PERFORMS NO OPERATION

REM is short for REMark. It does nothing, but allows the user to add com-

ments to program. Comments are usually needed to make a program a little

easier to understand.  Once a REM statement appears on a line the entire

line  is assumed to be remark,  so a REM statement may not be terminated

by a colon (:),  however,  it may be placed after a colon.  This can  be

used to allow the programmer to place a comment on each line.

EXAMPLES:

>10 REM INPUT ONE VARIABLE

>20 INPUT A

>30 REM INPUT ANOTHER VARIABLE

>40 INPUT B

>50 REM MULTIPLY THE TWO

>60 Z=A*B

>70 REM PRINT THE ANSWER

>80 PRINT Z

>10 INPUT A : REM INPUT ONE VARIABLE

>20 INPUT B : REM INPUT ANOTHER VARIABLE

>30 Z=A*B : REM MULTIPLY THE TWO

>40 PRINT Z :REM PRINT THE ANSWER

The following will NOT work because the entire line would be interpreted

as a REMark, so the PRINT statement would not be executed:

>10 REM PRINT THE NUMBER : PRINT A

NOTE-The  reason  the  REM  statement was made executable in the command

mode in Version 1.1 of MCS BASIC-52 is that if  the  user  is  employing

some type of UPLOAD/DOWNLOAD routine with a computer, this lets the user

insert REM statements, without line numbers in the text and not download

them to the MCS BASIC-52 device. This helps to conserve memory.

68

4.29 DESCRIPTION OF STATEMENTS

STATEMENT: RETI

MODE: RUN

TYPE: CONTROL

The  RETI  is used to exit from to interrupts that are handled by an MCS

BASIC-52 program. specifically, the ONTIME and the ONEXI interrupts. The

RETI statement does the same thing as the RETURN statement  except  that

it also clears a software interrupt flags so interrupts can again be ac-

knowledged. If the user fails to execute the RETI statement in the inter-

rupt procedure, all future interrupts will be ignored.

69

4.30 DESCRIPTION OF STATEMENTS

STATEMENT: STOP

MODE: RUN

TYPE: CONTROL

The  STOP  statement allows the programmer to break program execution at

specific points in a program.   After a program is STOPped variables can

be  displayed  and/or modified.  Program execution may be resumed with a

CONTinue command. The purpose of the STOP statement is to allow for easy

program "debugging." More details of the STOP-CONT sequence are  covered

in the DESCRIPTION OF COMMAND-CONT section of this manual.

EXAMPLE:

>10 FOR I=1 to 100

>20 PRINT I

>30 STOP

>40 NEXT I

>RUN

1

STOP - IN LINE 40

READY

>CONT

2

Note  that  the  line number printed out after the STOP statement is ex-

ecuted is the line number following the STOP statement, NOT the line num-

ber that contains the STOP statement!!!

70

4.31 DESCRIPTION OF STATEMENTS

STATEMENT: STRING [expr], [expr]

MODE: COMMAND and/or RUN

TYPE: CONTROL

The string [expr],  [expr] statement allocates memory for strings.  Ini-

tially,  no  memory  is  allocated for strings.  If the user attempts to

define a string with a statement such as LET $(1)="HELLO" before  memory

has been allocated for strings, a MEMORY ALLOCATION ERROR will be genera-

ted.  The  first expression in the STRING [expr],[expr] statement is the

total number of bytes the user wishes to allocate  for  string  storage.

The  second  expression  denotes the maximum number of bytes that are in

each string.  These two numbers determine the total  number  of  defined

string variables.

You  might think that the total number of defined strings would be equal

to the first expression in the STRING [expr],[expr] statement divided by

the second expression. Ha,ha, do not be so presumptous. MCS BASIC-52 re-

quires one additional byte for each string,  plus  one  additional  byte

overall.  This  means  that  the  statement STRING 100,10 would allocate

enough memory for 9 string variables,  ranging from $(0) to $(8) and all

of  the  100  allocated  bytes would be used.  Note that $(0) is a valid

string in MCS BASIC-52.

After memory is allocated for string storage, neither commands,  such as

NEW nor statements,  such as CLEAR,  will "de-allocate" this memory. The

only way memory can be de-allocated is to execute a  STRING  0,0  state-

ment. STRING 0,0 will allocate no memory to string variables.

IMPORTANT NOTE

Every time the STRING [expr],[expr] statement is executed,  MCS BASIC-52

executes the equivalent of a CLEAR statement.  This is a  necessity  be-

cause  string  variables  and  numeric variable occupy the same external

memory space. So, after the STRING statement is executed,  all variables

are "wiped-out."Because of this, string memory allocation should be per-

formed  early  in  a program (like the first statement or so) and string

memory should never be "re-allocated" unless the programmer  is  willing

to destroy all defined variables.

71

4.32 DESCRIPTION OF STATEMENTS

STATEMENTS: UI1 and UI0 (USER INPUT)

MODE: COMMAND and/or RUN

TYPE: CONTROL

UI1

The  UI1  statement  permits  the  user  to write specific console input

drivers for MCS BASIC-52. After UI1 is executed BASIC will call external

program memory location 4033H when a console  input  is  requested.  The

user  must  provide  a  JUMP  instruction  to an ASSEMBLY LANGUAGE INPUT

ROUTINE at this location.  The appropriate ASCII input from this routine

is placed in the 8052AH's accumulator and the user input routine returns

back  to  BASIC  by executing an ASSEMBLY LANGUAGE RET instruction.  The

user must NOT modify any of the 8052AH"s registers in the assembly  lan-

guage  program  with the exception of the MEMORY and REGISTER BANK allo-

cated to the USER.  THE ASSEMBLY LANGUAGE LINKAGE section of this manual

explains what memory MCS BASIC-52 allocates to the user and how the user

may allocate additional memory if needed.

In addition to providing the INPUT driver routine for the UI1 statement,

the user must also provide a CONSOLE STATUS CHECK routine.  This routine

checks to see if the CONSOLE DEVICE has a character ready for MCS BASIC-

52 to read. BASIC CALLS external memory location 4036H to check the CON-

SOLE STATUS. The CONSOLE STATUS ROUTINE sets the CARRY BIT to 1 (C=1) if

a character is ready for BASIC to read and CLEARS the CARRY BIT (C=0) if

no character is ready. Again,  the contents of the REGISTERS must not be

changed.   MCS BASIC-52 uses the CONSOLE STATUS CHECK routine to examine

the keyboard for a control-C  character  during  program  execution  and

during a program LISTING.

This routine is also used to perform the GET operation.

UI0

The UI0 statements assigns the console input console routine back to the

software drivers resident on the MCS BASIC-52 device. UI0 and UI1 may be

placed  anywhere within a program.  This allows the BASIC program to ac-

cept inputs from different devices at different times.

72

NOTE: The UI0 and UI1 function is controlled by  BIT  30  (1EH)  in  the

8052AH's  internal memory.  BIT 30 is in   internal memory location 35,6

(23.6H) i.e.  the sixth bit in internal memory location 35  (23H).  When

BIT 30 is SET (BIT 30 = 1), the user routine will be called. When BIT 30

is  CLEARED (BIT 30 = 0),  THE MCS BASIC-52 input driver routine will be

used. The assembly language

programmer can use this information to change the input device selection

in assembly language.

73

4.33 DESCRIPTION OF STATEMENTS

STATEMENTS: UO1 and UO0 (USER OUTPUT)

MODE: COMMAND AND/OR RUN

TYPE: CONTROL

UO1

The UO1 STATEMENT permits the user  to  write  specific  console  output

drivers for MCS BASIC-52. After UO1 is executed BASIC will call external

program  memory  location 4030H when a console output is requested.  The

user must provide a JUMP instruction  to  an  ASSEMBLY  LANGUAGE  OUTPUT

ROUTINE  at  this location.  MCS BASIC-52 places the output character in

REGISTER 5 (R5) of REGISTER BANK 0 (RBO). The user returns back to BASIC

executing an assembly language RET instruction. The user must NOT modify

any of the 8052AH's REGISTERS, including the ACCUMULATOR during the user

output procedure with the exception of the MEMORY and REGISTER BANK allo-

cated to the user. UO1 gives the user the freedom to write custom output

routines MCS BASIC-52.

UO0

UO0 STATEMENT assigns the console output routine back  to  the  software

drivers  resident on the MCS BASIC-52 device.  UO0 and UO1 may be placed

anywhere within a program.  This allows  the  BASIC  program  to  output

characters to different devices at different times.

NOTE:  The  UO0  and  UO1  function is controlled by BIT 28 (1CH) in the

8052AH's internal memory. BIT 28 is in the internal memory location 35.4

(23.4H),  i.e.  the fourth bit in the internal memory location  35(28H).

When BIT 28 is SET (BIT 28 = 1),  the user routines will be called. When

BIT 28 is cleared, (BIT 28 = 0), the MCS BASIC-52 output drivers will be

used.  The assembly language programmer  can  use  this  information  to

change the output device selection in assembly language.

74

4.34 DESCRIPTION OF STATEMENTS

STATEMENT: IDLE (VERSION 1.1 ONLY)

MODE: RUN

TYPE: CONTROL

The IDLE statement forces the MCS BASIC-52 device into a "wait until in-

terrupt mode."  Execution of statements is halted until either an ONTIME

[expr],  [1n num] or an ONEX1 [1n num] interrupt is received.   The user

must make sure that one or both of these interrupts  have  been  enabled

before  executing   the IDLE instruction or else the MCS BASIC-52 device

will enter a "wait forever mode" and for all practical purposes the sys-

tem will have crashed.

When an ONTIME [expr],  [1n num] or an ONEX1 [1n num] is received  while

in  the  IDLE  mode,  the MCS BASIC-52 device will execute the interrupt

routine,  the execute the  statement  following  the  IDLE  instruction.

Hence, the execution of the IDLE instruction is terminated when an inter-

rupt is received.

While in the IDLE mode, the MCS Basic-52 device asserts the /DMA ACKNOW-

LEDGE  pin (PORT 1,  BIT 6 = 0) to indicate that the IDLE instruction is

active and that no external bus activity will occur.  This PIN is physi-

cally pin 7 on the MCS BASIC-52 device.   When the MCS  BASIC-52  device

exits  from the IDLE mode,  this pin is placed back into the logically 1

(non-active) state.

The may also exit from the IDLE mode with an assembly language interrupt

routine.   This is accomplished by setting BIT 33 (21H) (which is in BIT

addressable RAM location 36.1) when returning from the assembly language

interrupt routine.  If this bit is not set by the user, the MCS BASIC-52

device  will  remain  in  the  IDLE mode when the user assembly language

routine returns to BASIC.

An attempt to execute the IDLE statement in the direct mode will yield a

BAD SYNTAX ERROR.

75

4.35 DESCRIPTION OF STATEMENTS

STATEMENT: RROM [integer] (VERSION 1.1 ONLY)

MODE: COMMAND AND/OR RUN

TYPE: CONTROL

RROM stands for RUN ROM.   What it does is select a program in the EPROM

file,  then  execute the program.   The integer after the RROM statement

selects what program in the EPROM file,  then execute the program.   The

integer  after the RROM statement selects what program in the EPROM file

is to be executed.   In the COMMAND mode RROM 2 would be  equivalent  to

typing ROM 2, then RUN.  But, notice that RROM [integer] is a statement.

This  means  that a program that is already executing can actually force

the execution of a completely different program that  is  in  the  EPROM

file.  This gives the user the ability to "change programs" on the fly.

If  the  user executes a RROM [integer] statement and an invalid integer

is entered (say 6 programs are contained in the EPROM file and the  user

enters RROM 8, or no EPROM is in the system), no error will be generated

and MCS BASIC-52 will execute the statement following the RROM [integer]

statement.

NOTE - Everytime the RROM [integer] statement is executed, all variables

and  strings  are set equal to zero,  so variables and strings CANNOT be

passed from one program to another by using the  RROM  [integer]  state-

ment.  Additionally, all MCS BASIC-52 evoked interrupts are cleared.

76

4.36 DESCRIPTION OF STATEMENTS

STATEMENTS: LD@ [expr] and ST@ [expr] (VERSION 1.1 ONLY)

MODE: COMMAND AND/OR RUN

TYPE: INPUT/OUTPUT

ST@

The ST@ [expr] statement lets the user specify where MCS BASIC-52 float-

ing point numbers are to be stored.  The expression [expr] following the

ST@  statement specifies the address of where the number is to be stored

and the number is assumed to be on the argument stack.   The ST@  [expr]

statement  is  designed  to  be  used in conjunction with the LD@ [expr]

statement.   The purpose of these two statements is to allow the user to

save  floating point numbers anywhere in memory with the assumption that

the user will employ some type of battery back-up or non-volatile scheme

with this memory.

LD@

The LD@ [expr] statement lets the user retrieve floating  point  numbers

that  were  saved with the ST@ [expr] statement.   The expression [expr]

following the LD@ statement specifies where the  number  is  stored  and

after  executing  the LD@ [expr] statement,  the number is placed on the

argument stack.

EXAMPLE: Saving and retrieving a ten element array at location array  at

location 0F000H

10   REM *** ARRAY SAVE ***

20   FOR I = 0 TO 9

30   PUSH A (I) : REM PUT AWAY VALUE ON STACK

40   ST@ 0F005H+6*I : REM STORE IT, SIX BYTES PER NUMBER

50   NEXT I

60   REM *** GET ARRAY ***

70   FOR I = 0 TO 9

80   LD@ 0F005H+6*I

90   POP B (I)

100  NEXT I

Remember  that  each  floating point number requires 6 bytes of storage.

Also note that expression in the ST@ [expr]  and  LD@  [expr]  statement

point to the most significant by the of the stored number.   Hence,  ST@

(0F005H) would save the number  in  locations  0F005H,  0F004H,  0F003H,

0F002H, 0F001H, and 0F000H.

4.37 DESCRIPTION OF STATEMENTS

77

STATEMENT: PGM

MODE: COMMAND AND/OR RUN

TYPE: INPUT/OUTPUT

The  PGM  statement  gives  the  user the ability to program an EPROM or

EEPROM while executing a BASIC program.  The PGM statement requires that

the user set up internal memory locations  18H  (24D),  19H  (25D),  1AH

(26D),  1BH  (27H),  1EH (30D) and 16H (31D).   Note that these internal

memory locations are normally reserved for the user!!

The User must initialize these internal memory locations with the follow-

ing:

EXAMPLE:

LOCATION              CONTENTS

1BH: 19H   THE ADDRESS OF THE SOURCE INFORMATION THAT IS TO BE

(27D: 25D)  PROGRAMMED INTO THE EPROM, LOCATION 19H IS THE LOW

BYTE AND LOCATION 1BH IS THE HIGH BYTE

1AH: 18H   THE ADDRESS - 1 OF THE EPROM LOCATION (S) THAT ARE

(26D: 24D)  TO BE PROGRAMMED, LOCATION 18H IS THE LOW BYTE AND

LOCATION 1AH IS THE HIGH BYTE

1FH: 1EH   THE NUMBER OF BYTES THAT THE USER WANTS TO PROGRAM

(31D: 30D)  LOCATION 1EH IS THE LOW BYTE AND LOCATION 1FH IS

THE HIGH BYTE

The user must also initialize the width of the desired EPROM programming

pulse and store the value in internal memory locations 40H  (64D)  (high

byte)  and 41H (65D) (low byte).   The reload for a 50 millisecond EPROM

programming pulse is calculated as follows:

10   REM R = RELOAD VALUE, W = WIDTH IN SECONDS (50 MILLISECONDS)

20   W = .05

30   R = 65536 - W * XTAL/12

40   DBY (40H) = R/256

50   DBY (41H) = R .AND. 0FFH

In addition,  the user must also SET or CLEAR BIT 38.3 (26.3H) to select

the  INTELligent EPROM programming algorithm.   The Bit is SET to select

INTELligent programming and CLEARED to select the normal 50  millisecond

algorithm.   To SET the BIT,  execute a DBY(38) = DBY(38) .OR. 8H State-

ment, to CLEAR the BIT, execute a DBY(38) = DBY(38) .AND.  OF7H instruc-

tion.

4.37 DESCRIPTION OF STATEMENTS

IMPORTANT NOTE!

78

When  executed  in the RUN mode,  The PGM statement will not generate an

error if the EPROM fails to program properly.   Instead,  the control of

the  program  will  be  passed  back  to  the  user just as if the EPROM

programmed properly.   The user must then examine locations 1EH and 1FH.

If  the contents of locations 1EH and 1FH both equal to zero,  the EPROM

programmed properly.  If they do not,  then an ERROR occurred during the

programming  process.    The  user can then examine locations 1AH:18H to

determine what location in the EPROM failed to program.

Well,  this sounds like a lot to do just to program an EPROM,  but  it's

not  so  bad.    The  following  program  is  an  example of a universal

EPROM/EEPROM programmer built around MCS BASIC-52.    This  program  can

program  a  block  of  RAM  into an EPROM or EEPROM that is addressed at

8000H or above.

EXAMPLE:

10    PRINT "UNIVERSAL PROM PROGRAMMER: : PRINT "WHAT TYPE OF

OF DEVICE ?"

20    PRINT : PRINT "1 = EEPROM" : PRINT "2 = INTELLIGENT

EPROM"

30    PRINT "3 = NORMAL (50 MS) EPROM" : PRINT : INPUT "TYPE

(1,2,3) - ",T

40    ON (T-1) GOSUB 340,350,360

50    REM this sets up intelligent programming if needed

60    IF W=. 001 THEN DBY(26)=DBY)26).OR. ELSE DBY(26) .AND.

OF7H

70    REM calculate pulse width and save it

80    PUSH (65536-(W*XTAL/12)) : GOSUB 380

90    POP G1: DBY(40H)=G1 : POP G1 : DBY(41H)=G1 : PRINT

100   INPUT " STARTING DATA ADDRESS - ",S : IF S<512.OR.

S>0FFFFH THEN 100

110   PRINT : INPUT "ENDING DATA ADDRESS - ",E

120   IF E<S.OR.E>0FFFFH THEN 110

130   PRINT : INPUT " PROM ADDRESS - ",P : IF P<8000H.OR.

P>0FFFFH THEN 130

140   REM calculate the number of bytes to program

150   PUSH (E-S)+1 : GOSUB 380 : POP G1 : DBY(31)=G1 :

POP G1 : DBY(30)=G1

160   REM set up the eprom address

170   PUSH (P-1) : GOSUB 380 : POP G1 : DBY(26)=G1 :

POP G1 : DBY(24)=G1

180   REM set up the source address

190   PUSH S : GOSUB 380 : POP G1 : DBY(27)=G1 :

POP G1 : DBY(25)=G1

200   PRINT : PRINT "TYPE A 'CR' ON THE KEYBOARD WHEN READY

TO PROGRAM"

210   REM wait for a 'cr' then program the eprom

220   X=GET : IF X<>0DH THEN 220

230   REM program the eprom

240   PGM

79

250   REM see if any errors

260   IF (DBY(30).OR.DBY(31))=0 THEN PRINT "PROGRAMMING

COMPLETE" : END

270   PRINT : PRINT "***ERROR***ERROR***ERROR***" : PRINT

280   REM these routines calculate the address of the source

and

290   REM eprom location that failed to program

300   S1=DBY(25)+256*DBY(27) : S1=S1-1 : D1=DBY(24)+256*DBY(26)

310   PH0. "THE VALUE "'XBY(S1), : PH1. "WAS READ AT

LOCATION ",D1 : END

320   PH0. "THE EPROM READ ",XBY(D1), : PH1. " AT LOCATION ",

D1 : END

330   REM these subroutines set up the pulse width

340   W=.0005 : RETURN

350   W=.001 : RETURN

360   W=.05 : RETURN

370   REM this routine takes the top of stack and returns

high, low bytes

380   POP G1 : PUSH (G1.AND.0FFH) : PUSH (INT(G1/256)) :

RETURN

80

CHAPTER 5

Description of Arithmetic/Logic Operators and Expressions

5.1 DUAL OPERAND OPERATORS

MCS BASIC-52  contains  a  complete  set  of  arithmetical  and  logical

operators.    Operators  are  divided  into two groups,  dual operand or

dyadic operators and single operand or unary operators.  The generalized

form of all dual operand instructions is as follows:

[expr] OP [expr], where OP is one of the following operators:

________________

+ ADDITION OPERATOR

EXAMPLE:

PRINT 3+2

5

/ DIVISION OPERATOR

EXAMPLE:

PRINT 100/5

20

** EXPONENTIATION OPERATOR

Raises the first expression to the power of the second expression.   The

power  any  number can be raised to is limited to 255.   The notation **

was instead of the sometimes used  symbol because the "up arrow"  symbol

appears  different  on various terminals.   To eliminate the ** notation

was chosen.

EXAMPLE:

PRINT 2**3

8

* MULTIPLICATION OPERATOR

EXAMPLE:

PRINT 3*3

9

81

- SUBTRACTION OPERATOR

EXAMPLE:

PRINT 9-6

3

82

5.1 DUAL OPERAND OPERATIONS

.AND. LOGICAL AND OPERATOR

EXAMPLE:

PRINT 3. AND. 2

2

.OR. LOGICAL OR OPERATOR

EXAMPLE:

PRINT 1. OR. 4

5.

.XOR. LOGICAL EXCLUSIVE OR OPERATOR

EXAMPLE:

PRINT 7. XOR. 6

1

COMMENTS ON LOGICAL OPERATORS .AND., AND .XOR.

These operators perform a BIT-WISE logical function on  valid  INTEGERS.

That  means  both arguments for these operators must between 0 and 65535

(0FFFFH) inclusive.  If they are not,  MCS BASIC-52 will generate a  BAD

ARGUMENT ERROR. All non-integer values are truncated. NOT rounded.

You  may  wonder why the notation .OP.  was chosen for the logical func-

tions.  The only reason for this is that  MCS  BASIC-52  eliminates  ALL

spaces when it processes a user line and inserts spaces before and after

STATEMENTS  when  it LISTS a user program.  MCS BASIC-52 does not insert

space before and after operators.  So,  if the user types in a line such

as  10 A = 10 * 10,  this line will be listed as 10 A=10*10.  All spaces

entered by the user before and after the operator  will  be  eliminated.

The  .OP.  notation was chosen for the logical operators because entered

as 10 B = A AND B would be listed as 10 B= AANDB. This just look confus-

ing, so the dots were added to the logical instructions and the previous

example would be listed as 10 B = A.AND.B, which is easier to read.

83

5.2.1 UNARY OPERATORS- GENERAL PURPOSE

ABS([expr])

Returns the ABSOLUTE VALUE of the expression.

EXAMPLES:

PRINT ABS(5)       PRINT ABS(-5)

5                   5

NOT([expr]

Returns a 16 bit one's complement of the expression. The expression must

be a valid integer (i.e.  between 0 and 65535 (0FFFF)  inclusive).  Non-

integers will be truncated, not rounded.

EXAMPLES:

PRINT NOT(65000)    PRINT NOT(0)

535                 65535

INT([expr])

Returns the integer portion of the expression.

EXAMPLES:

PRINT INT(3.7)      PRINT INT(100.876)

3                   100

SGN([expr])

Will return a value of +1 if the argument is greater than zero,  zero if

the argument is equal to zero, and -1 if the argument is less than zero.

EXAMPLES:

PRINT SGN(52)       PRINT SGN(0)        PRINT SGN(-8)

1                   0                   -1

84

5.2.1 UNARY OPERATORS-GENERAL PURPOSE

SQR([expr])

Returns the square root of the argument.  The argument may not  be  less

than zero. The result returned will be accurate to within +/- a value of

5 on the least significant digit.

EXAMPLES:

PRINT SQR(9)        PRINT SQR(45)       PRINT SQR(100)

3                   6.7082035           10

RND

Returns  a pseudo-random numbers in the range between 0 and 1 inclusive.

The RND operator uses a 16-bit binary seed and generates  65536  pseudo-

random numbers before repeating the sequence.  The numbers generated are

specifically between 0/65535  and  65535/65535  inclusive.  Unlike  most

BASICS, the RND operator in MCS BASIC-52 does not require an argument or

a  dummy  argument.  In  fact,  if  an  argument is placed after the RND

operator, a BAD SYNTAX error will occur.

EXAMPLES:

PRINT RND

30278477

PI

PI is not really an operator, it is a stored constant.  In MCS BASIC-52,

PI  is stored as 3.1415926.  Math will notice that PI is actually closer

to 3.141592653,  so proper rounding  for  PI  should  yield  the  number

3.1415927.  The reason MCS BASIC-52 uses a 6 instead of a 7 for the last

digit is that errors in the SIN,  COS and TAN operators were found to be

greater when 7 was used instead of 6. This is because the number PI/2 is

needed for these  calculations and it is desirable,  for the sake of ac-

curacy to have the equation PI/2 + PI/2 = PI hold true.  This cannot  be

done  if the last digit is PI is an odd number,  so the last digit of PI

was rounded to 6 instead of 7 to make these calculations more accurate.

85

5.2.2 UNARY OPERATORS-LOG FUNCTIONS

LOG([expr])

Returns the natural logarithm of the  argument.  The  argument  must  be

greater than 0. This calculation is carried out to 7 significant digits.

EXAMPLES:

PRINT LOG(12)        PRINT LOG(EXP(1))

2.484906             1

EXP([EXPR])

This function raises the number "e" (2.7182818) to the power of the argu-

ment.

EXAMPLES:

PRINT EXP(1)         PRINT EXP(LOG(2))

2.7182818            2

5.2.3 UNARY OPERATORS-TRIG FUNCTIONS

SIN([expr])

Returns  the SIN of the argument.  The argument is expressed in radians.

Calculations are carried out to 7 significant digits.  The argument must

be between + 200000.

EXAMPLES:

PRINT SIN(PI/4)      PRINT SIN(0)

.7071067             0

COS([expr])

Returns  the COS of the argument.  The argument is expressed in radians.

Calculations are carried out to 7 significant digits.  The argument must

be between +200000.

EXAMPLES:

PRINT COS(PI/4)      PRINT COS(0)

.7071067             1

86

5.2.3 UNARY OPERATORS-TRIG FUNCTIONS

TAN([expr])

Returns  the TAN of the argument.  The argument is expressed in radians.

The argument must be between +-200000.

EXAMPLES:

PRINT TAN(PI/4)      PRINT TAN(0)

1                    0

ATN([expr])

Returns the ARCTANGENT of the argument.  The result is in radians.  Cal-

culations  are  carried  out  to 7 significant digits.  The ATN operator

returns a result between -PI/2(3.1415926/2) and PI/2.

EXAMPLES:

PRINT ATN(PI)        PRINT ATN(1)

1.2626272            .78539804

COMMENTS ON TRIG FUNCTIONS

The SIN, COS,  and TAN operators use a Taylor series to to calculate the

function.  These  operators first reduce the argument to a value that is

between 0 and PI/2. This reduction is accomplished by the following equa-

tion:

REDUCED ARGUMENT = (user arg/PI - INT(user arg/PI) *PI

The REDUCED ARGUMENT, from the above equation, will be between 0 and PI.

The REDUCED ARGUMENT is then tested to see if it is greater  than  PI/2.

If it is,  then it is subtracted from PI to yield the final value. If it

isn't, then the REDUCED ARGUMENT is the final value.

Although this method of angle reduction provides a simple and economical

means of generating the appropriate arguments for a Taylor series, there

is an accuracy problem associated  with  this  technique.  The  accuracy

problem  is  noticed when the user argument is large (i.e.  greater than

1000). That is because significant digits, in the decimal (fraction) por-

tion of REDUCED ARGUMENT  are  lost  in  the  (user  arg/PI  -  INT(user

arg/PI))  expression.  As a general rule,  try to keep the arguments for

the TRIG functions as small as possible!

5.3 UNDERSTANDING PRECEDENCE OF OPERATORS

87

The hierarchy of mathematics dictates that some operations  are  carried

before others. If you understand the hierarchy of mathematics, it is pos-

sible to write complex expressions using only a minimum amount of paren-

theses.  It's  easy to illustrate what precedence is all about,  examine

the following question:

4+3*2 = ?

Should you add (4+3) then multiply seven by 2,  or should  you  multiply

(3*2)  then add 4?  Well,  the hierarchy of mathematics says that multi-

plication has precedence over addition,  so  you  would  multiply  (3*2)

first then add 4. So,

4+3*2 = 10

The  rules  for the hierarchy of math are simple.  When an expression is

scanned from left to right  an  operation  is  not  performed  until  an

operator of lower or equal precedence is encountered. In the example ad-

dition   could  not  be  performed  because  multiplication  has  higher

precedence.  The precedence of operators from highest to lowest  in  MCS

BASIC-52 is as follows:

1)OPERATORS THAT USE PARENTHESES ()

2)EXPONENTIATION (**)

3)NEGATION (-)

4)MULTIPLICATION (*) AND DIVISION (/)

5)ADDITION (+) AND SUBTRACTION (-)

6)RELATIONAL EXPRESSIONS (=,<>,>,>=,<,<=)

7)LOGICAL AND (.AND.)

8)LOGICAL OR (.OR.)

9)LOGICAL XOR (.XOR.)

Relative  to  operator  precedence,  the rule of thumb should always be;

when it doubt, use parentheses.

88

5.4 HOW RELATIONAL EXPRESSIONS WORK

Relational expressions involve the  operators  =,<>,>,>=,<,and<=.  These

operators are typically used to "test" a condition. In MCS BASIC-52 rela-

tional operators return a result of 65535 (0FFFFH) if the relational ex-

pressions  is  true,  and  a result of 0,  if the relation expression is

false. But, where is the result returned? It is returned to the argument

stack. Because of this,  it's possible to actually display the result of

a relational expression.

EXAMPLES:

PRINT 1=0     PRINT 1>0     PRINT A<>A     PRINT A=A

0             65535         0              65535

It  may  seem  strange to have a relational expression actually return a

result,  but it offers a unique benefit in that  relational  expressions

can   actually   be  "chained"  together  using  the  logical  operators

.AND.,.OR.  and .XOR..  This makes it possible to test a rather  complex

condition with ONE statement.

EXAMPLE:

>10 IF A<B. AND. A>C. OR. A>D THEN.........

Additionally, the NOT([expr]) operator can be used.

EXAMPLE:

>10 IF NOT(A>B). AND. A<C THEN.........

By "changing" together relational expressions with logical operators, it

is possible to test very particular conditions with one statement.  When

using logical operators to link together relational expressions,  it  is

very  important  that  the  programmer  pay  careful  attention  to  the

precedence of operators.  The  logical  operators  were  assigned  lower

precedence, relative to relational expressions, just to make the linking

of relational expressions possible without using parentheses.

89

CHAPTER 6

Description of String Operators

6.1 WHAT ARE STRINGS?

A  string  is  a  character  or a bunch of characters that are stored in

memory.  Usually,  the characters stored in a string make up a word or a

sentence.  Strings  are  handy because they allow the programmer to deal

with words instead of numbers.  This is useful because it allows one  to

write "friendly" programs, where individuals can be referred to by their

names instead of number.

MCS  BASIC-52 contains ONE dimensioned string variable,  $([expr]).  The

dimension of the string variable (the [expr] value)  ranges  from  0  to

254.  This  means  that 255 different strings can be defined and manipu-

lated in MCS BASIC-52.  Initially,  NO memory is allocated for  strings.

Memory is allocated by the STRING [expr],  [expr] STATEMENT. The details

of this statement are covered in the DESCRIPTION OF  STATEMENTS  chapter

of this manual.

In MCS BASIC-52, strings can be defined in two ways, with the LET STATE-

MENT and with the INPUT STATEMENT.

EXAMPLE:

>10 STRING 100,20

>20 $(1)="THIS IS A STRING, "

>30 INPUT "WHAT'S YOUR NAME? - "'$(2)

>40 PRINT $(1), $(2)

>RUN

WHAT'S YOUR NAME? - FRED

THIS IS A STRING, FRED

STRINGS can also be assigned to each other with a LET statement.

EXAMPLE:

$(2)=$(1)

Would assign the STRING value in $(1) to the STRING $(2).

90

6.2 THE ASC OPERATOR

In  MCS  BASIC-52,two operators manipulate STRINGS.  These operators are

ASC( ) and CHR( ).  Admittedly,  the string operators contained  in  MCS

BASIC-52  are not quite as powerful as the string operators contained in

some BASICS.  But surprisingly enough,  by using  the  string  operators

available in MCS BASIC-52 it is possible to manipulate strings in almost

anyway available imaginable.  This in itself is a commendable feat since

MCS BASIC-52 was designed primarily to be a sophisticated  BASIC    lan-

guage  oriented  controller,   not  a  string  manipulator.  The  string

operators available in MCS BASIC-52 are as follows:

ASC( )

The ASC( ) operator returns the integer value  of  the  ASCII  character

placed in the parentheses.

EXAMPLE:

>PRINT ASC(A)

65

65  is  the  decimal representation for the ASCII "A." In addition,  in-

dividual characters in a pre-defined ASCII string can be evaluated  with

the ASC( ) operator.

EXAMPLE:

>10 $(1)="THIS IS A STRING"

>20 PRINT $(1)

>30 PRINT ASC($(1),1)

>RUN

THIS IS A STRING

84

When  the  ASC(  )  operator  is  used  in  the manner shown above,  the

$([expr]) denotes what string is being accessed and the expression after

the comma "picks out" an individual character  in  the  string.  In  the

above  example,  the first character in the string was picked out and 84

is the decimal representation for the ASCII character "T."

91

6.2 THE ASC OPERATOR

EXAMPLE:

>10 $(1)="ABCDEFGHIJKL"

>20 FOR X=1 TO 12

>30 PRINT ASC ($(1),X),

>40 NEXT X

>RUN

65 66 67 68 69 70 71 72 73 74 75 76

The numbers printed in the previous example are the values  that  repre-

sent the ASCII characters A,B,C,...L.

Additionally,  the  ASC(  )  operator  can  be used to change individual

characters in a defined string.

EXAMPLE:

>10 $(1)="ABCDEFGHIJKL"

>20 PRINT $(1)

>30 ASC($(1),1)=75

>40 PRINT $(1)

>50 ASC ($(1)<2)=ASC($(1),3)

>60 PRINT $(1)

>RUN

ABCDEFGHIJKL

KBCDEFGHIJKL

KCCDEFGHIJKL

In general,  the ASC( ) operator  lets  the  programmer  manipulate  in-

dividual  characters in a string.  A simple program can determine if two

strings are identical.

EXAMPLE:

>10 $(1)="SECRET" : REM SECRET IS THE PASSWORD

>20 INPUT "WHAT'S THE PASSWORD - ",$(2)

>30 FOR I=1 TO 6

>40 IF ASC($(1),I)=ASC($(2),I) THEN NEXT I ELSE 70

>50 PRINT "YOU GUESSED IT"

>60 END

>70 PRINT "WRONG, TRY AGAIN" : GOTO 20

>RUN

WHAT'S THE PASSWORD - SECURE

WRONG, TRY AGAIN

WHAT'S THE PASSWORD - SECRET

YOU GUESSED IT

92

6.3 THE CHR OPERATOR

CHR( )

The CHR( ) operator is the converse of the ASC( ) operator.  It converts

a numeric expression to an ASCII character.

EXAMPLE:

>PRINT CHR(65)

A

Like  the  ASC( ) operator,  the CHR( ) operator can also "pick out" in-

dividual characters in a defined ASCII string.

EXAMPLE:

>10 $(1)="MCS BASIC-52"

>20 FOR I=1 TO 12 : PRINT CHR($(1),I), : NEXT I

>30 PRINT : FOR I=12 TO 1 STEP -1

>40 PRINT CHR($(1),I), : NEXT I

>RUN

MCS BASIC-52

25-CISAB SCM

In the above example,  the expressions contained within the parentheses,

following  the  CHR operator have the same meaning as the expressions in

the ASC( ) operator.

Unlike the ASC( ) operator,  the CHR( ) operator CANNOT  be  assigned  a

value. A statement such as CHR($(1),1) = H, is INVALID and will generate

a  BAD  SYNTAX  ERROR.  Use  the  ASC( ) operator to change a value in a

string. The CHR( ) operator can only be used within a print statement!

93

CHAPTER 7

Special Operators

7.1 SPECIAL FUNCTION OPERATORS

SPECIAL FUNCTION OPERATORS are called SPECIAL FUNCTION OPERATORS because

they directly manipulate the I/O hardware and the  memory  addresses  on

the 8052AH device. All SPECIAL FUNCTION OPERATORS, with the exception of

CBY([expr])  and  GET,  can  be placed on either side of the replacement

operator (=) in a LET STATEMENT.

EXAMPLES:

A= DBY(100) and DBY 100) = A+2

Both of the above are valid statements in MCS BASIC-52. The SPECIAL FUNC-

TION OPERATORS in MCS BASIC-52 include the following:

CBY([expr])

The CBY([expr]) operator is used to retrieve data from  the  PROGRAM  or

CODE  MEMORY  address  space of the 8052AH.  Since CODE memory cannot be

written into on the 8052AH,  the CBY([expr]) operator cannot be assigned

a value. It can only be read.

EXAMPLE:  A = CBY(1000) Causes the value in code memory space 1000 to be

assigned to the variable A.  The argument for the  CBY([expr])  operator

MUST  be  a  valid integer (i.e.  between 0 and 65535 (0FFFFH)).If it is

not, a BAD ARGUMENT ERROR will occur.

DBY([expr])

The DBY([expr]) operator is used to retrieve or assign a  value  to  the

8052AH's  internal  data memory.  Both the value and argument in the DBY

operator must be between 0 and 255 inclusive.  This is because there are

only  256  internal memory locations in the 8052AH and one byte can only

represent a quantity between 0 and 255 inclusive.

EXAMPLES:

A=DBY(B) and DBY(250) = CBY(1000)

The first example would assign variable A the value that is in  internal

memory location B.  B would have to be between 0 and 255. The second ex-

ample would load internal memory location 250 with the same  value  that

is in program memory location 1000.

7.1 SPECIAL FUNCTION OPERATORS

94

XBY([expr])

The  XBY([expr])  operator  is used to retrieve or assign a value to the

8052AH's external data memory.  The argument in the XBY([expr]) operator

must be a valid integer (i.e.  between 0 and 65535 (0FFFFH)).  The value

assigned to the XBY([expr]) operator must be between 0 and 255. If it is

not a BAD ARGUMENT ERROR will occur.

EXAMPLES:

XBY (4000H)=DBY(100) and A=XBY(0F000H)

The first example would load external memory  location  4000H  with  the

same value that was in internal memory location 100.  The second example

would make the variable A equal to the value in external memory location

0F000H.

GET

The GET operator only produces a meaningful result when used in the  RUN

mode.  It will always return a result of zero in the command mode.  What

GET does is read   the  console  input  device.  Actually,  it  takes  a

"snapshot"  of  the console input device.  If a character is a available

from the console device,  the value of the character will be assigned to

GET. After GET is read in the program, GET will be assigned the value of

zero  until  another is sent from the console device.  The following ex-

ample will print the decimal representation of any character  sent  from

the console:

EXAMPLE:

>10 A=GET

>20 IF <>0 THEN PRINT A

>30 GOTO 10

>RUN

65    (TYPE "A" ON CONSOLE)

49    (TYPE "1" ON CONSOLE)

24    (TYPE "CONTROL-X ON CONSOLE)

50    (TYPE "2" ON CONSOLE)

The  reason the GET operator can be read only once before it is assigned

a value of zero is that this implementation guarantees  that  the  first

character  entered  will  always  be read,  independent on where the GET

operator is placed in the program.

7.1 SPECIAL FUNCTION OPERATORS

95

The following operators directly manipulate the 8052AH's  special  func-

tion registers.  Specific details of the operation of these registers is

in the MICROCONTROLLER USERS HANDBOOK, available from INTEL.

IE

The IE operator is used to retrieve or assign a value  to  the  8052AH's

special  function register IE.  Since the IE register on the 8052AH is a

BYTE register, the value assigned to IE must be between 0 and 255.The IE

register on the 8052AH contains an unused bit,  BIT IE.6> Since the  bit

is "undefined," it may be read as a random one or zero,  so the user may

want to mask this bit when reading the IE register.  This  can  be  done

with as statement like A=IE>AND.0BFH.  The only statements in MCS BASIC-

52 that write to the IE register are the CLOCK0, CLOCK1,  ONEX1,  CLEAR,

and CLEARI statements.

EXAMPLES:

IE = 81H  and   A = IE.AND.0BFH

IP

The  IP  operator  is used to retrieve or assign a value to the 8052AH's

special function register IP.  Since the IP register on the  8052AH is a

BYTE register, the value assigned to IP must be between 0 and 255.The IP

register  on  the  8052AH  contains two unused bits,  BIT IP.6 and IP.7.

since these bits are "undefined," they may be read as a random 1  or  0,

so  the  user  may want to mask these bits when reading the IP register.

This can be done  with a statement such  as  B=IP.AND.3FH  MCS  BASIC-52

does not write to the IP register during initialization, so user can es-

tablish  whatever  interrupt priorities are required in a given applica-

tion.

EXAMPLES:

IP = 3 and A = IP. AND. 3FH

PORT1

The PORT1 operator is used to retrieve or assign a value to the 8052AH's

P1 I/O port.  Since P1 on the 8052AH is a BYTE wide register,  the value

assign  to P1 must be between 0 and 255 inclusive.    Certain bits on P1

have pre-defined functions,  If the user does not implement any  of  the

hardware associated with these pre-defined functions, The PORT1 instruc-

tion can be used in any manner appropriate in the application.

7.1 SPECIAL FUNCTIONS OPERATORS

PCON

96

The  PCON operator is used to retrieve or assign a value to the 8052AH's

PCON register. In the 8052AH,  only the most significant bit of the PCON

register  is used,  all other bits are undefined.  Setting this bit will

double the baud rate if TIMER/COUNTER 1 is used as the baud rate genera-

tor for the serial port. PCON is a byte register.

RCAP2

The RCAP2 is used to retrieve and/or assign a value to the 8052AH's spe-

cial function registers RCAP2H and RCAP2L.  This operator treats  RCAP2H

and RCAP2L as a 16-bit register pair. RCAP2H is the high byte and RCAP2L

is the low byte.  The RCAP2H and RCAP2L registers are the reload/capture

registers for TIMER2.  The user must use caution when writing  to  RCAP2

register  because RCAP2 controls the BAUD rate of the serial port on the

MCS BASIC-52 device.  The following can be used to determine  what  BAUD

rate the MCS BASIC-52 device is operating at:

BAUD = XTAL/(32*(65536-RCAP2))

T2CON

The  T2CON  operator  is  used  to retrieve and/or assign a value to the

8052AH's special function register T2CON.  The T2CON is a byte  register

that  controls  TIMER2'S  mode  of  operation and determines which timer

(TIMER1 or TIMER2) is used as the  8052AH's  baud  rate  generator.  MCS

BASIC-52  initializes T2CON with the value 52 (34H) and assumes that its

value is never changed.  Randomly changing the value of  T2CON,  without

knowing  what  you  are doing can "crash" the serial port on the 8052AH.

Beware!

97

7.1 SPECIAL FUNCTION OPERATORS

TCON

The TCON operator is  used  to  retrieve  and/or  assign  value  to  the

8052AH's special function register TCON. TCON is a byte register that is

used  to  enable or disable TIMER0 and TIMER1,  plus the interrupts that

are associated with these timers. Additionally,  TCON determines whether

the external interrupt pins on the 8052AH are operating in a level sensi-

tive  or  edge-triggered  mode.  MCS  BASIC-52 initializes TCON with the

value 244(0F4H) and assumes that it is  never  changed.  The  value  244

(0F4H)  places  both TIMER0 and TIMER1 in the run(enabled) mode.  If the

user disables the operation of TIMER0,  by clearing BIT 4  in  the  TCON

register,  the  REAL TIME CLOCK will NOT work.  If the user disables the

operation of TIMER1,  by clearing BIT6 in the TCON register,  the  EPROM

programming  routines,  the software serial port,  and the PWM statement

will NOT work. Use caution when changing TCON!!!

TMOD

The TMOD operator is used to retrieve  and/or  assign  a  value  to  the

8052AH's  special  function register TMOD.  TMOD is a byte register that

controls TIMER0 and TIMER1's mode of operation. MCS BASIC-52 initializes

the TCON register with a value 16  (10H).  The  value  16  (10H)  places

TIMER0  in mode 0,  which is a 13-bit counter mode and TIMER1 in in mode

1,  which is a 16-bit counter mode.  MCS BASIC-52 assumes that the modes

of  these two timer/counters are never changed.  If the user changes the

mode of TIMER0, the REAL TIME CLOCK will not operate timely. If the user

changes the mode of TIMER1, EPROM programming, the software serial port,

and the PWM statement will not work properly.  If the user does not  use

these  features  available in MCS BASIC-52,  either timer/counter can be

placed in any mode required by the specific application.

98

7.1 SPECIAL FUNCTION OPERATORS

TIME

The TIME operator is used to retrieve and/or assign a value to the  REAL

TIME  CLOCK.  When  the REAL TIME CLOCK resident in MCS BASIC-52.  After

reset,  TIME is equal to 0.  The CLOCK1 statement enables the REAL  TIME

CLOCK.  When  the  REAL  TIME  CLOCK  is  enabled,the  SPECIAL  FUNCTION

OPERATOR,  TIME will increment  once  every  5  milliseconds.  The  TIME

operator  uses  TIMER0  and the interrupts associated with TIMER0 on the

8052AH.  The unit of TIME is seconds and the appropriate XTAL value must

be assigned to insure that the TIME operator is accurate.

When  TIME  is  assigned a value with a LET statement (i.e.  TIME = 100,

only the integer portion of TIME will be changed.

EXAMPLE:

>CLOCK1        (enable REAL TIME CLOCK)

>CLOCK0        (disable REAL TIME CLOCK)

>PRINT TIME    (display TIME)

3.315

>TIME = 0      (set TIME = 0)

>PRINT TIME    (display TIME)

.315          (only the integer is changed)

The "fraction" portion of TIME can be changed by

manipulating the contents of internal memory

location 71 (47H). This accomplished by a DBY(71)

statement. Note that each count in internal memory

location 71 (47H) represents 5 milliseconds of TIME.

Continuing with the EXAMPLE.

>DBY (71) = 0  (fraction of TIME = 0)

>PRINT TIME

0

>DBY(71) = 3   (fraction of TIME = 3, 15 ms)

>PRINT TIME

1.5 E-2

7.1 SPECIAL FUNCTION OPERATORS

99

The reason only the integer portion of TIME is changed when  assigned  a

value  is  that  it allows the user to generate accurate time intervals.

For instance,  let's say you want to create an accurate 12  hour  clock.

There  43200  seconds  in a 12 hour period,  so an ONTIME 43200,[ln num]

statement is used.  Now,  when the TIME interrupt occurs  the  statement

TIME  = 0 is executed,  but the millisecond counter is not re-assigned a

value so if interrupt latency happens  to  exceed  5  milliseconds,  the

clock will still remain accurate.

TIMER0

The  TIMER0  operator  is  used  to  retrieve  or  assign a value to the

8052AH's special function registers TH0 and TL0.  This  operator  treats

the  byte  registers  TH0 and TL0 as a 16-bit register pair.  TH0 is the

high byte and TL0 is the low byte.  MCS BASIC-52 uses  TH0  and  TL0  to

implement  the REAL TIME CLOCK function.  If the user does not implement

the REAL TIME CLOCK function (i.e. does not use the statement CLOCK1) in

the BASIC program TH0 and TL0 may be used in any manner suitable to  the

particular application.

TIMER1

The  TIMER1  operator  is  used  to  retrieve  or  assign a value to the

8052AH's special function registers TH1 and TL1.  This  operator  treats

the  byte  registers  TH1 and TL1 as a 16-bit register pair.  TH1 is the

high byte and TL1 is the low byte.  MCS BASIC-52 uses  TH1  and  TL1  to

implement  the timings for the software serial port,  the EPROM program-

ming feature,  and the PWM statement.  If the user does not use  any  of

these features TH1 and TL1 may be used in any manner suitable to the par-

ticular application.

TIMER2

The  TIMER2  operator  is  used  to  retrieve  or  assign a value to the

8052AH's special function registers TH2 and TL2.  This  operator  treats

the  byte  registers  TH2 and TL2 as a 16-bit register pair.  TH2 is the

high byte and TL2 is the low byte. MCS BASIC-52 uses TH2 and TL2 to gen-

erate the baud rate for the serial port. If the user does not use TIMER2

to clock the serial port, TH2 and TL2 may be used in any manner suitable

to the particular application.

100

7.1 SPECIAL FUNCTION OPERATORS

XTAL

The XTAL operator tells MCS BASIC-52 what frequency the system is operat-

ing at.  The XTAL operator is used by MS BASIC-52 to calculate the  REAL

TIME CLOCK reload value,  the PROM programming timing,  and the software

serial port baud rate generation. The XTAL value is expressed in Hz. So,

XTAL = 9000000

would set the XTAL value to 9 MHz.

101

7.2 EXAMPLES OF MANIPULATING SPECIAL FUNCTION VALUES

Using the logical operators available in MCS BASIC-52, it is possible to

write or read from any byte of the special function registers  that  MCS

BASIC-52 treats as a register pair:

EXAMPLE:

WRITING TO THE HIGH BYTE

>TIMER0 = (TIMER0 .AND. 00FFH) + INT (256*(USER BYTE))

EXAMPLE:

WRITING TO THE LOW BYTE

>TIMER0 = (TIMER0 .AND. 0FF00H) + (USER BYTE)

EXAMPLE:

READING HIGH BYTE

>PHO. INT(TIMER0/256)

EXAMPLE:

READING LOW BYTE

>PHO. TIMER0 .AND. 0FFH

TIMER1 can function as the baud rate generator for MCS BASIC-52.  To as-

sign TIMER1 as the baud rate generator,  the following instructions must

be executed:

>TMOD = 32     -      TIMER1 in auto reload mode

>TIMER1 = 256*(256-(65536-RCAP2)/12) - load TIMER1

>T2CON = 0     -      use TIMER1 as baud rate gen

This  sequence of instructions can be executed in either the direct mode

or as part of a program. When TIMER1 is used as the baud rate generator,

TIMER2 can be used in anyway suitable  to  the  application.  The  PROG,

FPROG, LIST# and PWM commands/statements cannot be used when TIMER1 func-

tions  as  the baud rate generator for the MCS BASIC-52 device.  Certain

crystals may not be able to use TIMER1 as the baud rate generator, espe-

cially at high (above 2400) baud rates.

7.3 SYSTEM CONTROL VALUES

102

The SYSTEM CONTROL VALUES determine or reveal how memory is allocated by

MCS BASIC-52.

MTOP

After reset,  MCS BASIC-52 sizes the external memory and assign the last

valid  memory  address to the SYSTEM CONTROL VALUE,  MTOP,  MCS BASIC-52

will not use any external RAM memory beyond the value assign to MTOP. If

the user wishes to allocate some external memory for  an  assembly  lan-

guage routine the LET statement can be used (e.g.  MTOP = USER ADDRESS).

If the user assigns a value to MTOP that is greater than the last  valid

memory address, a MEMORY ALLOCATION ERROR will be generated.

EXAMPLE:

>PRINT MTOP

2047

>MTOP=2000

>PRINT MTOP

2000

LEN

The SYSTEM CONTROL VALUE,  LEN,  tells the user how many bytes of memory

the current selected program occupies. Obviously, LEN cannot be assigned

a value,  it can only be read.  A NULL program (i.e.  no  program)  will

return a LEN of 1. The 1 represents the end of program file character.

FREE

The  SYSTEM  CONTROL VALUE,  FREE,  tells the user how many bytes of RAM

memory are available to the user.  When the current selected is  in  RAM

memory, the following relationship will always hold true.

FREE = MTOP - LEN - 511

NOTE: Unlike some BASICS, MCS BASIC-52 does not require any "dummy" argu-

ments for the SYSTEM CONTROL VALUES.

CHAPTER 8

Error Messages, Bells, Whistles, and Anomalies

8.1 ERROR MESSAGES

103

MCS BASIC-52 has a relatively sophisticated ERROR processor.  When BASIC

is in the RUN mode the generalized form of the ERROR message is as  fol-

lows:

ERROR:  XXX - IN LINE YYY

YYY BASIC STATEMENT

-----------X

Where XXX is the ERROR TYPE and YYY is the line number of the program in

which the error occurred. A specific example is:

ERROR:  BAD SYNTAX - IN LINE 10

10 PRINT 34*21*

--------------X

The  X signifies approximately where the ERROR occurred in the line num-

ber.  The specific location of the X may be off by one or two characters

or  expressions  depending  on the type of error and where the error oc-

curred in the program.  If an ERROR occurs in the COMMAND MODE only  the

ERROR  TYPE  will be printed out NOT the Line number.  This makes sense,

because there are no line numbers in the COMMAND MODE.  The ERROR  TYPES

are as follows:

BAD SYNTAX

A  BAD  SYNTAX  error means that either an invalid MCS BASIC-52 COMMAND,

STATEMENT,  or OPERATOR was entered and BASIC cannot process the  entry.

The  user  should  check and make sure that everything was typed in cor-

rectly. In Version 1.1 of MCS BASIC-52 a BAD SYNTAX ERROR is also genera-

ted if the programmer attempts to use a reserved key word as a part of a

variable.

BAD ARGUMENT

When the argument of an  operator  is  not  within  the  limits  of  the

operator a BAD ARGUMENT ERROR will be generated.  For instance, DBY(257)

would generate a BAD ARGUMENT ERROR because the  argument  for  the  DBY

operator  is limited to the range 0 to 255.  Similarly,  XBY(5000H) = -1

would generate a BAD  ARGUMENT  ERROR  because  the  value  of  the  XBY

operator is limited to the range 0 to 255.

8.1 ERROR MESSAGES

ARITH. UNDERFLOW

104

If  the  result of an arithmetic operation exceeds the lower limit of an

MCS BASIC-52 floating point number,  an ARITH> UNDERFLOW ERROR will  oc-

cur. The smallest floating point number in MCS BASIC-52 IS + 1E-127. For

instance, 1E-80/1E+80 would cause an ARITH. UNDERFLOW ERROR.

ARITH. OVERFLOW

If  the  result of an arithmetic operation exceeds the upper limit of an

MCS BASIC-52 floating point number, an ARITH. OVERFLOW ERROR will occur.

The largest floating point number in MCS BASIC-52 IS +99999999E+127. For

instance, 1E+70*1E+70 would cause an ARITH.OVERFLOW ERROR.

DIVIDE BY ZERO

A division by ZERO was attempted i.e.  12/0 will cause a DIVIDE BY  ZERO

ERROR.

ILLEGAL DIRECT (VERSION 1.0 ONLY)

Some  statements,  such as IF-THEN and DATA cannot be executed while the

MCS BASIC-52 device is in the COMMAND MODE.  If you attempt  to  execute

one of these statements the ERROR: ILLEGAL DIRECT will be printed to the

console  device.  The ILLEGAL DIRECT ERROR is not trapped in Version 1.1

of MCS BASIC-52. ILLEGAL DIRECT ERRORS return a BAD SYNTAX ERROR in Ver-

sion 1.1.

LINE TOO LONG (VERSION 1.0 ONLY)

If you type in a line that contains more than 73 characters the  message

ERROR: LINE TOO LONG will be printed the console device.  MCS BASIC-52's

input buffer can only handle up to 73 characters.

NOTE

This error does not exist in Version 1.1.  Instead the input buffer  has

been increased to 79 characters and MCS BASIC-52 will echo a bell charac-

ter to the user terminal if too many characters are entered into the in-

put buffer.

NO DATA

If  a READ STATEMENT is executed and no DATA STATEMENT exist or all DATA

has been read and a RESTORE instruction was not executed the message ER-

ROR: NO DATA-IN LINE XXX will be printed to the console device.

8.1 ERROR MESSAGES

CAN'T CONTINUE

Program execution can be halted by either typing in a control-C  to  the

console device or by executing a STOP STATEMENT. Normally, program execu-

tion can be resumed by typing in the CONT command.  However, if the user

105

edits the program after halting execution and then enters the CONT  com-

mand, a CAN'T CONTINUE ERROR will be generated. A control-C must be type

during    program  execution or a STOP STATEMENT must be executed before

the CONT command will work.

PROGRAMMING

If an error occurs while the  MCS  BASIC-52  device  is  programming  an

EPROM,  a  PROGRAMMING  ERROR  will  be generated.  An error encountered

during programming destroys the EPROM FILE STRUCTURE, so the user cannot

save any more programs on that particular EPROM once a PROGRAMMING ERROR

occurs.

A-STACK

An A-STACK (ARGUMENT STACK) error occurs when the argument stack pointer

is forced "out of the bounds." This can happen if the user overflows the

argument by PUSHing too many expressions onto the stack,  or by attempt-

ing to POP data off the stack when no data is present.

C-STACK

A  C-STACK (CONTROL STACK) error will occur if the control stack pointer

is forced "out of bounds." 158 bytes of external  memory  are  allocated

for  the  control  stack,  FOR  - NEXT loops require 17 bytes of control

stack DO - UNTIL,  DO - WHILE,  and GOSUB require  3  bytes  of  control

stack. This means that 9 nested FOR - NEXT loops is the maximum that MCS

BASIC-52 can handle because 9 times 17 equals 153.  If the user attempts

to use more control stack than is available in MCS  BASIC-52  a  C-STACK

error  will  be generated.  In addition,  C-STACK errors will occur if a

RETURN is executed before a GOSUB,  a WHILE or UNTIL before a DO,  or  a

NEXT before a FOR.

106

8.1 ERROR MESSAGES

I-STACK

An I-STACK (INTERNAL STACK) error occurs when MCS BASIC-52 does not have

enough stack space to evaluate an expression.  Normally,  I-STACK errors

will not occur unless insufficient memory  has  been  allocated  to  the

8052AH's  stack pointer.  Details of how to allocate memory to the stack

pointer are covered in the ASSEMBLY LANGUAGE  LINKAGE  section  of  this

manual.

ARRAY SIZE

If  an  array  is dimensioned by a DIM statement and then you attempt to

access a variable that is outside of the dimensioned  bounds,  an  ARRAY

SIZE error will be generated.

EXAMPLE:

>DIM A(10)

>PRINT A(11)

ERROR:  ARRAY SIZE

READY

MEMORY ALLOCATION

MEMORY  ALLOCATION  ERRORS  are  generated  when user attempts to access

STRINGS that are "outside" the defined string limits.  Additionally,  if

the SYSTEM CONTROL VALUE, MTOP is assigned a value that does not contain

any RAM memory, a MEMORY ALLOCATION ERROR will occur.

107

8.2 DISABLING CONTROL-C

In  some  applications,  it  may be desirable or even a requirement that

program execution not accidentally be halted.  Under "normal"  operation

the  execution of any MCS BASIC-52 program can be terminated by typing a

"control-C on the console device. However, it is possible to disable the

"control-C" break function in MCS BASIC-52. This is accomplished  by set-

ting BIT 48 (30H) to a one.  BIT 48 is located in internal memory  loca-

tion 38.0 (26.0H). This BIT may be set by executing the following state-

ment in an MCS BASIC-52 program:

DBY(38) = DBY(38).OR.01H

Once this BIT is set to one, the control-C break function, for both LIST

and RUN operations will be disabled. The user has the option to create s

custom  break  character  or  string  of  characters  by  using  the GET

operator. The following is an example of how to implement a custom break

character:

EXAMPLE:

>10 STRING 100,10: A=1: REM INITIALIZE STRINGS

>20 $(1) = "BREAK" : REM "BREAK" IS THE PASSWORD

>30 DBY (38)=DBY(38).OR. 1 : REM DISABLE CONTROL-C

>40 FOR I=1 TO 1000 : REM DUMMY LOOP

>50 J=SIN(I)

>60 K=GET : IF K<>0 THEN 100 ELSE NEXT I

>70 END

>100 IF K=ASC($(1),A) THEN A=A+1 ELSE A=1

>110 REM TEST FOR MATCH

>120 IF A=1 THEN NEXT .I

>130 IF A=6 THEN 200 ELSE NEXT I

>140 END

>200 PRINT "BREAK"

>210 DBY (38)=DBY(38).AND.0FEH : REM ENABLE CONTROL-C

In this example,  typing the word BREAK will stop program execution.  In

other words, BREAK is a password.

108

8.3 IMPLEMENTING "FAKE DMA"

The MCS BASIC-52 device does not contain any hardware mechanism that sup-

ports Direct Memory Access (DMA). However, the DMA function is supported

in  software  by  MCS  BASIC-52.   During  DMA  operation  MCS  BASIC-52

guarantees that no external memory access will be performed.  To  enable

the DMA function, the following must be performed:

1)  BIT  49,  which  is located in internal memory location 38.1 (26.1H)

must be set to a one. This can be accomplished in      BASIC  by   using

the statement - DBY(38) = DBY(38).OR.02H

2)BIT 0 and BIT 7 of the SPECIAL FUNCTION REGISTER, IE(Interrupt enable)

must  be  set  to a one.  This can be accomplished in BASIC by using the

statement - IE = IE.OR.81H

After the three BITS mentioned above are set to a one,  external  inter-

rupt zero (INTO) acts as a DMA input pin.  INTO is pin 12 on the 8052AH.

Whenever INTO is pulled low (to a logical zero state),  the MCS BASIC-52

device  will enter the DMA mode and no accesses will be made to external

memory.  To acknowledge that MCS BASIC-52 has entered the DMA mode,  MCS

BASIC-52 outputs a zero on pin 7 (P1.6). In essence, PORT 1.6 is the DMA

ACK pin of the MCS BASIC-52 device.  In most applications, this would be

used to disable three-state buffers  that  would  be  placed  on  PORT2,

PORT0,  and the address latch of the MCS BASIC-52 system. After the user

pulls the INTO pin high,  MCS BASIC-52 will output a one on P1.6 program

does  nothing except wait for the INTO pin to pulled high.  So,  program

execution is halted.

It should be noted that although this "fake DMA" operation does  provide

the same functionality as a normal DMA hardware mechanism, it also takes

substantially  longer for the normal DMA REQUEST - DMA ACKNOWLEDGE cycle

to be performed. That is because MCS BASIC-52 uses interrupts  to imple-

ment the DMA operation,  instead of dedicated  hardware.  As  a  general

rule,  cycle  stealing  DMA  is not an option with MCS BASIC-52's "fake"

DMA.  Only "burst mode" DMA cycles can be  implemented  without  a  sig-

nificant  time  penalty.  When "fake DMA" is implemented,  the user must

provide three-state buffers on the PORT2 on the PORT2,  PORT0,  and  the

address latch of the MCS BASIC-52 system.

109

8.4 RUN TRAP OPTION (Version 1.1 Only)

Version  1.1 of MCS BASIC-52 permits the user to trap the interpreter in

the RUN MODE.  This option is evoked by putting a 34H (52D) in  external

data memory location 5EH (94D).  After a 34H (52D) is placed in external

data memory location 5EH (94D) the  MCS  BASIC-52  interpreter  will  be

trapped  in  the RUN mode forever or until the contents of external data

memory location is changed to something other  than  34H  (52D).  If  no

program is present when a 34H (52D) is placed in location 5EH (94D), MCS

BASIC-52  will  print  the  READY message forever and it will be time to

RESET the device.  The RUN TRAP option can be employed  with  the  other

RESET  options  to  permit  the  user to execute a program from RAM on a

RESET or power-up condition when some type  of  battery  back-up  memory

scheme is employed.

110

8.5 ANOMALIES

Most  dictionaries define anomaly as a deviation from the normal or com-

mon order or as an irregularity.  Anomalies to an extreme become  "BUGS"

or  something  that  is wrong with the program.  Like all programs,  MCS

BASIC-52 contains some anomalies, hopefully, no bugs. The purpose of men-

tioning the known anomalies here is that it may save the programmer some

time,  should strange things happen during program execution.  The known

anomalies deal mainly with the way MS BASIC-52 compacts or tokenizes the

BASIC program. The known anomalies and cautions are as follows:

1)When  using  the  variable H after a line number,  make sure you put a

space between the line number and the H,  or else BASIC will assume that

the line number is a HEX number.

EXAMPLES:

>20H=10   (WRONG)          >20 H=10   (RIGHT)

>LIST                      >LIST

32   =10                   20    H=10

2)    When using the variable I before an ELSE statement,  make sure you

put a space between the I and the ELSE statement, or else BASIC will as-

sume that the IE portion of IELSE is the special function operator IE.

EXAMPLES:

>20 IF I>10 THEN PRINT IELSE 100

>LIST

20    IF I>10 THEN PRINT IELSE 100  (WRONG)

>20 IF I>10 THEN PRINT I ELSE 100

>LIST

20    IF I>10 THEN PRINT I ELSE 100 (RIGHT)

3) A space character may not be placed inside the ASC(  )  operator.  In

other  words,  a statement like PRINT ASC( ) will yield a BAD SYNTAX ER-

ROR.  Spaces may be placed in strings however,  so a statement like  LET

$(1) = "HELLO, HOW ARE YOU" will work properly. The reason ASC( ) yields

an  error  is  because MCS BASIC-52 eliminates all spaces when a line is

processed,  so ASC( ) will be stored as ASC( ) and MCS  BASIC-52  inter-

prets this as an error.

CHAPTER 9

Assembly Language Linkage

111

9.1 OVERVIEW

NOTE:  This  assumes  that  the designer has an understanding of the ar-

chitecture and assembly language of the MCS-51 Microcontroller family!!!

MCS BASIC-52 contains a complete library of routines that can easily  be

accessed  with  assembly  language  CALL instructions.  The advantage of

using assembly language is that it offers a significant  improvement  in

execution speed relative to interpreted BASIC,  In order to successfully

interface MCS BASIC-52 with an assembly language program,  the  software

designer must be aware of a few simple facts.

READ THIS CAREFULLY!!!

1.  MCS BASIC-52 uses REGISTER BANKS 0,  1,  and 2 (RB0,  RB1, and RB2).

REGISTER BANK 3 (RB3) is never used except during a PGM  statement.  RB3

is  designated the USER REGISTER BANK and the users can do whatever they

want to with REGISTER BANK 3 (RB3) and MCS BASIC-52 will never alter the

contents of this REGISTER BANK except during the execution of PGM state-

ment.  The contents of REGISTER BANK (RB3) can be changed by executing a

DBY ([expr])=[0 to 255] statement.  Where the [expr] evaluates to a num-

ber between 24 (18H) and  31  (1FH)  inclusive.  In  addition,  INTERNAL

MEMORY LOCATIONS 32 (20H) and 33 (21H) are also NEVER used by MCS BASIC-

52.These  two  BIT  and/or  BYTE  addressable locations are specifically

reserved for assembly language programs.

2. MCS BASIC-52 uses REGISTER BANK 0 (RB0) as the WORKING REGISTER FILE,

Whenever assembly language is used to access  MCS  BASIC-52's  routines,

the WORKING REGISTER FILE, REGISTER BANK 0 (RB0) MUST BE SELECTED!!!This

means  that  the  USER  MUST  MAKE  SURE  THAT  REGISTER BANK 0 (RB0) IS

SELECTED BEFORE CALLING ANY ANY OF MCS BASIC-52's ROUTINES. This is done

simply by setting       BITS 3 and 4 in the PSW equal to ZERO.  If  this

is not done, MCS BASIC-52 will "KICK OUT" the USER and NO operation will

be performed. When an ASSEMBLY LANGUAGE program is accessed by using the

MCS  BASIC-52's  CALL instruction,  REGISTER BANK 0 (RB0) will always be

selected.  So unless the user selects REGISTER BANK 3 (RB3) in  assembly

language, it is NOT NECESSARY to change the designated REGISTER BANK.

3.  ALWAYS ASSUME THAT MCS BASIC-52 DESTROYS THE CONTENTS OF THE WORKING

REGISTER FILE AND THE DPTR,  UNLESS OTHERWISE STATED  IN  THE  FOLLOWING

DOCUMENTATION.

4.  Certain  routines in MCS BASIC-52 REQUIRE that REGISTERS be initial-

ized BEFORE the user CALLS the specific ROUTINE. These registers are AL-

WAYS in the WORKING REGISTER FILE, REGISTER BANK 0 (RB0).

5. Certain routines in MCS BASIC-52 return the result of an operation in

a register or registers.  The result registers are ALWAYS in the WORKING

REGISTER FILE, REGISTER BANK 0, (RB0).

112

9.1 OVERVIEW

READ THIS CAREFULLY!!!

6.  MCS  BASIC-52  loads  the  INTERNAL  STACK POINTER (SPECIAL FUNCTION

REGISTER-SP with the value that is in INTERNAL MEMORY LOCATION 62 (3EH).

MCS BASIC-52 initializes INTERNAL MEMORY LOCATION 62 (3EH) by writing  a

77 (4DH) to this location after a hardware RESET.  MCS BASIC-52 does NOT

use any memory beyond 77 (4DH) for anything EXCEPT STACK SPACE.  If  the

user wants to ALLOCATE some additional internal memory for their applica-

tion,  this is done by changing the contents of INTERNAL MEMORY LOCATION

62 (3EH) to a value that is GREATER than 77 (4DH).  This  will  allocate

the  INTERNAL MEMORY LOCATIONS from 77 (4DH) to the value that is placed

in INTERNAL MEMORY LOCATION 62 (3EH) to the user. As a guideline,  it is

a  good  idea  to  allow  at least 48 (30H) bytes of stack space for MCS

BASIC-52.  The bad news about reducing the stack space is that  it  will

reduce  the  amount of nested parentheses that MCS BASIC-52 can evaluate

in an expression [expr].  This will either cause a I-STACK ERROR or will

cause  a  fatal  CPU "crash." Use caution and DON'T allocate more memory

than you need.

EXAMPLE OF THE EFFECTS OF ALTERING THE STACK LOCATION:

COMMENTS

>PRINT DBY(62)        AFTER RESET INTERNAL MEMORY LOCATION 62

77                   CONTAINS A 77

>PRINT (1*(2*(3)))    BASIC HAS NO PROBLEM EVALUATING 3.LEVELS

6                    OF NESTED PARENTHESES

>DBY(62)=230          NOW ALLOCATE 255-230 = 25 BYTES OF STACK

SPACE TO BASIC, REMEMBER, THE STACK ON

>PRINT  (1*(2*(3)))   THE 8052AH GROWS "UP"

ERROR:  I-STACK       BASIC CANNOT EVALUATE THIS EXPRESSION

READY                 BECAUSE IT DOES NOT HAVE ENOUGH STACK

>DBY(62)=210          NOW ALLOCATE 255-210 = 45 BYTES OF STACK

SPACE TO BASIC

>PRINT (1*(2*(3)))    THE I-STACK ERROR GOES AWAY

6

7.  Throughout this section a 16-BIT REGISTER PAIR is  designated-Rx:Ry,

where  Rx  is  the most significant byte and Ry is the least significant

byte.

EXAMPLE:

113

R3: R1 - R3=MOST SIGNIFICANT BYTE, R1=LEAST SIGNIFICANT BYTE

114

9.2 GENERAL PURPOSE ROUTINES

Accessing MCS BASIC-52 routines with assembly language is easy. The user

just loads the ACCUMULATOR with a specific value and CALLS  LOCATION  48

(30H).  The  value  placed  in the ACCUMULATOR determines what operation

will be performed. Unless otherwise stated, the CONTENTS of the DPTR and

REGISTER BANK  0  (RB0)  will  ALWAYS  be  altered  when  calling  these

routines.  The generalized form for accessing NCS BASIC-52's routines is

as follows:

ANL    PSW,#11100111B    ; make sure

RBO is

selected

MOV    A,#OPBYTE         ; load the

instruction

CALL   30H               ; execute the

instruction

The value of OPBYTE determines what operation will be performed. The fol-

lowing operation can be performed:

OPBYTE = 0 (00H) RETURN TO COMMAND MODE

This instruction causes MCS BASIC-52 to enter the COMMAND MODE.  Control

of the CPU is handed back to the MCS BASIC-52 interpreter and BASIC will

respond by outputting a READY and a PROMPT character (>).

OPBYTE = 1 (01H) POP ARGUMENT STACK AND PUT VALUE IN R3:R1

This  instruction  converts the value that is on the ARGUMENT STACK to a

16 BIT BINARY INTEGER and returns the BINARY  INTEGER  in  registers  R3

(high  byte)  and  R1 (low byte) of REGISTER BANK 0 (RB0).  The ARGUMENT

STACK gets popped after this instruction is executed.  If the  value  on

the ARGUMENT STACK cannot be represented by a 16-BIT BINARY NUMBER (i.e.

it  is NOT between 0 and 65535 (0FFFFH) inclusive),  BASIC WILL TRAP THE

ERROR and print a BAD ARGUMENT ERROR MESSAGE.  The BINARY VALUE returned

is TRUNCATED, NOT ROUNDED.

115

EXAMPLE:

BASIC PROGRAM -            10 PUSH 260

20 CALL 5000H

ASSEMBLY LANGUAGE PROGRAM -ORG 5000H

MOV  A,#01H   ; load opbyte

CALL 30H      ; RBO still selected

;

; at this point R3 (of RB0) = 01H

; and R1 (of RB0) = 04H

;   so,   R3:   R1  =260,   which  was  the

value

;   that  was  placed   on   the   ARGUMENT

STACK

116

9.2 GENERAL PURPOSE ROUTINES

COMMENTS ON THE NEXT TWO INSTRUCTIONS:

The next two instructions permit the user to transfer floating point num-

bers  between  an  assembly language program and MCS BASIC-52.  The user

provides the address where a floating point number is stored or will  be

stored in a 16-bit REGISTER PAIR.  depending on the operation requested,

the floating point numbers are either PUSHED ON or POPPED OFF MCS BASIC-

52's ARGUMENT STACK.  This instruction permits the user to keep track of

variables  in assembly language and bypass the relatively slow procedure

BASIC must follow.

As mentioned earlier,  when a floating point number is PUSHED  onto  the

ARGUMENT STACK,  the ARGUMENT STACK POINTER is decremented by a count of

6.  This is because a floating point number  requires  6  bytes  of  RAM

storage.  Although  it may seem confusing to the novice,  the LAST value

placed on the ARGUMENT STACK is referred to as the value on the  TOP  of

the  ARGUMENT STACK,  even though it is on the BOTTOM of the STACK rela-

tive to the sequential numbering of memory addresses.  No one knows  why

this is so.

The  ARGUMENT  STACK resides in EXTERNAL RAM MEMORY LOCATIONS 301 (12DH)

through 510 (1FEH).   The lower  BYTE  of  the  ARGUMENT  STACK  POINTER

resides  in  INTERNAL MEMORY LOCATION 9 (09H).   MCS BASIC-52 always as-

sumes that the upper BYTE (higher order address) of the  ARGUMENT  STACK

POINTER  is  1  (01H).   The software designer can use this information,

along with the next two instructions to perform operations like  copying

the back.

OPBYTE  =  2  (02H) PUSH THE FLOATING POINT NUMBER ADDRESSED BY REGISTER

PAIR R2:R0 ONTO THE ARGUMENT STACK.

R2 and R0 (in REGISTER BANK 0, RB0) contain the ADDRESS (R2 = high byte,

R0 = low byte) of the  location  where  the  floating  point  number  is

stored.    After  this instruction is executed the floating point number

that the REGISTER PAIR R2:R0 points to is PUSHED onto the TOP of the AR-

GUMENT STACK.   The ARGUMENT STACK  POINTER  automatically  gets  DECRE-

MENTED, by a count of 6, when the value is placed on the stack.  A float-

ing  point number in MCS BASIC-52 requires 6 BYTES of RAM storage.   The

register Pair R2:R0 points to the MOST SIGNIFICANT BYTE of the  floating

point  number  and is DECREMENTED BY 6 after the CALL instruction is ex-

ecuted.  So,  if R2:R0 = 7F18H before this instruction was executed,  it

would equal 7F12H after this instruction was executed.

9.2 GENERAL PURPOSE ROUTINES

117

OPBYTE = 3 (03H) POP THE ARGUMENT STACK AND SAVE THE FLOATING POINT NUM-

BER IN THE LOCATION ADDRESSED BY R3:R1.

The TOP of the ARGUMENT STACK is moved to the location pointed to by the

REGISTER PAIR R3:R1(R3 = high byte,  R1 = low byte,  in REGISTER BANK 0,

RB0).   The ARGUMENT STACK POINTER is automatically  INCREMENTED  BY  6.

Just as in the previous PUSH instruction, the REGISTER PAIR R3:R1 points

to  the MOST SIGNIFICANT BYTE of the floating point number and is DECRE-

MENTED BY 6 after the CALL instruction is executed.

EXAMPLE OF USER PUSH AND POP:

BASIC PROGRAM:  >5 REM PUT 100 AND 200 ON THE ARGUMENT STACK

>10 PUSH 100, 200

>15 REM CALL THE USER ROUTINE TO SAVE NUMBERS

>20 CALL 5000H

>25 REM CLEAR THE STACK

>30 CLEARS

>35 REM USE ASM TO PUT NUMBERS BACK ON STACK

>40 CALL 5010H

>50 POP A,B

>60 PRINT A,B

>RUN

100 200

READY

118

ASM PROGRAM:      ORG  5000H

MOV  R3, #HIGH USER _ SAVE  ; LOAD POINTERS TO

WHERE

MOV  R1, #LOW USER_SAVE     ; NUMBERS WILL BE

SAVED

MOV  A, #03H                ; LOAD OPBYTE

CALL 30H                    ; SAVE ONE NUMBER

MOV  A, #03H                ; LOAD OPBYTE AGAIN

CALL 30H                    ; SAVE ANOTHER

NUMBER

RET                         ; BACK TO BASIC

;

ORG  5010H                  ;

MOV  R2, #HIGH USER_SAVE    ;LOAD ADDRESS WHERE

MOV  R0, #LOW USER_SAVE     ;NUMBERS ARE STORED

MOV  A, #02H                ;LOAD OPBYTE

CALL 30H                    ;PUT ONE NUMBER ON

STACK

MOV  A, #02H                ;LOAD OPBYTE

CALL 30H                    ;NEXT NUMBER ON

STACK

RET                         ;BACK TO BASIC

119

9.2 GENERAL PURPOSE ROUTINES

OPBYTE = 4(04H) PROGRAM  A  PROM  USING  R3:R1  AS  THE  SOURCE  ADDRESS

POINTER,  R2:R0 AS THE DESTINATION (PROM) ADDRESS POINTER,  AND R7:R6 AS

THE BYTE COUNTER.

This instruction assumes that the DATA to be programmed into a  PROM  is

stored in external memory and that the REGISTER PAIR R3:R1 (in RB0) con-

tains the address of this external memory,  REGISTER PAIR R7:R6 contains

the total number of bytes  that  are  to  be  programmed.  The  PROM  is

programmed  sequentially  and  everytime is programmed the REGISTER PAIR

R7:R6 is decremented and the REGISTER PAIRS R3:R1 and R2:R0  are  incre-

mented. Programming continues until R7:R6 equals ZERO. The REGISTER PAIR

R2:R0  must  be initialized with the desired ADDRESS of the EPROM  to be

programmed MINUS 1.  This may sound strange,  but that  is  the  way  it

works.  So, if you wanted to program an EPROM starting at address 9000H,

with the data stored in address 0D00H and  you  wanted  to  program  500

BYTES,  then  the  registers  would be set up as follows: R2:R0 = 8FFFH,

R3:R1 = 0D00H,  and R7:R6 = 01F4H (500 decimal).  You would then put a 4

(04H) in the ACC and CALL location 30H.

NOTE: In Version 1.0,  if an ERROR OCCURS DURING PROGRAMMING, MCS BASIC-

52 WILL TRAP THE ERROR AND ENTER  THE  COMMAND  MODE.  The  user  cannot

handle errors that occur during the EPROM programming operation!!!!

In  Version  1.1,  programming  errors  will  only be trapped if the MCS

BASIC-52 device is in the COMMAND MODE. If the MCS BASIC-52 device is in

the run mode,  control will be passed back to the user.  The  user  must

then examine registers R6 and R7.  If R6 = R7 = 0,  then the programming

operation was successfully completed,  if these registers do  not  equal

zero then registers R2:R0 contain the address of the EPROM location that

failed  to  program.  This  feature  in  Version 1.1 permits the user to

program EPROMS in embedded applications and manage errors,  should  they

occur in the programming process, without trapping to the command mode.

In  addition  to setting up the pointers previously described,  the user

must also initialize the INTERNAL  MEMORY  locations  that  control  the

width  of  the  programming pulse.  This gives the user complete control

over this critical prom programming parameter. The internal memory loca-

tions that must be initialized with this information are 64 (40H) and 65

(41H).  These locations are treated as a 16 bit register pair with loca-

tion  64  (40H)  designated as the most significant byte and location 65

(41H) as the least significant byte. Locations 64 (40H) and 65 (41H) are

loaded into TH1 (TIMER 1 high byte) and TL1 (TIMER 1 low  byte)  respec-

tively.  The  width of the programming pulse,  in microseconds is deter-

mined by the following equation:

WIDTH = (65536-256*DBY(64)+DBY(65)*12/XTAL microseconds

conversely;

DBY(64):DBY(65) = 65536- WIDTH*XTAL/12

120

The proper values for the "normal" 50 millisecond programming  algorithm

and  the 1 millisecond "INTELligent" algorithm are calculated and stored

by MCS BASIC-52 in external memory  locations  296:297  (128H:129H)  and

298:299  (12AH:12BH)  respectively.  If  the  user wants to use the pre-

calculated values the statements DBY(64)=XBY(296)  and  DBY(65)=XBY(297)

may be used to initialize the prom programming width for the normal algo-

rithm  and  the  statements DBY(64)=xby(298) and DBY(65)=XBY(299) can be

used to initialize for the INTELligent algorithm.

121

9.2 GENERAL PURPOSE ROUTINES

The select the "INTELLIGENT" EPROM PROGRAMMING  algorithm  the  directly

addressable   BIT 51 (33H) MUST be set to 1 before the EPROM PROGRAMMING

routine is called.  The "STANDARD" 50 ms EPROM PROGRAMMING algorithm  is

selected  by  CLEARING BIT 51 (33H) (i.e.  BIT 51 = ) before calling the

EPROM PROGRAMMING routine. The directly addressable BIT 51 is located in

internal memory location 38.3 (26 3H) (BIT 3 of BYTE 38 (26H) in  inter-

nal  memory).  This  BIT  can  be SET or CLEARED by the BASIC STATEMENTS

DBY(38) = DBY(38).OR.08H to SET and DBY (38)=DBY(38).AND.0F7H to  CLEAR.

Of course,  the user can set or clear this bit in assembly language with

a SETB 51 or CLR 51 instruction.

The user must also turn on the EPROM PROGRAMMING voltage BEFORE  calling

the  EPROM PROGRAMMING routine.  This is done by CLEARING BIT P1.5,  the

fifth  BIT  on  PORT  1.   This  too  can  be  done  in  BASIC  with   a

PORT1=PORT1.AND.0DFH instruction or in assembly language with a CLR P1.5

instruction.  The  user must also set this bit when the PROM PROGRAMMING

procedure is complete.

This instruction assumes that the hardware surrounding the MCS  BASIC-52

device  is  the same as the suggestions in the EPROM PROGRAMMING chapter

of this manual.

122

9.2 GENERAL PURPOSE ROUTINES

OPBYTE = 5(05H) INPUT A STRING OF CHARACTERS AND STORE IN THE BASIC  IN-

PUT BUFFER.

This instruction inputs a line of text from the console device and saves

the information in the MCS BASIC-52's input buffer. MCS BASIC-52's input

buffer begins at EXTERNAL MEMORY LOCATION 7(0007H). All of the line edit-

ing  features available in MCS BASIC-52 are implemented in this instruc-

tion.  If a control-C is typed during the input  process,  MCS  BASIC-52

will trap back into the command mode.  A carriage return (cr) terminates

the input procedure.

OPBYTE = 6(06H) OUTPUT THE  STRING  OF  CHARACTERS  POINTED  TO  BY  THE

REGISTER PAIR R3:R1 TO THE CONSOLE DEVICE.

This instruction is used to OUTPUT a string of characters to the console

device.  R3:R1  contains the initial address of this string.  The string

can either be stored in PROGRAM MEMORY or EXTERNAL DATA MEMORY.  If  BIT

52  (34H) (which is BIT 4 of internal RAM location 38 (26H) is set,  the

output will be from PROGRAM MEMORY.  If BIT 52 is  cleared,  the  output

will be from EXTERNAL DATA MEMORY. The DATA stored in MEMORY is sent out

to the console device one byte at a time and the memory pointer is incre-

mented.  The output is stopped when a termination character is read. The

termination character for EXTERNAL DATA MEMORY is a (cr) 0DH.  The  ter-

mination character for PROGRAM MEMORY is a " or 22H.

OPBYTE  = 7 (07) OUTPUT A CARRIAGE RETURN-LINE FEED SEQUENCE TO THE CON-

SOLE DEVICE.

Enough said.

OPBYTE = 128 (80H) OUTPUT THE CHARACTER IN R5 (REGISTER BANK 0)  TO  THE

CONSOLE DEVICE.

This  routines  takes  the character that is in R5 (register bank 0) and

directs it to the console device.  Any console device  may  be  selected

(i.e. U0 or U1 or the software serial port).

123

9.2 GENERAL PURPOSE ROUTINES

OPBYTE = 144 (90H) OUTPUT THE NUMBER ON THE TOP OF ARGUMENT STACK TO THE

CONSOLE DEVICE.

The  floating  point  number that is on the top of the argument stack is

outputted to the console device.  The FORMAT is determined by the  USING

statement. The argument stack is POPPED after the output operation.

OPBYTE  = 154 (9AH) THE 16 BIT NUMBER REPRESENTED BY REGISTER PAIR R2:R0

IS PUSHED ON THE ARGUMENT STACK.

This instruction converts the 16 bit register pair R2:R0 to  a  floating

point  number  and pushes this number onto the argument stack.  This in-

struction is the converse of the OPBYTE = 1 instruction.

124

9.3 UNARY OPERATORS

The next group of instructions perform an operation on the  number  that

is on the TOP of the ARGUMENT STACK. If the TOP of the ARGUMENT STACK is

represented  by the symbol [TOS],  then the following instructions would

take the general form:

[TOS] < OP [TOS]

Where OP is one of the following operators:

OPBYTE = 24 (18H)-ABSOLUTE VALUE

[TOS] <ABS([TOS]). The [TOS] is replaced by the absolute value of [TOS].

OPBYTE = 25 (19H)-INTEGER

[TOS] <INT([TOS]).  The [TOS] is replaced  by  the  integer  portion  of

[TOS].

OPBYTE = 26 (1AH)-SIGN

[TOS],SGN([TOS]). If [TOS] > 0 then [TOS] = 1, if [TOS] = 0 then [TOS] =

0, and if [TOS] < 0 then [TOS] = -1.

OPBYTE = 27 (1BH) - ONE'S COMPLEMENT

[TOS] < NOT([TOS]). [TOS] must be a valid integer.

OPBYTE = 28 (1CH) - COSINE OPERATOR

[TOS] <COS([TOS]). [TOS] must be between +-200000.

OPBYTE = 29 (1DH) - TANGENT OPERATOR

[TOS]  <  TAN([TOS]).  [TOS]  must  be between +-200000 and [TOS] cannot

equal PI/2, 3*PI/2, 5*PI/2, .... (2*N + 1)*PI/2.

125

9.3 UNARY OPERATORS

OPBYTE = 30 (1EH) - SINE OPERATOR

[TOS] <SIN([TOS]). [TOS] must be between +-200000.

OPBYTE = 31 (1FH) - SQUARE ROOT

[TOS] < SQR ([TOS]). [TOS] must be > = 0.

OPBYTE = 32 (20H) - CBY OPERATOR

[TOS] < CBY ([TOS]). [TOS] must be a valid integer.

OPBYTE = 33 (21H) - E TO THE [TOS] OPERATOR

[TOS] < e(2.7182818)**[TOS]. e is raised to the [TOS] power.

OPBYTE = 34 (22H) - ATN OPERATOR

[TOS] <ATN ([TOS]). Arctangent, the value returned is between +- PI/2.

OPBYTE = 35 (23H) - LOG OPERATOR (natural LOG)

[TOS] < LOG([TOS]) - [TOS] must be > 0.

OPBYTE = 36 (24H) - DBY OPERATOR

[TOS] < DBY([TOS]). [TOS] must be between 0 and 255 inclusive.

OPBYTE = 37 (25H) - XBY OPERATOR

[TOS] <XBY([TOS]). [TOS] must be a valid integer.

126

9.4 SPECIAL OPERATORS

The next group of instructions place a value on the stack.    The  value

placed on the stack is as follows:

OPBYTE = 38 (26H) - PI

[TOS] = PI.PI (3.1415926) is placed on the [TOS].

OPBYTE = 39 (27H) - RND

[TOS] = RND. A random number is placed on the [TOS].

OPBYTE = 40 (28H) - GET

[TOS] = GET.  The value of the SPECIAL FUNCTION OPERATOR,  GET is put on

the [TOS].

OPBYTE = 41 (29H) - FREE

[TOS] = FREE. The value of the SYSTEM CONTROL VALUE,  FREE is put on the

[TOS].

OPBYTE = 42 (2AH) - LEN

[TOS]  = LEN.  The value of the SYSTEM CONTROL VALUE,  LEN is put on the

[TOS].

OPBYTE = 43 (2BH) - XTAL

[TOS] = XTAL. The value of the SPECIAL FUNCTION OPERATOR, XTAL is put on

the [TOS].

OPBYTE = 44 (2CH) - MTOP

[TOS] = MTOP. The value of the SYSTEM CONTROL VALUE,  MTOP is put on the

[TOS].

127

9.4 SPECIAL OPERATORS

OPBYTE = 45 (2DH) - TIME

[TOS] = TIME. The value of the SPECIAL FUNCTION OPERATOR, TIME is put on

the [TOS].

OPBYTE = 46 (2EH) - IE

[TOS] = IE. The value of the IE register is put on the [TOS].

OPBYTE = 47 (2FH) - IP

[TOS] = IP. The value of the IP register is put on the [TOS].

OPBYTE = 48 (30H) - TIMER0

[TOS] = TIMER0. The value of TIMER0 (TH0:TL0) is put on the [TOS].

OPBYTE = 49 (31H) - TIMER1

[TOS] = TIMER1. The value of TIMER1 (TH1:TL1) is put on the [TOS].

OPBYTE = 50 (32H) - TIMER 2

[TOS] = TIMER2. The value of TIMER2 (TH2:TL2) is put on the [TOS].

OPBYTE = 51 (33H) - T2CON

[TOS] = T2CON. The value of the T2CON register is put on the [TOS].

OPBYTE = 52 (34H) - TCON

[TOS] = TCON. The value of the TCON register is put on the [TOS].

128

9.4 SPECIAL OPERATORS

OPBYTE = 53 (35H) - TMOD

[TOS] = TMOD. The value of the TMOD register is put on the [TOS].

OPBYTE = 54 (36H) - RCAP2

[TOS]  = RCAP2.  The value of the RCAP2 registers (RCAP2H:RCAP2L) is put

on the [TOS].

OPBYTE = 55 (37H) - PORT1

[TOS] = PORT1. The value of the PORT1 (P1) pins is placed on the [TOS].

OPBYTE = 56 (38H) - PCON

[TOS] = PCON. The value of the PCON register is put on the [TOS].

129

9.5 DUAL OPERAND OPERATORS

The next group of instructions assume that TWO values are on  the  ARGU-

MENT STACK.   If number on the TOP of the ARGUMENT STACK is presented by

the symbol [TOS] and the number NEXT to TOP of the ARGUMENT STACK is rep-

resented by the symbol [NxTOS] and the ARGUMENT STACK POINTER is  repre-

sented  by  the symbol AGSP,  then the following instructions would take

the general form:

TEMP1 = [TOS]

TEMP2 = [NxTOS]

AGSP  < AGSP+6

RESULT = TEMP2 OP TEMP1

[TOS]= RESULT

Where OP is one of the following operators to be  described.  NOTE  that

the  group of instructions ALWAYS POP the ARGUMENT STACK by one FLOATING

POINT NUMBER SIZE (i.e. 6 BYTES).

ERRORS can be handled in two different ways with the ADD, SUBTRACT, MUL-

TIPLY, and DIVIDE routines.  One option, is to let MCS BASIC-52 trap ER-

RORS,  should they occur during the operation.   With  this  option  MCS

BASIC-52 will print the appropriate error message to the console device.

The other option passes a STATUS CODE to the user.   After the operation

the Accumulator contains the status code information. The Status informa-

tion is as follows:

ACC.0 - ARITHMETIC UNDERFLOW

ACC.1 - ARITHMETIC OVERFLOW

ACC.2 - RESULT WAS ZERO (not an error, just a condition)

ACC.3 - DIVIDE BY ZERO

ACC.4 - NOT USED, ZERO RETURNED

ACC.5 - NOT USED, ZERO RETURNED

ACC.6 - NOT USED, ZERO RETURNED

ACC.7 - NOT USED, ZERO RETURNED

130

If an ARITH.  OVERFLOW or a DIVIDE BY ZERO ERROR occurs and the user  is

handling the error condition, the floating point processor will return a

result of +- 99999999E + 127 to the argument stack. The user can do what

they want to with this result (i.e. use it or waste it). An ARITH. UNDER-

FLOW ERROR will return to the argument stack a result of 0 (zero).

131

9.5 DUAL OPERAND OPERATORS

MCS BASIC-52 can perform the following DUAL OPERAND OPERATIONS:

OPBYTE  =  9  (09)  EXPONENTIATION  - The [NxTOS] value is raised to the

[TOS] power.  RESULT = [NxTOS]** [TOS].NOTE - [TOS] MUST  BE  LESS  THAN

256.

OPBYTE = 10 (0AH) MULTIPLY

RESULT = [NxTOS] * [TOS]. If an ERROR occurs during this operation (i.e.

ARITH. OVERFLOW or UNDERFLOW) MCS BASIC-52 will trap the error and print

the error message to the console device.

OPBYTE = 136 (88H) MULTIPLY

RESULT = [NxTOS] * [TOS]. If an ERROR occurs during this operation (i.e.

ARITH. OVERFLOW or UNDERFLOW) MCS BASIC-52 will trap the error and print

the error message to the console device.

OPBYTE = 136 (88H) MULTIPLY

RESULT = [NxTOS] * [TOS].  If an error occurs during this operation, the

status byte previously discussed will be returned to the user.

OPBYTE = 11 (0BH) ADD

RESULT = [NxTOS] + [TOS]. BASIC handles errors.

OPBYTE = 130 (82H) ADD

RESULT = [NxTOS] + [TOS]. User handles errors.

OPBYTE = 12 (0CH) DIVIDE

RESULT = [NxTOS] / [TOS]. BASIC handles errors.

OPBYE = 138 (8AH) DIVIDE

RESULT = [NxTOS] / [TOS]. User handles errors.

OPBYTE = 13 (0DH) SUBTRACT

RESULT = [NxTOS] - [TOS]. BASIC handles errors.

132

9.5 DUAL OPERAND OPERATORS

OPBYTE = 132 (84H) SUBTRACT

RESULT = [NxTOS] - [TOS]. User handles errors.

OPBYTE = 14 (0EH) EXCLUSIVE OR

RESULT = [NxTOS] XOR [TOS], both values must be GREATER THAN OR EQUAL TO

ZERO and LESS THAN OR EQUAL TO 65535 (0FFFFH).

OPBYTE = 15 (0FH) LOGICAL AND

RESULT = [NxTOS] and [TOS], both values must be GREATER THAN OR EQUAL TO

ZERO and LESS THAN OR EQUAL TO 65535 (0FFFFH).

OPBYTE = 16 (10H) LOGICAL OR

RESULT = [NxTOS] OR [TOS],  both values must be GREATER THAN OR EQUAL TO

ZERO and LESS THAN OR EQUAL TO 65535 (0FFFFH).

OPBYTE = 18 (12H) TEST FOR EQUALITY

IF [NxTOS] = [TOS] then, RESULT = 65535 (0FFFFH), else RESULT = 0

OPBYTE = 19 (13H) TEST FOR GREATER THAN OR EQUAL

IF [NxTOS] > = [TOS] then, RESULT = 65535 (0FFFFH), else RESULT = 0.

OPBYTE = 20 (14H) TEST FOR LESS THAN OR EQUAL

IF [NxTOS] < = [TOS] then, RESULT = 65535 (0FFFFH), else RESULT = 0.

133

9.5 DUAL OPERAND OPERATORS

OPBYTE = 21 (15H) TEST FOR NOT EQUAL

IF [NxTOS] <> [TOS] then, RESULT = 65535 (0FFFFH), else RESULT = 0.

OPBYTE = 22 (16H) TEST FOR LESS THAN

IF [NxTOS] < [TOS] then, RESULT = 65535 (0FFFFH), else RESULT = 0.

OPBYTE = 23 (17H) TEST FOR GREATER THAN

IF [NxTOS] > [TOS] then, RESULT = 65535 (0FFFFH), else RESULT = 0.

134

9.6 ADDED LINK ROUTINES TO VERSION 1.1

Version  1.1  of  MCS BASIC-52 contains a number of useful assembly lan-

guage link routines that were not available  in  Version  1.0.  Most  of

these  routines  were  designed  to  be used in conjunction with the new

Command/Statement extensions that are described in Chapter  11  of  this

manual.  The added link routines are as follows:

OPBYTE  =  57  (39H) EVALUATE AN EXPRESSION WITHIN THE BASIC TEXT STRING

AND PLACE THE RESULT ON THE ARGUMENT STACK

This routine permits the user to evaluate a BASIC expression [expr] con-

taining variables, operators and constants.  The result of the evaluated

expression is placed on the floating point argument stack.    This  lets

the  user  evaluate expressions in "customized" statements and commands.

An example of use of this OPBYTE is given at the end of this section.

OPBYTE = 58 (3AH) PERFORM CRYSTAL DEPENDENT CALCULATIONS WITH THE  VALUE

THAT IS ON THE ARGUMENT STACK

This  routine  is provided mainly to let the user write an assembly lan-

guage RESET routine and perform all of the  crystal  dependent  calcula-

tions  that  are  required by MCS BASIC-52.   An example of a customized

RESET routine that uses this OPBYTE is presented in Chapter 11  of  this

manual.

OPBYTE = 63 (3FH) GET A CHARACTER OUT OF THE BASIC TEXT STRING

This  routine  permits  the  user to "pick" a character out of the BASIC

program.  For instance, in BASIC the user could have the following:

10     CALL 1000H A

If the user executed the following in assembly language at 1000H:

MOV      A, #63

LCALL    30H

The character A would be returned in the accumulator.   The  Basic  text

pointer is located in located in location 8 (8H) (low byte) and 10 (0AH)

(high byte) of the internal ram on the MCS BASIC-52 device.  If the user

were to implement the above function, the basic text pointer must be ad-

vanced to the carriage return at the end of the statement before return-

ing  back  to  Basic.   Failure to do this will cause a BAD SYNTAX ERROR

when the user returns back to Basic.   The following can be used to  ad-

vance the Basic text pointer.

9.6 ADDED LINK ROUTINES TO VERSION 1.1

OPBYTE = 64 (40H) GET CHARACTER, THEN INCREMENT TEXT POINTER

135

This  OPBYTE  does the same thing as the previous one described,  except

that the BASIC text pointer is INCREMENTED AFTER the character is  read.

An example of this OPBYTE is presented at the end of this section.

OPBYTE  = 65 (41H) INPUT A CHARACTER FROM THE CONSOLE DEVICE,  PUT IT IN

THE ACCUMULATOR THEN RETURN

This OPBYTE permits the user to input characters from MCS BASIC-52's con-

sole input routine.   The character is placed in  the  accumulator  upon

return.

OPBYTE = 66 (42H) ENTER THE RUN MODE

This  OPBYTE  permits the user to start the execution of an MCS BASIC-52

program from assembly language.  The user need only insure the locations

19 (13H) and 20 (14H) of internal data memory contain the start  address

(high byte, low byte respectively) of the BASIC program.

OPBYTE  = 129 (81H) INPUT AN ASCII FLOATING POINT NUMBER AND PLACE IT ON

THE ARGUMENT STACK.  THE DPTR POINTS TO THE EXTERNAL RAM LOCATION, WHERE

THE ASCII TEXT STRING IS STORED

This routine assumes that the user has placed an ASCII text string some-

where in memory and that this ASCII text string represents a valid float-

ing point number.   The user then puts the DPTR to the starting  address

of this text string.  After this OPBYTE is executed the text string will

be converted to a valid MCS BASIC-52 floating point number and placed on

the argument stack and the DPTR will be advanced to the end of the float-

ing point number.  If the DPTR does not point to a text string that con-

tains  a  valid  floating point number,  the accumulator will contain an

0FFH upon return.

OPBYTE = 152 (98) OUTPUT, IN HEX, TO THE CONSOLE OUTPUT DRIVER, THE CON-

TENTS OF R3:R1

This routine is used to display HEX numbers,  assuming that they are  in

registers R3:R1.   If R3 = 0, leading zeros can be suppressed by setting

BIT 54 (36H) before calling this routine.   If BIT 54 (36H)  is  cleared

when  this  routine  is  called,  the driver will always output four hex

digits followed by the character H.  this routine always outputs a space

character (20H) to the console device, before any hex digits are output.

BIT 54 (36H) is bit 6 of internal RAM location 38.

9.6 ADDED LINK ROUTINES TO VERSION 1.1

EXAMPLE:

MCS-51 MACRO ASSEMBLER

ISIS-II MCS-51 MACRO ASSEMBLER V1.0

136

OBJECT MODULE PLACED IN :F4: DEMO.HEX

ASSEMBLER INVOKED BY:  ASM51 : F4: DEMO

LOC OBJ  LINE  SOURCE

1  ;*************************************************

2  ;

3  ; The following is an example of a program that

4  ; uses the new OPBYTES available in Version 1.1 of

5  ; MCS BASIC-52.  This code is by no means opti-

6  ; mized, but it is meant to demonstrate how the

user can define "customized" commands and state-

ments in Version 1.1 of MCS BASIC-52.

7  ;

8  ; The new command defined here is DISPLAY. What it

9  ; does is display a region of external data

10  ; memory to the console device. The syntax for

this statement is:

11  ;

12  ;              DISPLAY [expr], [expr]

13  ;

14  ; Where the first expression is the starting

15  ; address and the last expression is the ending

16  ; address.  In this example the DISPLAY is treated

17  ; like a command which means that it cannot be

; executed  in RUN mode.

18  ;

19  ; The output for the DISPLAY command is as follows

20  ;

21  ; ADDRESS then 16 Bytes of Characters i.e.

22  ;

137

23  ; 1000H 00H 22H 33H 27H .........

24  ;

25  ; Now, on to the program.

26  ; ************************************************

138

LOC  OBJ    LINE  SOURCE

27

2002          28      ORG     2002H

29

2002 5A       30      DB      5AH       ; Tell basic that

expansion option is

31                        ; present

2048          32      ORG     2048H

33

2048 D22D     34      SET B   45        ; Set the bit that says

so

204A 22       35      RET

36

2070          37      ORG     2070H     ; Set up DPTR to jump

table

38

2070 90207C   39      MOV     DPTR,#VECTOR_TABLE

2073 22       40      RET

41

208           42      ORG     2078H     ; Set up DPTR to ex-

pansion table

43

2078 90207E   44      MOV     DPTR, #USER_TABLE

207B 22       45      RET

46

47  VECTOR_TABLE:

48

207C 2087     49      DW      DO_DISPLAY; This is the address of

DISPLAY

139

50

51  USER_TABLE:

52

207E 10       53      DB      10H       ; Token for Display

207F 44495350 54      DB      'DISPLAY' ; ASCII for display

2083 4C4159

2086 FF       55      DB      0FFH      ; End of table (short

table)

56

57

58

59  DO_DISPLAY:

60

2087 302F63   61      JNB     47,DUMMY  ; make sure that MCS

BASIC-52 is in

62                        ; the command mode.

63                        ; Bit 47 is set if it is.

64

208A 7439     65      MOV     A,#57     ; Evaluate the first

expression after

208C 120030   66      LCALL   30H       ; the keyword display,

67                        ; MCS BASIC-52 will

68                        ; handle any errors. The

69                        ; value of the expression

70                          ;  will  be  on  the  ARGUMENT

STACK

140

208F 7440     71      MOV     A,#64     ; Get the character after

the expression

2091 120030   72      LCALL   30H       ; and bump the BASIC text

pointer

73

2094  B42C6A      74         CJNE    A,#',',C_ERROR ;  Make sure it is a

comma, if not do an

75                        ; error

76

2097 7439     77      MOV     A,#57     ; Evaluate the next ex-

pression (the

2099 120030   78      LCALL   30H       ; ending address) and put

it on the

79                        ; Argument Stack

80

209C 7401     81      MOV     A,#1      ; Convert the last ex-

pression (the

209E 120030   82      LCALL   30H           ;  ending  address)  on  the

stack to

83                        ; an integer and put it

R3:R1

84

20A1 8918     85      MOV     18H,R1    ; Save the ending address

in the user

20A3 8B19     86      MOV     19H,R3    ; reserved locations 18H

and 19H. This

87                        ; is reserved as register

bank 3

88

20A5 7401     89      MOV     A,#1      ; Convert the first ex-

pression (the

20A7 120030   90      LCALL   30H       ; starting address) on

the stack to

91                        ; an integer and put it

141

in R3:R1

92

20AA 891A     93      MOV     1AH,R1    ; Save the starting ad-

dress in the user

20AC 8B1B     94      MOV     1BH,R3    ; reserved locations 1AH

and 1BH

95

96                        ; Now everything is set

up to loop

97

20AE C3       98 LOOP1: CLR   C         ; Check to make sure that

the starting

20AF E518     99      MOV     A,18H     ; or current address is

<= the ending

20B1 951A    100      SUBB    A,1AH     ; address

20B3 E519    101      MOV     A,19H

20B5 951B    102      SUBB    A,1BH

20B7 5004    103      JNC     LOOP2     ; If the carry is set,

it's over

20B9 E4      104      CLR     A         ; Go to the command mode

20BA 020030  105      LJMP    30H       ; (if display was a

statement instead

106                        ; of a command, this

routine would

107                        ; exit with a RET

108

20BD 7407    109 LOOP2: MOV   A,#7      ; Do a carriage return,

line feed

20BF 120030  110      LCALL   30H

111

20C2 A91A    112      MOV     R1:1AH    ; Output the Starting

address

142

20C4 AB1B    113      MOV     R3,EBH

114

20C6 C236    115      CLR     36H       ; Don't suppress leading

zeros

20C8 7498    116      MOV     A,#98H

20CA 120030  117      LCALL   30H

118

20CD 851A82  119 LOOP3: MOV   DPL,1AH   ; Now, set up to read

16 bytes

20D0 851B83  120      MOV     DPH,1BH   ; put address in DPTR

121

20D3 E0      122      MOVX    A, @DPTR  ; Read the byte in exter-

nal RAM

20D4 A3      123      INC     DPTR      ; Bump to the next loca-

tion

124

20D5 85821A  125      MOV     1AH,DPL   ; Save the address

20D8 85831B  126      MOV     1BH,DPH

127

20DB F9      128      MOV     R1,A      ; Output the byte

20dc 7B00    129      MOV     R3,#0     ; The high byte is always

zero

20DE D236    130      SETB    36H       ; Suppress leading Zeros

20E0 7498    131      MOV     A,#98H

20E2 120030  132      LCALL   30H

133

20E5 E51A    134      MOV     A,1AH     ; Check to see if on a 16

byte boundary

20E7 540F    135      ANL     A,#0FH

143

20E9 70E2    136      JNZ     LOOP3     ; Loop until on a 16 Byte

Boundary

20EB 80C1    137      SJMP    LOOP1

138

139 DUMMY:

140

20ED 7407    141      MOV     A,#7      ; Do a carriage return-

line feed

20EF 120030  142      LCALL   30H

143

20F2 7B21    144      MOV     R3,#HIGH D_MSG ; Display the error

message

20F4 7915    145      MOV     R1,#LOW D_MSG

20F6 D234    146      SETB    52        ; Print from ROM

20F8 7406    147      MOV     A,#6

20FA 120030  148      LCALL   30H

20FD E4      149      CLR     A         ; Go back to the command

mode

20FE 020030  150      LJMP    30H

151

152 C_ERROR:

153

2101 7407    154      MOV     A,#7      ; Do what we did before

2103 120030  155      LCALL   30H

156

2106 7B21    157      MOV     R3,#HIGH C_MSG

2108 793B    158      MOV     R1,#LOW C_MSG

210A D234    159      SETB    52

210C 7406    160      MOV     A,#6

144

210E 120030  161      LCALL   30H

145

2111 E4      162      CLR     A

2112 020030  163      LJMP    30H

164

2115 44495350 165 D_MSG: DB   'DISPLAY IS A COMMAND, NOT A STATE-

MENT"'

2119 4C415920

211D 49532041

2121 20434F4D

2125 4D414E44

2129 2C204E4F

212D 54204120

2131 53544154

2125 454D454E

2139 5422

166

213B 594F5520 167 C_MSG: DB   ' YOU NEED A COMMA TO MAKE DISPLAY

WORK"'

213F 4E454544

2143 20412043

2147 4F4D4D41

214B 20544F20

214F 4D414B45

2153 20444953

2157 504C4159

215B 20574F52

215F 4B22

146

168

169         END

ASSEMBLY COMPLETE, NO ERRORS FOUND

(that's all it takes)

147

9.7 INTERRUPTS

Interrupts  can  be  handled by MCS BASIC-52 in two distinct ways.   The

first,  which has already been discussed,  allows statements in  an  MCS

BASIC-52  program to perform the required interrupt routine.  The ONTIME

and ONEX1 statements enable this particular  interrupt  mode.  Addition-

ally,  setting BIT 26.1H permits EXTERNAL INTERRUPT 0 to act as a "fake"

DMA input and the details of this feature are in  the  BELLS,  WHISTLES,

and ANOMALIES section of this manual.  The second method of handling in-

terrupts  in  MCS  BASIC-52 allows the programmer to write assembly lan-

guage routines to perform the interrupt task.  This method yields a much

faster interrupt response time,  but,  the programmer must exercise some

caution.

All interrupt vectors on the MCS BASIC-52 device are "mirrored" to exter-

nal PROGRAM MEMORY LOCATIONS 4003H through 402BH inclusive. The only MCS

BASIC-52 STATEMENTS that enable the interrupts on  the  8052AH  are  the

CLOCK1 and the ONEX1 STATEMENTS.  If interrupts are NOT enabled by these

STATEMENTS,  BASIC assumes that the  USER  is  providing  the  interrupt

routine in assembly language. The vectors for the various interrupts are

as follows:

LOCATION---INTERRUPT

4003H------EXTERNAL INTERRUPT 0

400BH------TIMER 0 OVERFLOW

4013H------EXTERNAL INTERRUPT 1

401BH------TIMER 1 OVERFLOW

4023H------SERIAL PORT

402BH------TIMER 2 OVERFLOW/EXTERNAL INTERRUPT 2

The programmer can enable interrupts in MCS BASIC-52 by using the state-

ment  IE = IE.OR.XXH,  where XX enables the appropriate interrupts.  The

bits in the interrupts register (IE) on the 8052AH are defined  as  fol-

lows:

BIT   7     6       5      4       3      2     1      0

EA    X      ET2     ES     ET1    EX1   ET0    EX0

ENABLE  UNDE- TIMER  SERIAL  TIMER   EXT  TIMER   EXT

ALL   FINED   2     PORT    1       1     0      0

9.7 INTERRUPTS

148

Interrupts  are enabled when the appropriate BITS in the IE register are

set to a one. Details of the 8052AH interrupt structure are available in

the MICROCONTROLLER USERS MANUAL available from the INTEL.

IMPORTANT NOTE!!

Before MCS  BASIC-52  vectors  to  the  USER  interrupt  locations  just

described, the PROCESSOR STATUS WORD (PSW) is PUSHED onto the STACK. So,

the  USER  does not have to save the PSW in the assembly language inter-

rupt routine!!! HOWEVER, THE USER MUST POP THE PSW BEFORE RETURNING FROM

THE INTERRUPT.

VERY IMPORTANT NOTE!!!

If the user is running some interrupt driven "background" routine  while

MCS BASIC-52 is running a program, the user MUST NOT CALL any of the as-

sembly language routines available in the MCS BASIC-52 device.  The only

way the routines in the MCS BASIC-52 device can be accessed is when  the

CALL  statement in MCS BASIC-52 is used to transfer control to the users

assembly language program.  The reason for this is that the MCS BASIC-52

interpreter  must  be  in  a  "known" state before the user can call the

routines available in the MCS BASIC-52 device and a  "random"  interrupt

does not guarantee that the interpreter is in this known state. The user

should use REGISTER BANK 3 to handle interrupt routines in assembly lan-

guage.

149

9.8 RESOURCE ALLOCATION

Specific  statements in MCS BASIC-52 require the use of certain hardware

features on the device. If the user wants to use these hardware features

for interrupt driven routines, conflicts between BASIC  and the assembly

language routine may occur.  To avoid  these  potential  conflicts,  the

programmer  needs  to know what hardware features are used by MCS BASIC-

52.  The following is a list of the COMMANDS and/or STATEMENTS that  use

the hardware features on the 8052AH.

CLOCK 1 - uses TIMER/COUNTER 0 in the 13 bit 8048 mode

PWM - uses TIMER/COUNTER 1 in the 16 bit mode

LIST# - uses TIMER/COUNTER 1 to generate baud rate in 16 bit

mode

PRINT# - same as LIST#

PROG - uses TIMER/COUNTER 1 for programming pulse

ONEX1 - uses EXTERNAL INTERRUPT 1

In  addition,  TIMER/COUNTER 2 is used to generate the baud rate for the

serial port.  What the preceding list means  is  that  if  CLOCK1,  PWM,

ONEX1,  LIST#,  PRINT#,  and  PROG  commands/statements  are used by the

programmer,  the user MAY NOT use the associated TIMER/COUNTER or EXTER-

NAL INTERRUPT pin for an assembly language routine.

MCS  BASIC-52  initializes  the  TIMER/COUNTER  modes  by  writing 1 244

(0F4H), 16 (10H),  and 52 (34H) to the TCON,  TMOD,  and T2CON registers

respectively.  These registers are initialized only during the RESET in-

itialization sequence, and MCS BASIC-52 assumes that these registers are

NEVER changed. So,  if the user changes the contents of TCON,  TMOD,  or

T2CON,  something  funny  and/or  disastrous  is  bound to happen if the

Statements/Commands listed above are executed.  If the user does not ex-

ecute  any of the previously mentioned Statements or Commands,  the user

is free to use the interrupts in any way suitable to the application.

150

CHAPTER 10

System Configuration

10.1 MEMORY/HARDWARE CONFIGURATION

MCS BASIC-52 always requires at least 1K bytes of external memory. After

reset, MCS BASIC-52 will not "sign on", in fact, it will internally loop

forever. This obviously is not too exciting,  so it is wise to hang some

external memory on the MCS BASIC-52 device.

MCS  BASIC-52 sizes consecutive external memory locations from 0000H un-

til a memory failure is detected.  The  sizing  operation  is  performed

simply  by  writing  a 5AH to an external memory location,  then testing

location. If the particular memory location passes this test, BASIC then

writes a 00H to the location,  then  again,  checks  the  location.  MCS

BASIC-52 only sizes the external memory from locations 0 through 0DFFFH.

Memory  locations 0E000H through 0FFFFH are reserved for user I/O and/or

assembly language programs.

The MCS BASIC-52 program resides in the 8K of ROM available  in  INTEL's

8052AH   device   and   as   a  result  requires  that  external  memory

be"partitioned" in a specific manner.  The architecture of the 8052AH is

NOT  Von Neumann.  This means that Data and Program Memory do not reside

in the same physical address space on the 8052AH. Specifically,  the \RD

(pin  17)  and  \WR  (pin 16) pins on the 8052AH are used to enable DATA

memory and \PSEN (pin 29) pin is used to enable PROGRAM memory.  Depend-

ing on the hardware configuration, MCS BASIC-52 operates in two distinct

" memory" modes.

RAM ONLY MODE

In  this  mode  of operation,  Read/write memory is connected to the MCS

BASIC-52 device starting at memory address 0000H.  Memory can be  placed

up  to location 0FFFFH.  In this mode of operation the decoded addresses

are used to generate the CHIP SELECT \(CS) signal foR the  RAM  devices.

The  \RD  pin  on the 8052AH is used to generate the OUTPUT ENABLE \(OE)

strobe and the \WR pin generates the WRITE ENABLE (\WE or  \WR)  strobe.

\PSEN  is not used in the RAM only mode of operation.  The RAM only mode

of operation offers the simplest hardware  configuration  available  for

the  MCS  BASIC-52 device.  An example of this configuration is shown in

Figure 1. Since \PSEN is not used in the RAM only mode, the user may not

CALL assembly language routines.  The RAM only  also  does  not  support

EPROM  programming.  In general,  the RAM only mode will be used only to

"check out" the device during the initial system development stage.

10.1 MEMORY/HARDWARE CONFIGURATION

RAM/EPROM MODE

151

The RAM/EPROM mode of operation allows for the complete system implemen-

tation of MCS BASIC-52.  This mode of operation requires  that  external

memory be mapped in a certain manner. The RAM/EPROM memory configuration

is as follows:

1)  The  \RD  and  the  \WR  pins on the MCS BASIC-52 device are used to

enable RAM memory that is addressed from 0000H to 7FFFH.  Addresses  are

used to decode the chip select (\CS) for the RAM devices and \RD and \WR

are used to enable the \OE and \WE or (\WR) pins respectively.

2)  The  \PSEN  pin  on  the MCS BASIC-52 device is used to enable EPROM

memory that is addressed from 2000H to  7FFFH.  Addresses  are  used  to

decode  the chip select (\CS) for the EPROM devices and \PSEN is used to

enable the \OE pin.

3) For addresses between 8000H and 0FFFFH both the \RD and the \PSEN pin

on the MCS BASIC-52 device are used to enable the memory.  Either  EPROM

or  RAM devices can be placed in this address space.  To permit both the

\RD and the \PSEN pins to enable addresses in this  address  space,  \RD

and  \PSEN must be logically "ANDED" together.  This can be accomplished

with a simple TTL gate such as a 74LS08. The \WR pin on the MCS BASIC-52

device is used to write to RAM memory in this same  address  space.  The

\PSEN  and  \RD  signals do not have to be anded beyond address 7FFFH to

enable MCS BASIC-52 to program an EPROM. This is only a suggestion since

it will permit the user to execute assembly language routines as well as

MCS BASIC-52 programs that are located in this address space.

152

10.1 MEMORY/HARDWARE CONFIGURATION

This scheme of memory addressing actually permits MCS  BASIC-52  to  ad-

dress 96K bytes of memory,  32K of RAM devices, 32K of EPROM/ROM devices

and 32K of combined RAM/EPROM/ROM devices. Since \RD and \PSEN are ANDED

for addresses from 8000H through 0FFFFH,  the 8052AH "looks like" a  Von

Neumann machine in this address space.  The XBY and CBY special function

operators will yield the same value when  their  arguments  are  between

8000H and 0FFFFH.

When  the  EPROM programming feature in MCS BASIC-52 is used,  BASIC as-

sumes that the EPROM to be programmed is addressed starting at  location

8000H.  MCS BASIC-52 can only program EPROMS addressed between 8000H and

0FFFFH.  When the PROG command is used for the first time,  on an erased

EPROM, MCS BASIC-52 stores this program beginning at address 8010H. Loca-

tions  8000H  through  800FH are used to save the baud rate information,

plus configuration information.  Some suggestions for implementation  of

the RAM/EPROM mode are shown in figure 2.

153

10.2 EPROM PROGRAMMING CONFIGURATION/TIMING

With the proper hardware, the MCS BASIC-52 device can program just about

any  EPROM or EEPROM device.  The only requirement for EPROM programming

is that the EPROM to be programmed is  addressed  starting  at  location

8000H.  MCS  BASIC-52 requires very little external hardware to programs

EPROMS.  All of the critical EPROM programming timings are generated  by

three  I/O port pins on the MCS BASIC-52 device.  These pins provide the

following signals:

P1.3 - ALE DISABLE

PORT 1, BIT 3 (pin 4 on the 8052AH) is used to DISABLE the ALE signal to

the external latched required by the 8052AH when external memory is  ad-

dressed.  This pin should be logically ANDED with ALE.A simple TTL gate,

such as a 74LS08 can be used to perform the ANDING function.  Under nor-

mal  operation,  P1.3  is  in logical high state (1).  ONLY DURING EPROM

PROGRAMMING IS P1.3 PLACED IN A LOGICAL LOW STATE (0). Disabling the ALE

signal to the external latch is required to program  EPROMS  because  of

the way MCS BASIC-52 carries out the EPROM programming process.

During programming, MCS BASIC-52 treats I/O PORT 0 and I/O PORT 2 as I/O

ports,  not as address/data ports. MCS BASIC-52 first writes the low or-

der address to be programmed to PORT 0.  The data  in  PORT  0  is  then

latched  into  the external address latch and then MCS BASIC-52 disables

the ALE signal to the latch by clearing bit P1.3.  Thus,  the low  order

address is "permanently" stored in the external latch. MCS BASIC-52 then

writes the high order address to PORT 2 and the DATA to be programmed to

PORT 0.  So,  the external address latch contains the low order address,

PORT 2 contains the high order address,  and PORT 0  contains  the  DATA

when EPROM programming occurs.

IMPORTANT NOTES

When  PORT 0 on the 8052AH is used as an I/O port,  the output structure

is an "open drain" configuration. This requires that "pull-up" resistors

be placed on PORT 0 to permit MCS BASIC-52 to program EPROMS. Experimen-

tally,  10K ohm pull-ups resistors on PORT 0 have  yielded  satisfactory

results.

In Version 1.1, INTO must be kept high when programming EPROMs.

154

10.2 EPROM PROGRAMMING CONFIGURATION/TIMING

P1.4 - PROGRAM PULSE WIDTH

PORT  1,  BIT  4  (pin  5  on the 8052AH) is used to provide the 50 mil-

lisecond or the 1 millisecond  programming  pulse.  The  length  of  the

programming   pulse  is  determined  by  whether  the  "normal"  or  the

"INTELligent" EPROM programming mode is selected.  MCS  BASIC-52  calcu-

lates  the  length  of  the  programming pulse from the assigned crystal

value.  So,  be sure the proper XTAL has been assigned.  The accuracy of

this pulse is within 10 CPU clock cycles. This pin is normally in a logi-

cal high (1) state. It is asserted low (0) to program the EPROMS. Depend-

ing  on the EPROM to be programmed this signal will be used in different

ways. More about this later.

P1.5 - ENABLE PROGRAM VOLTAGE

PORT 1, BIT 5 (pin 6 on the 8052AH) is used to enable the EPROM program-

ming voltage. This pin is normally in a logical high (1) state. Prior to

the EPROM programming operation,  this pin is brought to a  logical  low

(0)  state.  This  pin  is used to turn on or off the high voltage (12.5

volts to 25 volts,  depending on the  EPROM)  required  to  program  the

EPROMS.

The  timing  for  the  EPROM programming pins is shown in figure 3.  The

hardware required to program different devices is  shown  in  figure  4.

Note that with very little external hardware the MCS BASIC-52 device can

program  virtually  all  commercially  available  EPROMS.  Additionally,

figure 5 suggest a circuit using an INTEL  2816A  EEPROM.  This  circuit

also features a push button erase option.

IMPORTANT NOTE

MCS  BASIC-52 calculates the programming pulse width when the XTAL value

is assigned.  To ensure proper programming,  make sure XTAL is  assigned

the proper value. MCS BASIC-52 performs the programming pulse width cal-

culation  to  within 5 clock cycles,  so the accuracy of the programming

pulse is well within the limits of any EPROM device.

10.3 SERIAL PORT IMPLEMENTATION

The serial port I/O signals on the 8052AH are  TTL  compatible  signals.

They are typically not compatible with most terminals.  Figure 6 suggest

hardware options for making the serial interface  compatible  with  ter-

minals. The serial port is initialized by MCS BASIC-52 to the 8-bit uart

mode.  In  this  mode  8 data bits,  plus one start and one stop bit are

transmitted. Parity is not used.

155

CHAPTER 11

Reset Options (Version 1.1 Only)

Version 1.1 of MCS BASIC-52 contains numerous RESET  options  that  were

not  available in Version 1.0.  They are discussed in detail in Chapters

3.2 through 3.5 of this manual. Briefly, they are as follows:

PROG1

Saves only the serial port baud rate for a power-up or RESET condition.

PROG2

Saves the serial port baud rate and automatically runs the first program

that is saved in EPROM on a power-up or RESET condition.

PROG3

Saves the serial port baud rate plus the assigned MTOP value.  If RAM is

available beyond the assigned MTOP value,  it will not be cleared during

a power-up or RESET condition.

PROG4

Saves the serial port baud rate plus the assigned MTOP value,  just like

PROG3,  but  also  automatically runs the first program that is saved in

EPROM on a power-up or RESET condition.

PROG5

Does the same thing as PROG4,  however,  if external memory location 5FH

contains  the character 0A5H on a power-up or RESET condition,  external

memory will not be cleared. This mode assumes that the user has employed

some type of memory back-up.

156

RESET OPTIONS (VERSION 1.1 ONLY)

PROG6

Does the same thing as PROG5, but CALLS external program memory location

4039H during a RESET or power-up sequence. This option also requires the

user to put the character 0A5H in external memory location 5FH to insure

that external RAM will not be cleared during RESET or power-up. The user

must put an assembly language RESET routine in external memory  location

4039H or else this RESET mode will crash. When the user returns from the

customized assembly language RESET routine, three options exist:

OPTION 1 FOR PROG6

If  the  CARRY BIT is CLEARED (CARRY =0) upon return from the user RESET

routine MCS BASIC-52 will enter the auto-baud rate determining  routine.

The  user must then type a space character (20H) on the terminal to com-

plete the RESET routine and produce a RESET message on the terminal.

OPTION 2 FOR PROG6

If the CARRY BIT is SET (CARRY = 1) and BIT  0  of  the  ACCUMULATOR  is

CLEARED (ACC. 0 = 0) MCS BASIC-52 will produce the standard sign-on mes-

sage  upon  return  from the user supplied RESET routine.  The baud rate

will be the one that was saved when the PROG6 option was used.

OPTION 3 FOR PROG6

If the CARRY BIT is SET (CARRY = 1) and BIT 0 of the ACCUMULATOR is  SET

(ACC.  0 = 1), MCS BASIC-52 will execute the first program stored by the

user in EPROM (starting address of the program  is  8010H)  upon  return

from the user supplied RESET routine.

157

RESET OPTIONS (VERSION 1.1 ONLY)

If  these  options  are  still  not sufficient to address the needs of a

specific application,  one other option exists and it functions as  fol-

lows:

After  RESET,  MCS  BASIC-52  initializes the SPECIAL FUNCTION REGISTERS

SCON, TMOD, TCON,  and T2CON with the following respective values,  5AH,

10H, 54H and 34H. If the user places the character 0AAH in external CODE

MEMORY  location  2001H (remember CODE MEMORY is enabled by \PSEN),  MCS

BASIC-52 will CALL external CODE MEMORY location 2090H immediately after

these special function registers are initialized.  No other registers or

memory  locations  will be altered except that the ACCUMULATOR will con-

tain a 0AAH and the DPTR will contain a 2001H.

Since MCS BASIC-52 does not write to the above mentioned  Special  Func-

tion  Registers at any time except during the RESET or power-up sequence

the user has the  option  of  modifying  any  of  the  Special  Function

Registers  with this RESET option.  Upon returning from this RESET mode,

the MCS BASIC-52 software package will clear the internal memory of  the

8052AH  and proceed with the RESET routine.  The PROG1 through PROG6 op-

tions  will function as usual.

Now,  suppose the user does not want to enter the normal RESET routines,

or  the  user  wants  to implement some type of "warm" start-up routine.

This can be accomplished simple by initializing  the  necessary  Special

Function Registers and then jumping back into either MCS BASIC-52's COM-

MAND mode or RUN MODE. For a warm start-up or RESET (warm means that the

MCS BASIC-52 device was RESET, but power was not removed - i.e. the user

hit the RESET button) the following must be initialized:

SCON,  TMOD,  TCON,  T2CON,  if the user does not want to use the values

that MCS BASIC-52 supplies.

RCAP2H and RCAP2L must be loaded with the proper baud  rate  values.  If

the user has programmed an EPROM with one of the PROG1 through PROG6 op-

tions, the proper baud rate value will be stored in external DATA MEMORY

locations 8001H (RCAP2H) and 8002H (RCAP2L).

The  STACK  POINTER  (Special  Function Register SP) must be initialized

with the contents of the STACK POINTER SAVE location, which is in inter-

nal DATA MEMORY location 3EH.  A MOV SP,  3EH assembly language instruc-

tion will accomplishment the STACK POINTER initialization.

158

RESET OPTIONS (VERSION 1.1 ONLY)

After the above are initialized by the user supplied RESET routine,  the

user may enter MCS BASIC-52's command mode by executing the following:

CLR          A

LJMP         30H

Now,  it is important to remember that the previous description  applies

only  to a "warm" RESET with power remaining to the MCS BASIC-52 system.

This means that the user must also provide some way of detecting the dif-

ference between a warm RESET and a power-on RESET. This usually involves

some type of flip-flop getting set with a power-on-clear signal from the

users power supply.  The details of implementing  this  RESET  detection

mechanism  will  not  be discussed here as the possible hardware options

vary depending upon the design.

The user may also implement a "cold start" reset option with the  previ-

ously described reset mode. The following code details what is necessary

to implement a cold start option.

EXAMPLE:

ORG   2001H

;

DB    0AAH     ; TELL BASIC THAT RESET IS EXTERNAL

;

ORG   2090H    ; LOCATION BASIC WILL CALL FOR RESET

;

; AT THIS POINT BASIC HAS PLACED A 5AH IN

; SCON, A 10H IN TMOD, A 54H IN TCON AND

; A 34H IN T2CON

;

; FIRST CLEAR THE EXTERNAL MEMORY

MOV   R0, #0FFH; LOAD R0 WITH THE TOP OF INTERNAL MEMORY

CLR   A        ; SET ACCUMULATOR = 0

;

RESET1:MOV @R0,A    ; LOOP UNTIL ALL THE INTERNAL RAM IS CLEARED

159

DJNZ  R0,RESET1

;

; NOW SET UP THE STACK POINTER AND THE STACK

; POINTER HOLDING REGISTER

MOV   SP,#4DH  ; 4DH IS THE INITIALIZED VALUE OF THE STACK

MOV   3EH,#4DH ; THIS IS THE SP HOLDING REGISTER

;

; NOW CLEAR THE EXTERNAL RAM, IN THIS

; EXAMPLE ASSUME THAT 1FFFH BYTES OF RAM

; ARE AVAILABLE

; THE USER MUST CLEAR AT LEAST THE FIRST 512

; BYTES OF RAM FOR A COLD START RESET

;

MOV   R3,#HIGH 1FFFH

MOV   R1,#LOW 1FFFH

MOV   DPTR,#0FFFFH

;

160

RESET OPTIONS (VERSION 1.1 ONLY)

RESET 2: INC    DPTR         ; DPTR = 0 THE FIRST TIME THRU

CLR A

MOVX   @DPTR,A      ; CLEAR THE RAM , A MEMORY TEST

; PROGRAM COULD BE IN THIS LOOP

MOV    A, R3        ; NOW TEST FOR THE MEMORY LIMITS

CJNE   A, DPH,RESET2

MOV    A, R1

CJNE   A, DPL,RESET2

;

; WHEN YOU GET HERE, YOU ARE DONE

;

; NOW SET UP THE MEMORY POINTERS, FIRST MTOP

;

MOV    DPTR, #10AH   ; LOCATION OF MTOP IN EXTERNAL

RAM

MOV    A, #HIGH 1FFFH;SAVE MTOP

MOVX   @DPTR, A

INC    DPTR          ; NOW, SAVE THE LOW BYTE

MOV    A, #LOW 1FFFH

MOVX   @DPTR, A

;

; NOW SET UP THE VARTOP POINTER, WITH NO STRINGS,

; VARTOP = MEMTOP

;

MOV    DPTR, #104H   ; LOCATION OF VARTOP IN EXTERNAL

RAM

MOV    A, #HIGH 1FFFH

MOVX   @DPTR, A

INC    DPTR

MOV    A,#LOW 1FFFH

MOVX   @DPTR, A

;

;

;  NOW  SAVE  THE  MATRIX  POINTER  "DIMUSE",  THIS  POINTER

; IS DESCRIBED IN THE APPENDIX, WITH NO PROGRAM IN

RAM

; DIMUSE = 528 AFTER RESET

;

MOV  DPTR, #108H     ; LOCATION OF DIMUSE IN EXTERNAL

RAM

MOV    A, #HIGH 528

MOVX   @DPTR, A

INC    DPTR

MOV    A, #LOW 528

MOVX   @DPTR, A

;

; NOW SAVE THE VARIABLE POINTER "VARUSE" THIS POINTER

; IS ALSO DESCRIBED IN THE APPENDIX, AFTER RESET

VARUSE = VARTOP

161

;

MOV    DPTR, #106H   ; LOCATION OF VARUSE IN EXTERNAL

RAM

MOV    A, #HIGH 1FFFH

MOVX   @DPTR, A

INC    DPTR

MOV    A, #LOW 1FFFH

MOVX   @DPTR, A

;

; NOW SETUP BASICS CONTROL STACK AND ARGUMENT STACK

;

MOV    9H, #0FEH     ; THIS INITIALIZES THE ARGUMENT

STACK

MOV    11H, #0FEH    ; THIS INITIALIZES THE CONTROL

STACK

;

; NOW TELL BASIC THAT NO PROGRAM IS IN RAM, THIS IS

; NOT NEEDED IF THE USER HAS A PROGRAM IN RAM

;

MOV    DPTR, #512    ; LOCATION OF THE START OF A

USER PROGRAM

MOV    A, #01H       ; END OF FILE CHARACTER

MOVX   @DPTR, A

162

RESET OPTIONS (VERSION 1.1 ONLY)

;

; NOW PUSH THE CRYSTAL VALUE ON TO THE STACK AND LET

BASIC

; CALCULATE ALL CRYSTAL DEPENDENT PARAMETERS

;

SJMP   RESET3

;

XTAL:   DB     88H           ; THIS IS THE FLOATING POINT

VALUE

DB     00H           ; FOR AN 11.0592 MHZ CRYSTAL

DB     00H

DB     92H

DB     05H

DB     11H

;

;

RESET3:   MOV    DPTR, #XTAL   ; SET UP TO PUSH CRYSTAL VALUE

MOV    A, 9          ; GET THE ARG STACK

CLR    C

SUBB   A, #6         ; DECREMENT ARG STACK BY ONE FP

NUMBER

MOV    9, A

MOV    R0, A         ; SAVE THE CALCULATED ADDRESS IN

R0

MOV    P2, 31        ; THIS IS THE ARG STACK PAGE

ADDRESS

MOV    R1, #6        ; NUMBER OF BYTES TO TRANSFER

;

RESET4:   CLR    A             ; TRANSFER ROM CRYSTAL VALUE

TO THE

MOVC   A, @A+DPTR    ; ARGUMENT STACK OF BASIC

MOVX   @R0, A

INC    DPTR          ; BUMP THE POINTERS

DEC    R0

DJNZ   R1, RESET4    ; LOOP UNTIL THE TRANSFER IS

COMPLETE

;

; NOW CALL BASIC TO DO ALL THE CRYSTAL CALCULATIONS

;

MOV    A, #58        ; OPBYTE FOR CRYSTAL CALCULATION

LCALL  30H           ; DO THE CALCULATION

; NOW TELL BASIC WHERE START OF THE USER BASIC

PROGRAM IS

; BY LOADING THE START ADDRESS, IF THE PROGRAM IS IN

EPROM

; 13H WOULD = HIGH 8011H AND 14H = LOW 8011H, ANYWAY

; ADDRESS 13H: 14H MUST POINT TO THE START OF THE

BASIC

; PROGRAM

;

163

MOV    13H, #HIGH 512; THIS TELLS BASIC THAT THE

START OF

MOV    14H, #LOW 512 ; THE PROGRAM IS IN LOCATION 512

;

; NOW THE SERIAL PORT MUST BE INITIALIZED, THE USER

; CAN SET UP THE SERIAL TO ANY DESIRED CONFIGURATION

; HOWEVER, THIS DEMO CODE WILL SHOW THE AUTO BAUD

; ROUTINE

;

MOV    R3, #00H      ; INITIALIZE THE AUTO BAUD

COUNTERS

MOV    R1, #00H

MOV    R0, #04H

JB     RXD, $        ; LOOP UNITL A START BIT IS

RECEIVED

;

164

RESET OPTIONS (VERSION 1.1 ONLY)

;

RESET5:   DJNZ   R0, $         ; WASTE 8 CLOCKS INITIALLY, SIX

CLOCKS

; IN THE LOOP (16) TOTAL

CLR    C             ; 1 CLOCK (1)

MOV    A, R1         ; 1 CLOCK (2)

SUBB   A, #1         ; 1 CLOCK (3)

MOV    R1, A         ; 1 CLOCK (4)

MOV    A, R3         ; 1 CLOCK (5)

SUBB   A, #00H       ; 1 CLOCK -- R3:R1 - 1 (6)

MOV    R3, A         ; 1 CLOCK (7)

MOV    R0, #3        ; 1 CLOCK (8)

JNB    RXD, RESET5   ; 2 CLOCKS (10), WAIT FOR END OF

SPACE

JB     RXD, $        ; WAIT FOR THE SPACE TO END

(20H)

JNB    RXD, $        ; WAIT FOR THE STOP BIT

MOV    RCAP2H, R3    ; LOAD THE TIMER 2 HOLDING

REGISTERS

MOV    RCAP2L, R1

;

; NOW YOU CAN ADD A CUSTOM SIGN ON MESSAGE

;

MOV    R3, #HIGH MSG ; PUT ADDRESS OF MESSAGE IN

R3:R1

MOV    R1, #LOW MSG

SETB   52            ; PRINT FROM ROM

MOV    A, #6         ; OP BYTE TO PRINT TEXT STRING

LCALL  30H

;

; NOW OUTPUT A CR LF

;

MOV    A, #7         ; OP BYTE FOR CRLF

LCALL  30H

;

; GO TO THE COMMAND MODE

;

CLR    A

JMP    30H

;

MSG:      DB     'CUSTOM SIGN ON MESSAGE'

DB     22H           ; TERMINATES MESSAGE

;

END

165

RESET OPTIONS (VERSION 1.1 ONLY)

To summarize what the user do to successfully implement a  "COLD  START"

RESET:

1)  The  user must clear the internal RAM of the MCS BASIC-52 device and

at least the first 512 bytes of external RAM memory.

2) The user must initialize the stack pointer (special function register

- SP) and the stack pointer holding register (internal RAM location 3EH)

with a value that is between 4DH and 0E0H.  4DH gives MCS  BASIC-52  the

maximum stack size.

3) The user must initialize the following pointers in external RAM. MTOP

at  location  10AH (high byte) and 10BH (low byte).  VARTOP at locations

104H (high byte) and 105H (low byte.  DIMUSE  at  locations  108H  (high

byte) and 109H (low byte). VARUSE at locations 106H (high byte) and 107H

(low byte). Details of what needs to be in these locations are presented

in appendix 1.7 of this manual.

4.  The  Control stack pointer (location 11H in internal memory) and the

Argument stack pointer (location 09H in internal memory)  must  also  be

initialized with the value 0FEH.  If the user is not going to assign the

XTAL (crystal) value in BASIC,  then the XTAL value must be pushed  onto

the  argument stack and the user must to an OPBYTE 58 call to MCS BASIC-

52.

5) The user must also initialize the start address  of  a  program.  The

start  address is in locations 13H (high byte) and 14H (low byte) of in-

ternal data memory. If the user BASIC program is in RAM,  then 13H:14H =

512,  if the user program is the first program in EPROM,  then 13H:14H =

8011H.

6) The user must finally initialize the serial port.  Any scheme can  be

used (as long as it works!!)

The  added  reset options should go along way toward making MCS BASIC-52

configurable to any custom application.

166

CHAPTER 12

Command/Statement Extensions (Version 1.1 Only)

MCS BASIC-52 V1.1 provides a simple,  but yet effective way for the user

to  add  COMMANDS and/or STATEMENTS to the ones that are provided on the

chip.  All the user must do is write a few  simple  programs  that  will

reside in external code memory. The step by step approach is as follows:

STEP 1

The  user  must  first inform MCS BASIC-52 device that the expansion op-

tions are available.  This is done by putting the character 5AH in  CODE

memory  location  2002H.    When MCS BASIC-52 enters the command mode it

will examine CODE memory location 2048H.  The user  must  then  write  a

short  routine  to  SET BIT 45 (2DH),  which is bit 5 of internal memory

location 37 (decimal) and place this routine  at  code  memory  location

2048H.  Setting  BIT  45 tells MCS BASIC-52 that the expansion option is

available.  The following simple code will accomplish all that is stated

above:

ORG     2002H

DB      5AH

;

OG      2048H

SETB    45

RET

STEP 2

With  BIT  45 SET,  MCS BASIC-52 will CALL external CODE memory location

2078H everytime it attempts to tokenize a line that has been entered. At

location 2078H,  the user must load the DPTR (Data Pointer) with the ad-

dress of the user supplied lookup table, complete with tokens.

167

COMMAND/STATEMENT EXTENSIONS (VERSION 1.1 ONLY)

STEP 3

The user needs the following information to generate a user token table:

1) THE USER TOKENS ARE THE NUMBERS 10H THROUGH 1FH (16 TOKENS AVAILABLE)

2)  THE  USER  TOKEN TABLE BEGINS WITH THE TOKEN,  FOLLOWED BY THE ASCII

TEXT THAT IS TO BE REPRESENTED BY THAT TOKEN,  FOLLOWED BY A ZERO  (00H)

INDICATING THE END OF THE ASCII, FOLLOWED BY THE NEXT TOKEN.

3) THE TABLE IS TERMINATED WITH THE CHARACTER 0FFH.

EXAMPLE:

ORG      2078H

;

MOV      DPTR, #USER_TABLE

RET

;

ORG      2200H           ; THIS DOES NOT NEED TO BE

;

USER TABLE:

;

DB       10H             ; FIRST TOKEN

DB       'DISPLAY'       ; USER KEYWORD

DB       00H             ; KEYWORD TERMINATOR

;

DB       11H             ; SECOND TOKEN

DB       'TRANSFER'      ; SECOND USER KEYWORD

DB       00H             ; KEYWORD TERMINATOR

;

DB       12H             ; THIRD TOKEN (UP TO 16)

DB       'ROTATE'        ; THIRD USER KEYWORD

DB       0FFH            ; END OF USER TABLE

This  same  user  table  is used when MCS BASIC-52 "de-tokenizes" a line

during a LIST.

168

COMMAND/STATEMENT EXTENSIONS (VERSION 1.1 ONLY)

STEP 4

Step 3 tokenizes the user keyword,  this means that MCS BASIC-52  trans-

lates  the  user keyword into the user token.  So,  in the preceding ex-

ample,  the keyword TRANSFER would be replaced with the token 11H.  When

MCS  BASIC-52  attempts  to execute the user token,  it first makes sure

that the user expansion option BIT is set (BIT 45),  then CALLS location

2070H  to  get  the  address  of the user vector table.  This address is

placed in the DPTR.  The user vector table consist  of  series  of  Data

Words that define the address of the user assembly language routines.

EXAMPLE:

ORG      2070H           ; LOCATION BASIC CALLS TO

; GET USER LOOKUP

;

MOV      DPTR, #VECTOR_TABLE

RET

;

VECTOR_TABLE:

;

DW       RUN_DISPLAY     ; ADDRESS OF DISPLAY

; ROUTINE, TOKEN (10H)

DW       RUN_TRANSFER    ; ADDRESS OF TRANSFER

; ROUTINE, TOKEN (11H)

DW       RUN_ROTATE      ; ADDRESS OF ROTATE

; ROUTINE, TOKEN (12H)

;

ORG      2300H           ; AGAIN, THESE ROUTINES

; MAY BE PLACED ANYWHERE

;

RUN_DISPLAY:

;

; USER ASM CODE FOR DISPLAY GOES HERE

;

RUN_TRANSFER:

;

; USER ASM CODE FOR TRANSFER GOES HERE

RUN_ROTATE:

;

; USER ASM CODE FOR ROTATE GOES HERE

;

169

COMMAND/STATEMENT EXTENSIONS (VERSION 1.1 ONLY)

Note  that  the  ordinal  position  of the DATA WORDS in the user vector

table must correspond to the token, so the user statement with the token

10H must be the first DW entry in the vector  table,  11H,  the  second,

12H,  the third, and so on. The order of the tokens in the user table is

not important!!  The following user lookup table would function properly

with the previous example:

EXAMPLE:

;

USER_TABLE:

;

DB      13H             ; THE TOKENS DO NOT HAVE

DB      'ROTATE'        ; TO BE IN ORDER IN THE

DB      00H             ; USER LOOKUP TABLE

;

DB      10H

DB      'DISPLAY'

DB      00H

;

DB      12H

DB      'TRANSFER'

DB      0FFH            ; END OF TABLE

170

COMMAND/STATEMENT EXTENSIONS (VERSION 1.1 ONLY)

The  user  may  also  use  the command/statement extension option to re-

define the syntax of MCS BASIC-52.  This is done simply by placing  your

own  syntax in the user table and placing the appropriate BASIC token in

front of your re-defined keyword. A complete listing of all MCS BASIC-52

tokens and keywords are provided in the  back  of  this  chapter.    MCS

BASIC-52 will always list out the program using the user defined syntax,

but it will still accept the standard keyword as a valid instruction. As

an  example,  suppose that the user would like to substitute the keyword

HEXOUT for PH1., then the user would generate the following entry in the

user table:

EXAMPLE:

USER_TABLE:

;

DB       8FH             ; TOKEN FOR PH1.

DB       'HEXOUT'        ; TO BE IN ORDER IN THE

DB       00H             ; USER LOOKUP TABLE

;

DB       10H

DB       'DISPLAY'

DB       00H

;

;        REST OF USER_TABLE

;

DB       0FFH            ; END OF TABLE

MCS BASIC-52 will now accept the keyword HEXOUT and it will function  in

a manner identical to PH1.  PH1. will still function correctly, however,

HEXOUT will be displayed when the user LIST a program.

171

COMMAND/STATEMENT EXTENSIONS (VERSION 1.1 ONLY)

TOKEN   KEYWORD        TOKEN       KEYWORD    TOKEN       KEYWORD

80H     LET            0B0H        ABS        0ECH        <=

81H     CLEAR          0B1H        INT

82H     PUSH           0B2H        SGN

83H     GOTO           0B3H        NOT

84H     PWM            0B4H        COS

85H     PH0.           0B5H        TAN

86H     VI             0B6H        SIN

87H     VO             0B7H        SQR

88H     POP            0B8H        CBY

89H     PRINT          0B9H        EXP

89H     P.             0BAH        ATN

89H     ? (V1.1 ONLY)  0BBH        LOG

8AH     CALL           0BCH        DBY

8BH     DIM            0BDH        XBY

8CH     STRING         0BEH        PI

8DH     BAUD           0BFH        RND

8EH     CLOCK          0C0H        GET

8FH     PH1.           0C1H        FREE

90H     STOP           0C2H        LEN

91H     ONTIME         0C3H        XTAL

92H     ONEX1          0C4H        MTOP

93H     RETI           0C5H        TIME

94H     DO             0C6H        IE

95H     RESTORE        0C7H        IP

96H     REM            0C8H        TIMER0

97H     NEXT           0C9H        TIMER1

98H     ONERR          0CAH        TIMER2

99H     ON             0CBH        T2CON

9AH     INPUT          0CCH        TCON

9BH     READ           0CDH        TMOD

9CH     DATA           0CEH        RCAP2

9DH     RETURN         0CFH        PORT1

9EH     IF             0D0H        PCON

9FH     GOSUB          0D1H        ASC (

0A0H    FOR            0D2H        USING (

0A1H    WHILE          0D2H        U. (

0A2H    UNTIL          0D3H        CHR (

0A3H    END            0D4H-0DFH NOT USED

0A4H    TAB            0E0H        (

0A5H    THEN           0E1H        **

0A6H    TO             0E2H        *

0A7H    STEP           0E3H        +

0A8H    ELSE           0E4H        /

0A9H    SPC            0E5H        -

0AAH    CR             0E6H        .XOR.

172

0ABH    IDLE           0E7H        .AND.

0ACH    ST@ (V1.1 ONLY)0E8H        .OR.

0ADH    LD@ (V1.1 ONLY)0E9H        - (NEGATE)

0AEH    PGM (V1.1 ONLY)0EAH        =

0AFH    RROM(V1.1 ONLY)0EBH        >=

173

CHAPTER 13

Mapping User Code Memory

You might have noticed by now that some of external  CODE  memory  loca-

tions that MCS BASIC-52 calls and uses are located around 2000H and some

of the locations are located around 4000H. Specifically, they are as fol-

lows:

LOCATION   FUNCTION

2001H      ON RESET, MCS BASIC-52 LOOKS FOR A 0AAH IN THIS LOCA-

TION IF PRESENT, CALLS LOCATION 2090H

2002H      MCS BASIC-52 EXAMINES THIS LOCATION TO SEE IF THE USER

WANTS TO IMPLEMENT THE COMMAND/STATEMENT EXTENSION OP-

TION, A 05AH IS TO BE PLACED IN THIS LOCATION TO EVOKE

THE COMMAND/EXTENSION OPTION

2048H      MCS BASIC-52 CALLS THE LOCATION IF THE USER WANTS TO

IMPLEMENT THE COMMAND/STATEMENT EXTENSION OPTION. THE

USER WILL USUALLY SET BIT 45 THEN RETURN.

2070H      MCS BASIC-52 CALLS THIS LOCATION TO GET THE USE VECTOR

TABLE ADDRESS WHEN THE COMMAND/STATEMENT EXTENSION

OPTION IS EVOKED. THE ADDRESS OF THE VECTOR TABLE IS

PUT IN THE DPTR BY THE USER

2078H      MCS BASIC-52 CALLS THIS LOCATION TO GET THE USER LOOK-

UP TABLE ADDRESS WHEN THE COMMAND/STATEMENT EXTENSION

OPTION IS EVOKED. THE ADDRESS OF THE LOOKUP TABLE IS

PUT IN THE DPTR BY THE USER.

2090H      MCS BASIC-52 CALLS THIS LOCATION WHEN THE USER EVOKES

THE ASSEMBLY LANGUAGE RESET OPTION

4003H      EXTERNAL INTERRUPT 0

400BH      TIMER 0 INTERRUPT

4013H      EXTERNAL INTERRUPT 1

401BH      TIMER 0 INTERRUPT

4023H      SERIAL PORT INTERRUPT

402BH      TIMER 2 INTERRUPT

4030H      USER CONSOLE OUTPUT

4033H      USER CONSOLE INPUT

4036H      USER CONSOLE STATUS

174

403CH      USER PRINT@ OR LIST@ VECTOR

4100H-41FFH  USER CALLS FORM 0 TO 7FH

175

MAPPING USER CODE MEMORY

Others  vectors between 2040H and 2090H also exist,  but they are mainly

for testing purposes, but for your information they are:

LOCATION   FUNCTION

2040H      TRAP LOCATION FOR EXTERNAL INTERRUPT 0 IF BIT 26H OF

INTERNAL RAM IS SET AND THE DMA OPTION IS EVOKED. PSW

IS NOT PUSHED ONTO STACK. INTERRUPTS OF COURSE, MUST

BE ENABLED. ALSO, THIS LOCATION WILL BE CALLED FOR

CONSOLE OUTPUT IF BIT 2CH OF INTERNAL RAM IS SET.

2050H      TRAP LOCATION FOR SERIAL PORT INTERRUPT IF BIT 1FH

OF INTERNAL RAM IS SET. PSW IS PUSHED ONTO THE STACK.

2060H      CALLED FOR CONSOLE INPUT IF BIT 32H OF INTERNAL RAM IS

SET.

2068H      CALLED FOR CONSOLE STATUS CHECK IF BIT 32H OF INTERNAL

RAM IS SET.

2088H      TIMER 1 INTERRUPT TRAP IF BIT 1AH OF INTERNAL RAM IS

SET. PSW IS PUSHED ONTO THE STACK.

Contrary to popular belief,  these vectors were not chosen to force  the

user  to buy bigger EPROMS.  They are chosen so that addresses 2000H and

4000H can be overlayed and create no conflicts.  The Overlayed addresses

would appear as 2001H, 2002H, 4003H, 400BH, 4013H, 401BH, 4023H,  402BH,

4030H,  4033H,  4036H,  4039H, 2040H, 2048H, 2050H, 2060H, 2068H, 2070H,

2078H, 2088H, 2090H, and 4100H thru 41FFH.  The diagram on the next page

illustrates  how to implement overlapping addresses for 2000H and 4000H.

By using overlapping addresses,  the user can implement all MCS BASIC-52

user expansion options with only a few hundred bytes of EPROM.

The  reason this type of addressing scheme was chosen is that it permits

the designer to offer custom versions of MCS BASIC-52, by using the vec-

tor locations in the 2000H  region.  And  give  the  designers  OEM  the

ability  to  take  advantage  of  the  I/O  vectors located in the 4000H

region.

As an added note, the MCS-51 instruction set is object relocatable on 2K

boundaries if no LCALL or LJMP instructions are used. This means that it

is possible for the designer to ORG a program for 2000H and actually ex-

ecute the program at 2800H, 3000H, 3800H, etc.  If the user does not use

the LCALL or LJMP instructions.

176

APPENDIX A

1.1 MEMORY USAGE (Version 1.0)

The  following  list  specifies  what locations in internal and external

memory MCS BASIC-52 uses,  and what these locations are used  for.  This

information  can  largely  be regarded as "for your information," but it

can be used to do things like alter the pulse width of a EPROM  program-

ming pulse, etc.

INTERNAL MEMORY ALLOCATION:

LOCATION(S) IN HEX     MCS BASIC-52 USAGE

00H THRU 07H           "WORKING REGISTER BANK"

08H                    BASIC TEXT POINTER - LOW BYTE

09H                    ARGUMENT STACK POINTER

0AH                    BASIC TEXT POINTER - HIGH BYTE

0BH THRU 0FH           TEMPORARY BASIC STORAGE

10H                    READ TEXT POINTER - LOW BYTE

11H                    CONTROL STACK POINTER

12H                    READ TEXT POINTER - HIGH BYTE

13H                    START ADDRESS OF BASIC PROGRAM - HIGH BYTE

14H                    START ADDRESS OF BASIC PROGRAM - LOW BYTE

15H                    NULL COUNT

16H                    PRINT HEAD POSITION FOR OUTPUT

17H                    FLOATING POINT OUTPUT FORMAT TYPE

18H THRU 21H           NOT USED - RESERVED FOR USER

22H                    BITS USED SPECIFICALLY AS FOLLOWS

BIT 22.0H              SET WHEN "ONTIME" STATEMENT IS EXECUTED

BIT 22.1H              SET WHEN BASIC INTERRUPT IN PROGRESS

BIT 22.2H              SET WHEN "ONEX" STATEMENT IS EXECUTED

177

BIT 22.3H              SET WHEN "ONERR" STATEMENT IS EXECUTED

BIT 22.4H              SET WHEN "ONTIME" INTERRUPT IS IN PROGRESS

BIT 22.5H              SET WHEN A LINE IS EDITED

BIT 22.6H              SET WHEN EXTERNAL INTERRUPT IS PENDING

BIT 22.7H              WHEN SET, CONT COMMAND WILL WORK

23H                    BITS USED SPECIFICALLY AS FOLLOWS

BIT 23.0H              USED AS FLAG FOR "GET" OPERATOR

BIT 23.1H              SET WHEN INVALID INTEGER FOUND IN TEXT

BIT 23.2H              TEMPORARY BIT LOCATION

BIT 23.3H              CONSOLE OUTPUT CONTROL, 1 = LINE PRINTER

BIT 23.4H              CONSOLE OUTPUT CONTROL, 1 = USER DEFINED

BIT 23.5H              BASIC ARRAY INITIALIZATION BIT

BIT 23.6H              CONSOLE INPUT CONTROL, 1 = USER DEFINED

BIT 23.7H              RESERVED

178

1.1 MEMORY USAGE

INTERNAL MEMORY ALLOCATION:

LOCATION(S) IN HEX     MCS BASIC-52 USAGE

24H                    BITS USED SPECIFICALLY AS FOLLOWS

BIT 24.0H              STOP STATEMENT OR CONTROL-C ENCOUNTERED

BIT 24.1H              0 = HEX INPUT, 1 = FP INPUT

BIT 24.2H              0 = RAM MODE, 1 = ROM MODE

BIT 24.3H              ZERO FLAG FOR DOUBLE BYTE COMPARE

BIT 24.4H              SET WHEN ARGUMENT STACK HAS A VALUE

BIT 24.5H              RETI INSTRUCTION EXECUTED

BIT 24.6H              RESERVED

BIT 24.7               RESERVED

25H                    BITS USED SPECIFICALLY AS FOLLOWS

BIT 25.0H              RESERVED, SOFTWARE TRAP TEST

BIT 25.1H              FIND THE END OF PROGRAM , IF SET

BIT 25.2H              RESERVED

BIT 25.3H              INTERRUPT STATUS SAVE BIT

BIT 25.4H              SET WHEN PROGRAM EXECUTION IS COMPLETE

BIT 25.5H              RESERVED, EXTERNAL TRAP TEST

BIT 25.6H              SET WHEN CLOCK 1 EXECUTED, ELSE CLEARED

BIT 25.7H              SET WHEN BASIC IS IN THE COMMAND MODE

26H                    BITS USED SPECIFICALLY AS FOLLOWS

BIT 26.0H              SET TO DISABLE CONTROL-C

BIT 26.1H              SET TO ENABLE "FAKE" DMA

179

BIT 26.2H              RESERVED

BIT 26.3H             SET TO EVOKED "INTELLIGENT" FROM PROGRAMMING

BIT 26.4H              SET TO PRINT TEXT STRING FROM ROM

BIT 26.5H              RESERVED

BIT 26.6H              SET TO SUPPRESS ZEROS IN HEX MODE PRINT

BIT 26.7H              SET TO EVOKE HEX MODE PRINT

180

1.1 MEMORY USAGE

INTERNAL MEMORY ALLOCATION:

LOCATION(S) IN HEX     MCS BASIC-52 USAGE

27H                    "BIT" ADDRESSABLE BYTE COUNTER

28H THRU 3DH            BIT AND BYTE FLOATING POINT WORKING SPACE

3EH                     INTERNAL STACK POINTER HOLDING REGISTER

3FH                     LENGTH OF USER DEFINED STRING - $

40H                     TIMER 1 RELOAD LOCATION - HIGH BYTE

41H                     TIMER 1 RELOAD LOCATION - LOW BYTE

42H                     BASIC TEXT POINTER SAVE ALLOCATION - HIGH

BYTE

43H                     BASIC TEXT POINTER SAVE LOCATION - LOW

BYTE

44H                     RESERVED

45H                     TRANSCENDENTAL FUNCTION TEMP STORAGE

46H                     TRANSCENDENTAL FUNCTION TEMP STORAGE

47H                     MILLI-SECOND COUNTER FOR REAL TIME CLOCK

48H                     SECOND COUNTER FOR REAL TIME CLOCK - HIGH

BYTE

49H                     SECOND COUNTER FOR REAL TIME CLOCK - LOW

LOW BYTE

4AH                     TIMER 0 RELOAD FOR REAL TIME CLOCK

4BH                     SOFTWARE SERIAL PORT BAUD RATE - HIGH

BYTE

4CH                     SOFTWARE SERIAL PORT BAUD RATE - LOW

BYTE

4DH THRU 0FFH           8052AH STACK SPACE AND USER WORKING SPACE

181

1.1 MEMORY USAGE

EXTERNAL MEMORY ALLOCATION

LOCATION(S) IN HEX      MCS BASIC-52 USAGE

00H AND 01H             "LAST" END OF FILE ADDRESS FOR RAM FILE

(H-L)

02H AND 03H             CURRENT END OR FILE ADDRESS FOR RAM FILE

(H-L)

04H                     LENGTH OF THE CURRENT EDITED LINE

05H AND 06H             LN NUM IN BINARY OF CURRENT EDITED LINE

(H-L)

07H THRU 49H            BASIC INPUT BUFFER

50H THRU 5FH            FLOATING POINT OUTPUT TEMP

60H THRU 0FEH           CONTROL STACK

0FFH                    CONTROL STACK OVERFLOW

100H                    LOCATION TO SAVE "GET" CHARACTER

101H                    LOCATION TO SAVE ERROR CHARACTER CODE

102H AND 103H           LOCATION TO GO TO ON USER "ONERR" (H-L)

104H AND 105H           TOP OF VARIABLE STORAGE (H-L)

106H AND 107H           FP STORAGE ALLOCATION (H-L)

108H AND 109H           MEMORY ALLOCATED FOR MATRICES (H-L)

10AH AND 10BH           TOP OF MEMORY ASSIGNED TO BASIC (H-L)

10CH AND 10DH           RANDOM NUMBER SEED (H-L)

10EH THRU 113H          CRYSTAL VALUE

114H THRU 11FH          FLOATING POINT TEMPS

120H AND 121H           LOCATION TO GO TO ON ONEX1 INTERRUPT

(H-L)

122H AND 123H           NUMBER OF BYTES ALLOCATED FOR STRINGS

(H-L)

182

124H THRU 127H          ONTIME INTERRUPT AND LINE NUMBER (H-L)

128H AND 129H           "NORMAL" PROM PROGRAMMER TIME OUT (H-L)

12AH AND 12BH           "INTELLIGENT" PROM PROGRAMMER TIME OUT

(H-L)

12CH                    RESERVED

12DH THRU 1FEH          ARGUMENT STACK

NOTE:  (H-L)  means HIGH BYTE - LOW BYTE,  in external memory all 16 bit

binary numbers are stored with the HIGH BYTE in the  first  (low  order)

address and the LOW BYTE in the next sequential address.

183

1.1 MEMORY USAGE (VERSION 1.1)

The  following  list  specifies  what locations in internal and external

memory locations are used by Version 1.1 MCS BASIC-52.  Any  differences

between V1.0 and V1.1 are in bold face type.

INTERNAL MEMORY ALLOCATION:(VERSION 1.1)

LOCATION(S) IN HEX      MCS BASIC-52 USAGE

00H THRU 07H            "WORKING REGISTER BANK"

08H                     BASIC TEXT POINTER - LOW BYTE

09H                     ARGUMENT STACK POINTER

0AH                     BASIC TEXT POINTER - HIGH BYTE

0BH THRU 0FH            TEMPORARY BASIC STORAGE (Available to

user in BASIC CALLS to ASM routines)

10H                     READ TEXT POINTER - LOW BYTE

11H                     CONTROL STACK POINTER

12H                     READ TEXT POINTER - HIGH BYTE

13H                     START ADDRESS OF BASIC PROGRAM - HIGH

BYTE

14H                     START ADDRESS OF BASIC PROGRAM - LOW

BYTE

15H                     NULL COUNT

16H                     PRINT HEAD POSITION FOR OUTPUT

17H                     FLOATING POINT OUTPUT FORMAT TYPE

18H THRU 21H            NOT USED - RESERVED FOR USER

22H                     BITS USED SPECIFICALLY AS FOLLOWS

BIT 22.0H               SET WHEN "ONTIME" STATEMENT IS EXECUTED

BIT 22.1H               SET WHEN BASIC INTERRUPT IN PROGRESS

BIT 22.2H               SET WHEN "ONEX1" STATEMENT IS EXECUTED

BIT 22.3H               SET WHEN "ONERR" STATEMENT IS EXECUTED

184

BIT 22.4H               SET WHEN "ONTIME" INTERRUPT IS IN

PROGRESS

BIT 22.5H               SET WHEN A LINE IS EDITED

BIT 22.6H               SET WHEN EXTERNAL INTERRUPT IS PENDING

BIT 22.7H               WHEN SET, CONT COMMAND WILL WORK

23H                     BITS USED SPECIFICALLY AS FOLLOWS

BIT 23.0H               USED AS FLAG FOR "GET" OPERATOR

BIT 23.1H               SET WHEN PRINT@ OR LIST@ IS EVOKED

BIT 23.2H               RESERVED, TRAPS TIMER 1 INTERRUPT

BIT 23.3H               CONSOLE OUTPUT CONTROL, 1 = LINE PRINTER

BIT 23.4H               CONSOLE OUTPUT CONTROL, 1 = USER DEFINED

BIT 23.5H               BASIC ARRAY INITIALIZATION BIT

BIT 23.6H               CONSOLE INPUT CONTROL, 1 = USER DEFINED

BIT 23.7H               RESERVED, USED TO TRAP SERIAL PORT

INTERRUPT

185

INTERNAL MEMORY ALLOCATION (VERSION 1.1)

LOCATION(S) IN HEX      MCS BASIC-52 USAGE

24H                     BITS USED SPECIFICALLY AS FOLLOWS

BIT 24.0H               STOP STATEMENT OR CONTROL-C ENCOUNTERED

BIT 24.1H               USER IDLE BREAK BIT

BIT 24.2H               SET DURING AN INPUT INSTRUCTION

BIT 24.3H               RESERVED

BIT 24.4H               SET WHEN ARGUMENT STACK HAS A VALUE

BIT 24.5H               RETI INSTRUCTION EXECUTED

BIT 24.6H               RESERVED, TRAPS EXTERNAL INTERRUPT 0

BIT 24.7H               SET BY USER TO SIGNIFY THAT A VALID LIST@

OR PRINT@ DRIVER IS PRESENT

25H                     BITS USED SPECIFICALLY AS FOLLOWS

BIT 25.0H               RESERVED, SOFTWARE TRAP TEST

BIT 25.2H               SET DURING A DIM STATEMENT

BIT 25.3H               INTERRUPT STATUS SAVE BIT

BIT 25.4H               RESERVED, INPUT TRAP

BIT 25.5H               SET TO SIGNIFY EXPANSION IS PRESENT

BIT 25.6H               SET WHEN CLOCK1 EXECUTED, ELSE CLEARED

BIT 25.7H               SET WHEN BASIC IN IN THE COMMAND MODE

26H                     BITS USED SPECIFICALLY AS FOLLOWS

BIT 26.0H               SET TO DISABLE CONTROL-C

BIT 26.1H               SET TO ENABLE "FAKE" DMA

BIT 26.2H               RESERVED, OUTPUT TRAP

BIT 26.3H               SET TO EVOKE "INTELLIGENT" PROM

PROGRAMMING

186

BIT 26.4H               SET TO PRINT TEXT STRING FROM ROM

BIT 26.5H               SET WHEN CONTROL-S ENCOUNTERED

BIT 26.6H               SET TO SUPPRESS ZEROS IN HEX MODE PRINT

BIT 26.7H               SET EVOKE HEX MODE PRINT

187

INTERNAL MEMORY ALLOCATION (VERSION 1.1)

LOCATION(S) IN HEX      MCS BASIC-52 USAGE

27H                     "BIT" ADDRESSABLE BYTE COUNTER

28H THRU 3DH            BIT AND BYTE FLOATING POINT WORKING SPACE

3EH                     INTERNAL STACK POINTER HOLDING REGISTER

3FH                     LENGTH OF USER DEFINED STRING - $

40H                     TIMER 1 RELOAD LOCATION - HIGH BYTE

41H                     TIMER 1 RELOAD LOCATION - LOW BYTE

42H                     BASIC TEXT POINTER SAVE LOCATION - HIGH

BYTE

43H                     BASIC TEXT POINTER SAVE LOCATION - LOW

BYTE

44H                     RESERVED

45H                     TRANSCENDENTAL FUNCTION TEMP STORAGE

46H                     TRANSCENDENTAL FUNCTION TEMP STORAGE

47H                     MILLI-SECOND COUNTER FOR REAL TIME CLOCK

48H                     SECOND COUNTER FOR REAL TIME CLOCK - HIGH

BYTE

49H                     SECOND COUNTER FOR REAL TIME CLOCK - LOW

BYTE

4AH                     TIMER 0 RELOAD FOR REAL TIME CLOCK

4BH                     USER ARGUMENT FOR ONTIME - HIGH BYTE

4CH                     USER ARGUMENT FOR ONTIME - LOW BYTE

4DH THRU 0FFH           8052AH STACK SPACE AND USER WORKING SPACE

188

EXTERNAL MEMORY ALLOCATION (VERSION 1.1)

LOCATION(S) IN HEX      MCS BASIC-52 USAGE

00H THRU 03H            NOT USED, RESERVED

04H                     LENGTH OF THE CURRENT EDITED LINE

05H AND 06H             LN NUM IN BINARY OF CURRENT EDITED LINE

(H-L)

07H THRU 56H            BASIC INPUT BUFFER

56H THRU 5DH            BINARY TO INTEGER TEMP

5EH                     USED FOR RUN TRAP MODE (=34H)

5FH                     USED FOR POWER-UP TRAP (=0A5H)

60H THRU 0FEH           CONTROL STACK

00FH                    CONTROL STACK OVERFLOW

100H                    LOCATION TO SAVE "GET" CHARACTER

101H                    LOCATION TO SAVE ERROR CHARACTER CODE

102H AND 103H           LOCATION TO GO TO ON USER "ONERR" (H-L)

104H AND 105H           TOP OF VARIABLE STORAGE (H-L)

106H AND 107H           FP STORAGE ALLOCATION (H-L)

108H AND 109H           MEMORY ALLOCATED FOR MATRICES (H-L)

10AH AND 10BH           TOP OF MEMORY ASSIGNED TO BASIC (H-L)

10CH AND 10DH           RANDOM NUMBER SEED (H-L)

10EH THRU 113H          CRYSTAL VALUE

114 THRU 11FH           FLOATING POINT TEMPS

120H THRU 121H          LOCATION TO GO TO ON ONEX1 INTERRUPT

(H-L)

122H AND 123H           NUMBER OF BYTES ALLOCATED FOR STRINGS

(H-L)

124H AND 125H           SOFTWARE SERIAL PORT BAUD RATE (H-L)

189

126H AND 127H           LINE NUMBER FOR ONTIME INTERRUPT (H-L)

128H AND 129H           "NORMAL" PROM PROGRAMMER TIME OUT (H-L)

12AH AND 12BH           "INTELLIGENT" PROM PROGRAMMER TIME OUT

(H-L)

12CH                    RESERVED

12DH THRU 1FEH          ARGUMENT STACK

NOTE: (H-L) still means HIGH BYTE - LOW BYTE,  in external memory all 16

bit binary numbers are stored with the HIGH BYTE in the first (lower or-

der) address and the LOW BYTE in the next sequential address.

190

1.2 USING THE PWM STATEMENT

The PWM statement can be used to generate  quite  accurate  frequencies.

The  following  table lists the reload values 8 octaves of an equal tem-

pered chromatic scale. The reload values are for the first two arguments

of the PWM statement, so it is assumed that a square wave is being gener-

ated. The reload values assume a 11.0592 MHz crystal.

NOTE   OCTAVE     IDEAL         ACTUAL                 HEX

FREQUENCY      FREQUENCY    RELOAD    RELOAD

C       1       32.703         32.704       14090     370AH

C#      1       34.648         34.649       13299     33F3H

D       1       36.708         36.708       12553     3109H

D#      1       38.891         38.889       11849     2E49H

E       1       41.203         41.202       11184     2BB0H

F       1       43.654         43.653       10556     293CH

F#      1       46.246         46.215        9963     26EBH

G       1       48.999         49.000        9404     24BCH

G#      1       51.913         51.915        8876     22ACH

A       1       55.000         55.001        8378     20BAH

A#      1       58.270         58.270        7908     1EE4H

B       1       61.735         61.736        7464     1D28H

C       2       65.406         65.408        7045     1B85H

C#      2       69.296         69.293        6650     19FAH

D       2       73.416         73.411        6277     1885H

D#      2       77.782         77.785        5924     1724H

E       2       82.406         82.403        5592     15D8H

F       2       87.308         87.306        5278     149EH

F#      2       92.498         92.493        4982     1376H

G       2       97.998         98.000        4702     125EH

191

G#      2       103.826       103.830        4438     1156H

A       2       110.000       110.002        4189     105DH

A#      2       116.540       116.540        3954     0F72H

B       2       123.470       123.472        3732     0E94H

C       3       130.812       130.798        3523     0DC3H

C#      3       138.592       138.586        3325     0CFDH

D       3       146.832       146.845        3138     0C42H

D#      3       155.564       155.570        2962     0B92H

E       3       164.812       164.807        2796     0AECH

F       3       174.616       174.612        2639     0A4FH

F#      3       184.996       184.986        2491     09BBH

G       3       195.996       196.001        2351     092FH

G#      3       207.652       207.661        2219     08ABH

A       3       220.000       219.952        2095     082FH

A#      3       233.080       233.080        1977     07B9H

B       3       246.940       246.946        1866     074AH

192

1.2 USING THE PWM STATEMENT

NOTE    OCTAVE    IDEAL       ACTUAL        RELOAD      HEX

FREQUENCY    FREQUENCY                 RELOAD

C       4       261.624       261.669       1761      06E1H

C#      4       277.184       277.256       1662      067EH

D       4       293.664       293.690       1569      0621H

D#      4       311.128       311.141       1481      05C9H

E       4       329.624       329.614       1398      0576H

F       4       349.232       349.355       1319      0527H

F#      4       369.992       370.120       1245      04DDH

G       4       391.992       391.836       1176      0498H

G#      4       415.304       415.135       1110      0456H

A       4       440.000       440.114       1047      0417H

A#      4       466.160       465.925        989      03DDH

B       4       493.880       493.890        933      03A5H

C       5       523.248       523.042        881      0371H

C#      5       554.368       554.512        831      033FH

D       5       587.238       587.006        785      0311H

D#      5       622.256       621.862        741      02E5H

E       5       659.248       659.228        699      02BBH

F       5       698.464       698.182        660      0294H

F#      5       739.984       739.647        623      026FH

G       5       783.984       783.674        588      024CH

G#      5       830.608       830.270        555      022BH

A       5       880.000       879.389        524      020CH

A#      5       932.320       932.793        494      01EEH

193

B       5       987.760       986.724        467      01D3H

C       6      1046.496      1047.272        440      01B8H

C#      6      1108.736      1107.692        416      01A0H

D       6      1174.656      1175.510        392      0188H

D#      6      1244.512      1245.405        370      0172H

E       6      1318.496      1320.343        349      015DH

F       6      1396.928      1396.364        330      014AH

F#      6      1479.968      1481.672        311      0137H

G       6      1567.968      1567.347        294      0126H

G#      6      1661.216      1663.538        277      0115H

A       6      1760.000      1758.779        262      0106H

A#      6      1864.640      1865.587        247      00F7H

B       6      1975.520      1977.682        233      00E9H

194

1.2 USING THE PWM STATEMENT

NOTE      OCTAVE    IDEAL      ACTUAL                 HEX

FREQUENCY      FREQUENCY        RELOAD          RELOAD

C          7      2092.992     2094.545     220      00DCH

C#         7      2217.472     2215.385     208      00D0H

D          7      2349.312     2351.020     196      00C4H

D#         7      2489.024     2490.811     185      00B9H

E          7      2636.992     2633.143     175      00AFH

F          7      2793.856     2792.727     165      00A5H

F#         7      2959.936     2953.846     156      009CH

G          7      3135.936     3134.694     147      0093H

G#         7      3322.432     3315.108     139      008BH

A          7      3520.000     3517.557     131      0083H

A#         7      3729.280     3716.129     124      007CH

B          7      3951.040     3938.362     117      0075H

C          8      4185.984     4189.091     110      006EH

C#         8      4434.944     4430.770     104      0068H

D          8      4698.624     4702.041      98      0062H

D#         8      4987.048     5008.695      92      005CH

E          8      5273.984     5296.552      87      0057H

F          8      5587.712     5619.512      82      0052H

F#         8      5919.872     5907.692      78      004EH

G          8      6217.872     6227.027      74      004AH

G#         8      6644.864     6678.261      69      0045H

A          8      7040.000     7089.231      65      0041H

195

A#         8      7458.560     7432.258      62      003EH

B          8      7902.080     7944.827      58      003AH

196

1.2 USING THE PWM STATEMENT

The  following  program  generates the appropriate reload values for the

PWM statement,  using any crystal.   The user enters the desired    fre-

quency  and the crystal and the program determined the reload values and

errors.

>10  INPUT "ENTER CRYSTAL FREQUENCY - ", X

>20  T=12/X

>30  INPUT "ENTER DESIRED FREQUENCY FOR PWM - ",F

>40  F1=1/F

>50  C=(F1/T)/2 : REM CALCULATE RELOAD VALUE

>60  IF C<20 THEN 30

>70  C1=C-INT(C) : REM CALCULATE FRACTION

>80  IF C1<.5 THEN 90 : C=C+1

>90  PRINT : PRINT "THE DESIRED FREQUENCY IS - ",X, "HZ"

>100 C=INT (C) : PRINT

>110 PRINT "THE ACTUAL FREQUENCY IS - ", 1/(2*C*T), "HZ"

>120 PRINT

>130 PRINT "THE RELOAD VALUE FOR PWM IS - ",C," IN HEX - ",:

PH1.C

>140 INPUT "ANOTHER FREQUENCY, 1=YES, 0=NO - ",Q

>150 IF Q=1 THEN 20

197

1.3 BAUD RATES AND CRYSTALS

The 16 bit auto-reload timer/counter (TIMER2) that is used  to  generate

baud rates for the MCS BASIC-52 device is capable of generating accurate

baud  rates with a number of crystals.  The following is a list of crys-

tals that will accurately generate 9600 baud on the MCS BASIC-52 device.

Additionally, the crystal values on the left hand side of the table will

accurately generate 19200 baud.

XTAL         RCAP2 RELOAD       XTAL       RCAP2 RELOAD

3680400          65524           3993600        65523

4300800          65522           4608000        65521

4915200          65520           5222400        65519

5529600          65518           5836800        65517

6144000          65516           6451200        65515

6758400          65514           7065600        65513

7372800          65512           7680000        65511

7987200          65510           8294400        65509

8601600          65508           8908800        65507

9216000          65506           9523200        65505

9830400          65504          10137600        65503

10444800          65502          10752000        65501

11059200          65500          11366400        65499

11673600          65498          11980800        65497

With the crystal listed above,  the accuracy of the baud rate  generator

and the REAL TIME CLOCK will depend ONLY on the absolute accuracy of the

crystal.    Note  that  the baud rate generator for the 8052AH is so ac-

curate that any crystal above 10MHz will generate 9600  baud  to  within

1.5 accuracy.

198

1.3 BAUD RATES AND CRYSTAL

The following program generates the appropriate TIMER2 reload values for

a given baud rate.  The user supplies the system clock frequency and the

desired baud rate and the program calculates the  proper  TIMER2  reload

value.   Additionally,  percent error,  for both the baud rate generator

and MCS BASIC-52's REAL TIME CLOCK are calculated and displayed.

>10 INPUT"ENTER CRYSTAL - ", X

>20 INPUT"ENTER BAUD RATE - ",B

>30  R=X/(32*B) : T=X/76800

>40  R1=R-INT(R): T1=T-INT(T)

>50  IF R1<.5 THEN 80

>60  R1=1-R1

>70  R=R+1

>80  IF T1<.5 THEN 110

>90  T1=1-T1

>100 T=T+1

>110 PRINT "TIMER2 RELOAD VALUE IS - ",USING(######),INT

(65536-R)

>120 PRINT "BAUD RATE ERROR IS - ",USING(##.###), (R1/R)

*100, "%"

>130 PRINT "REAL TIME CLOCK ERROR IS - "(T1/T)*100, "%"

199

1.4 QUICK REFERENCE

COMMANDS:

COMMAND    FUNCTION                            EXAMPLE(S)

RUN        Execute a program                      RUN

CONT       CONTinue after a STOP or control-C     CONT

LIST       LIST program to the console device     LIST

LIST 10-50

LIST#      LIST program to serial printer         LIST#

LIST# 50

LIST@      LIST program to user driver            LIST@

(version 1.1 only)                     LIST@ 50

NEW        erase the program stored in RAM        NEW

NULL       set NULL count after carriage return-  NULL

line feed                              NULL 4

RAM        evoke RAM mode, current program in     RAM

READ/WRITE memory

ROM        evoke ROM mode, current program in     ROM

ROM/EPROM memory                       ROM 3

XFER       transfer a program from ROM/EPROM to   XFER

RAM

PROG       save the current program in EPROM      PROG

PROG1      save baud rate information in EPROM    PROG1

PROG2      save baud rate information in EPROM    PROG2

and execute program after RESET

PROG3      save baud rate and MTOP information in PROG3

EPROM (version 1.1 only)

PROG4      save baud rate and MTOP information in PROG4

EPROM and execute program after

RESET (version 1.1 only)

200

1.4 QUICK REFERENCE

COMMANDS:   FUNCTION                               EXAMPLE(S)

COMMAND

PROG5       same as PROG4 except that external       PROG5

RAM is not cleared on RESET or power

up if external RAM contains a 0A5H in

location 5EH (version 1.1 only)

PROG6       same as PROG6 except that external       PROG6

code location 4039H is CALLED after

RESET (version 1.1 only)

FPROG       save the current program in EPROM        FPROG

using the INTELligent algorithm

FPROG1      save baud rate information in EPROM      FPROG1

using the INTELligent algorithm

FPROG2      save baud rate information in EPROM      FPROG2

and execute program after RESET, use

INTELligent algorithm

FPROG3      same as PROG3, except INTELligent        FPROG3

programming algorithm is used

(version 1.1 only)

FPROG4      same as PROG4, except INTELligent        FPROG4

programming algorithm is used

(version 1.1 only)

FPROG5      same as PROG5, except INTELligent        FPROG5

programming algorithm is used

(version 1.1 only)

FPROG6      same as PROG6, except INTELligent        FPROG6

programming algorithm is used

(version 1.1 only)

201

1.4 QUICK REFERENCE

STATEMENTS:

STATEMENT   FUNCTION                                 EXAMPLE(S)

BAUD        set baud rate for line printer port      BAUD 1200

CALL        CALL assembly language program           CALL 9000H

CLEAR       CLEAR variables, interrupts and Strings  CLEAR

CLEARS      CLEAR Stacks                             CLEARS

CLEAR1      CLEAR Interrupts                         CLEAR1

CLOCK1      enable REAL TIME CLOCK                   CLOCK1

CLOCK0      disable REAL TIME CLOCK                  CLOCK0

DATA        DATA to be read by READ statement        DATA 100

READ        READ data in DATA statement              READ A

RESTORE     RESTORE READ pointer                     RESTORE

DIM         allocate memory for arrayed variables    DIM A(20)

DO          set up loop for WHILE or UNTIL           DO

UNTIL       test DO loop condition (loop if false)   UNTIL A=10

WHILE       test DO loop condition (loop if true)    WHILE A=B

END         terminate program execution              END

FOR-TO-{STEP}set up FOR NEXT loop                    FOR A=1 TO 5

NEXT        test FOR-NEXT loop condition             NEXT A

202

1.4 QUICK REFERENCE

STATEMENTS:

STATEMENT   FUNCTION                                 EXAMPLE(S)

GOSUB       execute subroutine                       GOSUB 1000

RETURN      RETURN from sub-routine                  RETURN

GOTO        GOTO program line number                 GOTO 500

ON GOTO     conditional GOTO                         ON A GOTO 5,

20

ON GOSUB    conditional GOSUB                       ON A GOSUB 2,

6

IF-THEN-    conditional test                         IF A<B THEN

{ELSE}                                               A=0

INPUT       INPUT a string or variable               INPUT A

LET         assign a variable or string a value      LET A=10

(LET is optional)

ONERR       ONERRor GOTO line number                 ONERR 1000

ONTIME      generate an interrupt when TIME is       ONTIME 10,

equal to or greater than ONTIME          1000

argument-line number is after comma

ONEX1       GOSUB to line number following ONEX1     ONEX1 1000

when INT1 pin is pulled low

PRINT       PRINT variables, strings or literals     PRINT A

P. is shorthand for PRINT

PRINT#      PRINT to software serial port            PRINT# A

PH0.        PRINT HEX mode with zero suppression     PH0.A

PH1.        PRINT HEX mode with no zero              PH1.A

suppression

PH0.#       PH0. to line printer                     PH0.# A

PH1.#       PH1.# to line printer                    PH1.# A

203

1.4 QUICK REFERENCE

STATEMENTS:

STATEMENT   FUNCTION                                 EXAMPLE(S)

PRINT@      PRINT to user defined driver             PRINT@ 5*5

(version 1.1 only)

PH0.@       PH0. to user defined driver              PH0. @

(version 1.1 only)                       XBY(5EH)

PH1.@       PH1. to user defined driver              PH1.@ A

(version 1.1 only)

PGM         Program an EPROM (version 1.1 only)      PGM

PUSH        PUSH expressions on argument stack       PUSH 10, A

POP         POP argument stack to variables          POP A, B, C

PWM         PULSE WIDTH MODULATION                PWM 50, 50, 100

REM         REMark                                   REM DONE

RETI        RETurn from Interrupt                    RETI

STOP        break program execution                  STOP

STRING      allocate memory for STRINGs             STRING 50, 10

UI1         evoke User console Input routine         UI1

UI0         evoke BASIC console console Input        UI0

routine

UO1         evoke User console Output routine        UO1

UO0         evoke BASIC console Output routine       UO0

ST@         store top of stack at user specified     ST@ 1000H

location (version 1.1 only)              ST@ A

LD@         load top of stack from user specified    LD@ 1000H

location (version 1.1 only)              LD@ A

IDLE        wait for interrupt (version 1.1 only)    IDLE

RROM        run a program in EP(ROM) (version 1.1    RROM 3

only)

204

1.4 QUICK REFERENCE

OPERATORS - DUAL OPERAND:

OPERATOR    FUNCTION                                 EXAMPLE(S)

+           ADDITION                                 1+1

/           DIVISION                                 10/2

**          EXPONENTIATION                           2**4

*           MULTIPLICATION                           4*4

-           SUBTRACTION                              8-4

.AND.       LOGICAL AND                              10.AND.5

.OR.        LOGICAL OR                               2.OR.1

.XOR.       LOGICAL EXCLUSIVE OR                     3.XOR.2

OPERATORS - SINGLE OPERAND:

ABS()       ABSOLUTE VALUE                           ABS( -3)

NOT()       ONES COMPLEMENT                          NOT(0)

INT()       INTEGER                                  INT(3.2)

SGN         SIGN                                     SGN( -5)

SQR()       SQUARE ROOT                              SQR(100)

RND         RANDOM NUMBER                            RND

LOG()       NATURAL LOG                              LOG(10)

EXP()       "e" (2.7182818) TO THE X                 EXP(10)

SIN()       RETURNS THE SINE OF ARGUMENT             SIN(3.14)

COS()       RETURNS THE COSINE OF                    COS(0)

ARGUMENT

TAN()       RETURNS THE TANGENT OF                   TAN(.707)

ARGUMENT

205

ATN()       RETURNS ARCTANGENT OF                    ATN(1)

ARGUMENT

206

1.4 QUICK REFERENCE

OPERATORS - SPECIAL FUNCTION:

CBY()       READ PROGRAM MEMORY                      P.CBY(4000)

DBY()       READ/ASSIGN INTERNAL DATA                DBY(99)=10

MEMORY

XBY()       READ/ASSIGN EXTERNAL DATA                P.XBY(10)

MEMORY

GET         READ CONSOLE                             P.GET

IE          READ/ASSIGN IE REGISTER                  IE=82H

IP          READ/ASSIGN IP REGISTER                  IP=0

PORT1       READ/ASSIGN I/O PORT 1(P1)               PORT1=0FFH

PCON        READ/ASSIGN PCON REGISTER                PCON=0

RCAP2       READ/ASSIGN RCAP2                        RCAP2=100

(RCAP2H:RCAP2L)

T2CON       READ/ASSIGN T2CON REGISTER               P.T2CON

TCON        READ/ASSIGN TCON REGISTER                TCON=10H

TMOD        READ/ASSIGN TMOD REGISTER                P.TMOD

TIME        READ/ASSIGN THE REAL TIME CLOCK          P.TIME

TIMER0      READ/ASSIGN TIMER0 (TH0:TL0)             TIMER0=0

TIMER1      READ/ASSIGN TIMER1 (TH1:TL1)             P.TIMER1

TIMER2      READ/ASSIGN TIMER2 (TH2:TL2)             TIMER2=0FFH

STORED CONSTANT:

PI          PI - 3.1415926

207

1.5 INSTRUCTION SET SUMMARY

COMMANDS                STATEMENTS            OPERATORS

RUN                     BAUD                  ADD(+)

CONT                    CALL                  DIVIDE(/)

LIST                    CLEAR                 EXPONENTIATION (**)

LIST#                   CLEAR(S&I)            MULTIPLY (*)

LIS@ (V1.1)             CLOCK(1&0)            SUBTRACT (-)

NEW                     DATA                  LOGICAL AND (.AND.)

NULL                    READ                  LOGICAL OR (.OR.)

RAM                     RESTORE              LOGICAL X-OR (.XOR.)

ROM                     DIM                   LOGICAL NOT (.OR.)

XFER                    DO-WHILE              ABS()

PROG                    DO-UNTIL              INT()

PROG1                   END                   SGN()

PROG2                   FOR-TO-STEP           SQR()

PROG3 (V1.1)            NEXT                  RND

PROG4 (V1.1)            GOSUB                 LOG()

PROG5 (V1.1)            RETURN                EXP()

PROG6 (V1.1)            GOTO                  SIN()

FPROG                   ON-GOTO               COS()

FPROG1                  ON-GOSUB              TAN()

FPROG2                  IF-THEN-ELSE          ATN()

FPROG3 (V1.1)           INPUT                 =,>,>=,<,<=,<>

FPROG4 (V1.1)           LET                   ASC()

FPROG5 (V1.1)           ONERR                 CHR()

FPROG6 (V1.1)           ONEX1                 CBY()

ONTIME                DBY()

PRINT                 XBY()

PRINT#                GET

PRINT@ (V1.1)         IE

PH0.                  IP

PH0.#                 PORT1

PH0.@ (V1.1)          PCON

PH1.                  RCAP2

PH1.#                 T2CON

PH1.@ (V1.1)          TCON

PGM (V1.1)            TMOD

PUSH                  TIME

POP                   TIMER0

PWM                   TIMER1

REM                   TIMER2

RETI                  XTAL

STOP                  MTOP

STRING                LEN

UI(1&0)               FREE

U0(1&0)               PI

LD@ (V1.1)

ST@ (V1.1)

IDLE (V1.1)

RROM (V1.1)

208

1.6 FLOATING POINT FORMAT

MCS BASIC-52 stores all floating point numbers in  a  normalized  packed

BCD  format  with  an  offset  binary  exponent.  The  simplest  way  to

demonstrate the floating point format is to use an example.  If the num-

ber PI (3.1415926) was stored in location X,  the following would appear

in memory.

LOCATION    VALUE     DESCRIPTION

X         81H      EXPONENT - 81H = 10**1, 82H = 10**2,

80H = 10**0, 7FH = 10** - 1 etc.

THE NUMBER ZERO IS REPRESENTED

WITH A ZERO EXPONENT

X-1       00H      SIGN BIT - 00H = POSITIVE, 01H = NEGATIVE

OTHER BITS ARE USED AS TEMPS

ONLY DURING A CALCULATION

X-2       26H      LEAST SIGNIFICANT TWO DIGITS

X-3       59H      NEXT LEAST SIGNIFICANT TWO DIGITS

X-4       41H      NEXT MOST SIGNIFICANT TWO DIGITS

X-5       31H      MOST SIGNIFICANT TWO DIGITS

Because MCS BASIC-52 normalizes all numbers,  the most significant digit

is never a zero unless the number is zero.

209

1.7 STORAGE ALLOCATION

This section is intended to answer the question - where does MCS  BASIC-

52 store its variables and strings?

Two  16 bit pointers stored in external memory control the allocation of

strings and variables and an additional two pointers control the alloca-

tion of scalar variables and dimensioned variables.  These pointers  are

located and defined as follows:

LOCATION (H-L)    NAME     DESCRIPTION

10AH-10BH         MTOP     THE TOP OF RAM THAT IS ASSIGNED TO

BASIC

104H-105H         VARTOP   VARTOP = MTOP - (THE NUMBER OF BYTES

OF MEMORY THAT THE USER HAS ALLOCATED

FOR STRINGS). IF STRINGS ARE NOT USED,

VARTOP = MTOP

106H-107H         VARUSE   AFTER A NEW, CLEAR, OR RUN IS EXECU-

TED, VARUSE = VARTOP, EVERYTIME THE

USER ASSIGNS OR USES A VARIABLE

VARUSE IS DECREMENTED BY A COUNT OF 8.

108H-109H         DIMUSE   AFTER A NEW, CLEAR, OR RUN IS EXECU-

TED, DIMUSE = [LENGTH OF THE USER

PROGRAM THAT IS IN RAM MEMORY + STAR-

TING ADDRESS OF THE USER PROGRAM IN

RAM (512) + THE LENGTH OF ONE FLOATING

POINT NUMBER (6)]. IF NO PROGRAM IS IN

RAM MEMORY, DIMUSE = 518 AFTER A CLEAR

IS EXECUTED

MCS  BASIC-52  stores string variables between VARTOP and MTOP.  $(0) is

stored from VARTOP to VARTOP + (user defined string length + 1), $(1) is

stored from VARTOP + (user defined string length + 1) + 1 to VARTOP +  2

* (user defined string length + 1) etc.  If MCS BASIC-52 attempts to ac-

cess a string that is outside the bounds established by MTOP,  a  MEMORY

ALLOCATION ERROR is generated.

Now, Scalar variables are stored from VARTOP "down" and Dimensioned vari-

ables  are  stored from DIMUSE "up." When the user dimensions a variable

either implicity or explicity the value of DIMUSE increases by the  num-

ber  of bytes required to store that dimensioned variable.  For example,

if the user executes a DIM A(10) statement, DIMUSE would increase by 66.

This is because the user is requesting  storage  for  11  numbers  (A(0)

through  A(10) and each number requires 6 bytes for storage and 6 * 11 =

66.

210

1.7 STORAGE ALLOCATION

As mentioned in the previous example,  everytime the user defines a  new

variable the VARUSE pointer decrements by a count of 8. Six of the eight

counts  are  due to the memory required to store a floating point number

and the other two counts are the storage required for the variable  name

(i.e. A1, B7, etc). The variable B7 would be stored as follows:

LOCATION  VALUE    DESCRIPTION

X       37H     THE ASCII VALUE - 7, IF B7 WAS A DIMENSIONED

VARIABLE THE MOST SIGNIFICANT BIT OF THIS

LOCATION WOULD BE SET. IN VERSION 1.1 THIS

LOCATION ALWAYS CONTAINS THE ASCII VALUE FOR

THE LAST CHARACTER USED TO DEFINE A VARIABLE

X-1     42H     THE ASCII VALUE - B, IN VERSION 1.1 OF MCS

BASIC-52 THIS LOCATION CONTAINS THE ASCII

VALUE OF THE FIRST CHARACTER USED TO DEFINE

A VARIABLE PLUS 26 * THE NUMBER OF CHARACTERS

USED TO DEFINE A VARIABLE, IF THE VARIABLE

CONTAINS MORE THAN 2 CHARACTERS.

X-2     ??      THE NEXT SIX LOCATIONS WOULD CONTAIN THE

THRU            FLOATING POINT NUMBER THAT THE VARIABLE IS

X-7             ASSIGNED TO, IF THE VARIABLE WAS A SCALAR

VARIABLE. IF THE VARIABLE WAS DIMENSIONED,

X-2 WOULD CONTAIN THE LIMIT OF THE DIMENSION

(I.E. THE MAX. NUMBER OF ELEMENTS IN THE

ARRAY) AND X-3: X-4 WOULD CONTAIN THE BASE

ADDRESS OF THE ARRAY. THIS ADDRESS IS EQUAL

TO THE OLD VALUE OF THE DIMUSE POINTER

BEFORE THE ARRAY WAS CREATED

Whenever a new scalar or dimensioned variable is used in a program,  MCS

BASIC-52 checks both the DIMUSE and VARUSE pointers to  make  sure  that

VARUSE>DIMUSE.  If the relationship is not true, a MEMORY ALLOCATION ER-

ROR is generated.

211

1.7 STORAGE ALLOCATION

To summarize:

Strings are stored from VARTOP to MTOP.

Scalar variables are stored from VARTOP "down" and VARUSE points to  the

next available scalar location.

Dimensioned variables are stored from the end of the user program in RAM

"up."  If no program is in RAM this location is 518.  DIMUSE keeps track

of the number of bytes the user has allocated for dimensioned variables.

If DIMUSE > = VARUSE a MEMORY ALLOCATION ERROR is generated.

212

1.8 FORMAT OF AN MCS BASIC-52 PROGRAM

This section answers  the  question  "How  does  MCS  BASIC-52  store  a

program?"

LINE FORMAT

Each  line of MCS BASIC-52 text consists of tokens and ASCII characters,

plus 4 bytes of overhead.  Three of these four bytes are stored  at  the

beginning of every line. The first byte contains the length of a line in

binary  and  the  second  two  bytes are the line number in binary.  The

fourth byte is stored at the end of the line and this byte is  always  a

0DH or a carriage return in ASCII. An example of a typical line is shown

below, assume that this is the first line of a program in RAM.

10  FOR I = 1 TO 10 : PRINT I : NEXT I

LOCATION   BYTE    DESCRIPTION

512         11H    THE LENGTH OF THE LINE IN BINARY (17D BYTES)

513         00H    HIGH BYTE OF THE LINE NUMBER

514         0AH    LOW BYTE OF THE LINE NUMBER

515        0A0H    THE TOKEN FOR "FOR"

516         49H    THE ASCII CHARACTER "I"

517        0EAH    THE TOKEN FOR "="

518         31H    THE ASCII FOR "1"

519        0A6H    THE TOKEN FOR "TO"

520         31H    THE ASCII FOR "1"

521         30H    THE ASCII FOR "0"

522         3AH    THE ASCII FOR ":"

523         89H    THE TOKEN FOR "PRINT"

524         49H    THE ASCII FOR "I"

525         3AH    THE ASCII FOR ":"

526         97H    THE TOKEN FOR "NEXT"

527         49H    THE ASCII FOR "I"

213

528         0DH    END OF LINE (CARRIAGE RETURN)

TO  FIND THE LOCATION OF THE NEXT LINE,  THE LENGTH OF THE LINE IS ADDED

TO THE LOCATION WHERE THE LENGTH OF THE LINE IS STORED. IN THIS EXAMPLE,

512 + 17D = 529, WHICH IS WHERE THE NEXT LINE IS STORED.

The END of a program is designated by the value 01H. So, in the previous

example if line 10 was the only line in the program,  location 529 would

contain  the  value 01H.  A program simply consists of a number of lines

packed together on one continuous block with the last line ending  in  a

0DH, 01H sequence.

214

1.8 FORMAT OF AN MCS BASIC-52 PROGRAM

EPROM FILE FORMAT

The EPROM FILE format consists of the same line and program format, pre-

viously described except that each program in the EPROM file begins with

the  value 55H.  The value 55H is only used by MCS BASIC-52 to determine

if a valid program is present. If the user types ROM 6, MCS BASIC-52 ac-

tually goes through the first program stored in EPROM line by line until

the END of PROGRAM (01H) is found, then it examines the next location to

see if a 55H is stored in that  location.  It  then  goes  through  that

program line by line. This process is repeated 6 times. If the character

55H  is  not found after the end of a program,  MCS BASIC-52 will return

with the PROM MODE error message.  This would mean that  less  than  six

programs were stored in that EPROM.

The  first  program  stored  in  EPROM (ROM 1) always begins at location

8010H and this location will always  contain  a  55H.  The  actual  user

program will begin at location 8011H.

EPROM locations 8000H through 800FH are reserved by MCS BASIC-52.  These

locations contains initialization information when the PROGX options are

used.  Version 1.0 of MCS BASIC-52 only used the first  three  bytes  of

this  reserved  EPROM area.  The information stored in these bytes is as

follows:

LOCATION   DESCRIPTION

8000H      CONTAINED A 31H IF PROG 1 WAS USED, CONTAINED A 32H IF

PROG 2 WAS USED

8001H      BAUD RATE (RCAP2H)

8002H      BAUD RATE (RCAP2L)

Version 1.1 of MCS BASIC-52 uses the same locations as Version 1.0,  but

additionally locations 8003H and 8004H (high byte, low byte) are used to

store the MTOP information for the PROG 3, 4, 5, 6 options.

IMPORTANT NOTE -

The  PROG X options simply store ASCII character following the PROG com-

mand in location 8000H.  That is why PROG 1 stores  a  31H  in  location

8000H,  PROG 2 a 32H,  PROG 3 (Version 1.1 only) a 33H etc.  If the user

employs the user defined reset option defined  in  Chapter  11  of  this

manual, it would be possible for the user to create unique PROG options.

For  example,  PROG A would store a 41H in location 8000H and upon RESET

the user could examine this location with an assembly  language  routine

and  generate a unique PROG A reset routine for that particular applica-

tion.

215

1.9 ANSWERS TO A FEW QUESTIONS

QUESTION

Why can't MCS BASIC-52 access the 8052-s SPECIAL FUNCTION REGISTER SCON?

ANSWER

The only time the user would likely change the contents of  SCON  is  if

the user is writing custom I/O drivers in assembly language. If the user

is  writing assembly language I/O drivers,  then the user can change the

contents of SCON in assembly language. Changing the contents of SCON can

cause MCS BASIC-52's console routines to crash.

QUESTION

I have written an upload/download routine using my computer,  but when I

download a program, MCS BASIC-52 misses characters, why?

ANSWER

MCS BASIC-52 is actually capable of accepting characters at 38,400 baud.

The  problem is that after MCS BASIC-52 receives a carriage return (cr),

it tokenizes the line of text that was just entered.  Depending  on  how

complicated  and  how  long  the line is,  MCS BASIC-52 can take up to a

couple of hundred milliseconds to tokenize the line.  If the user  keeps

stuffing  characters into the serial port while MCS BASIC-52 is tokeniz-

ing the line, the characters will be lost.  What the user must do in the

download  routine  is  wait  until MCS BASIC-52 responds with the prompt

character (>) after a carriage  return  is  sent  to  the  MCS  BASIC-52

device.  The  prompt  (>) informs the user that MCS BASIC-52 is ready to

receive characters from the console device.

QUESTION

I an writing in assembly language and I notice that the  8052AH  has  no

decrement  DPTR instruction.  What is the easiest,  shortest or simplest

way to decrement the DPTR?

ANSWER

The shortest one we know is:

XCH     A, DPL       ; SWAP A<>DPL

JNZ     DECDP        ; DPH = DPH - 1 IF DPL = 0

DEC     DPH

DECDP: DEC     A            ; DPL = DPL - 1

XCH     A, DPL

This routine affects no flags or registers (except the DPTR) either!

216

1.9 ANSWERS TO A FEW QUESTIONS

QUESTION

After RESET or power-up,  MCS BASIC-52 does not return the proper  value

for MTOP, what's the problem?

ANSWER

Virtually everytime this problem occurs it is because something is wrong

with  the decoding circuitry in the system or one or more of the address

lines to the RAM are open or shorted. The user should make sure that all

of the address lines to the system RAM are connected properly!

A simple memory test can be implemented in the COMMAND  MODE  to  verify

the  addressing  to the RAM.  First set XBY(1000H) = 55,  then walk ones

across the address (i.e. P. XBY (1001H) - P. XBY(1002H) - P.  XBY(1004H)

- P.  XBY(1008H) P.  XBY(1010H)) until all locations are tested.  If for

instance, P. XBY(1008H) returns a result of 55, then address line 3 (A3)

would probably be open or shorted.

217

1.11 8052AH SPECIAL FUNCTION REGISTERS

The following details the operation of the special function registers on

the 8052AH:

SYMBOL

NAME              NAME               ADDRESS        MCS BASIC-52

ACC       Accumulator                   0E0H      NOT ADDRESSABLE

B         B Register                    0F0H      NOT ADDRESSABLE

PSW       Program Status Word           0D0H      NOT ADDRESSABLE

SP        Stack Pointer                  81H      NOT ADDRESSABLE

DPTR      Data Pointer 2 Bytes:

DPH      Low Byte                       82H      NOT ADDRESSABLE

DPL      High Byte                      83H      NOT ADDRESSABLE

P0        Port 0                         80H      NOT ADDRESSABLE

P1        Port 1                         90H      PORT1

P2        Port 2                        0A0H      NOT ADDRESSABLE

P3        Port 3                        0B0H      NOT ADDRESSABLE

IP        Interrupt Priority Control    0B8H      IP

IE        Interrupt Enable Control      0A8H      IE

TMOD      Timer/Counter Mode control     89H      TMOD

TCON      Timer/Counter Control          88H      TCON

T2CON     Timer/Counter 2 Control       0C8H      T2CON

TH0       Timer/Counter 0 High Byte      8CH

}TIMER0

TL0       Timer/Counter 0 Low Byte       8AH

TH1       Timer/Counter 1 High Byte      8DH

}TIMER1

TL1       Timer/Counter 1 Low Byte       8BH

TH2       Timer/Counter 2 High Byte     0CDH

218

}TIMER2

TL2       Timer/Counter 2 Low Byte      0CCH

RCAP2H    T/C 2 Capture Reg. High Byte  0CBH

}RCAP2

RCAP2L    T/C 2 Capture Reg. Low Byte   0CAH

SCON      Serial Control                 98H      NOT ADDRESSABLE

SBUF      Serial Data Buffer             99H      NOT ADDRESSABLE

PCON      Power Control                  87H      NOT ADDRESSABLE

219

1.11 8052AH SPECIAL FUNCTION REGISTERS

PSW: PROGRAM STATUS WORD. ADDRESS 0D0H

CY    AC    FO    RS1    RS0    OV    -    P

CY    PSW.7    Carry Flag.

AC    PSW.6    Auxiliary Carry Flag.

F0    PSW.5    Flag 0 available to the user for general purpose.

RS1   PSW.4    Register Bank selector bit 1.

RS0   PSW.3    Register Bank selector bit 0.

OV    PSW.2    Overflow Flag.

-     PSW.1    RESERVED FOR FUTURE USE.

P     PSW.0    PARITY FLAG.

PCON: POWER CONTROL REGISTER. NOT BIT ADDRESSABLE.

SMOD    -    -     -      -      -    -    -

SMOD  Doubles the baud rate when TIMER 1 is used to generate

the baud rate for the serial port.

The remaining bits of PCON are not implemented on the

MCS BASIC-52 device.

1.11 8052AH SPECIAL FUNCTION REGISTERS

TMOD: Timer/Counter Mode Control Register

(MSB)                                  (LSB)

GATE  C/T   M1   M0    GATE   C/T   M1   M0

TIMER 1                TIMER 0

GATE  Gating control When set.          M1  M0  Operating Mode

220

Timer /Counter "x" is             0   0   MCS-48 Timer

enabled only while /"INTx"                "TLx" serves as

pin is high and "TRx"                     five-bit pres-

control pin is set. When                  scalar.

cleared Timer "x" is

enabled whenever "TRx"            0   1   16 bit Timer/

control bit is set                        Counter "THx"

and "TLx" are

cascaded; there

is no prescalar

-

C/T   Timer or Control Selector         1      0      8-bit  auto-reload

Cleared for Timer operation               timer-counter

(input from internal system               "THx" holds a

clock). Set for Counter                   value which is to

operation (input from "Tx"                be reloaded into

input pin).                               "TLx" each time

overflows.

1   1   (Timer 0) TL0 is

an eight-bit ti-

mer counter con-

trolled by the

standard Timer 0

control bits TH0

is an eight-bit

timer only con-

trolled by Timer

1 control bits.

1   1   (Timer 1) Timer-

counter 1 stopped

221

1.11 8052AH SPECIAL FUNCTION REGISTERS

Timer/Counter 2 Control Register

(MSB)

(LSB)

--      ----

TF2    EXF2    RCLK    TCLK    EXEN2    TR2   C/T2   CP/RLS2

Symbol  Position        Name and Significance

TF2   T2CON.7  Timer 2 overflow flag set by a Timer 2 overflow

and must be cleared by software. TF2 will not

set when either RCLK = 1 or TCLK = 1.

EXF2   T2CON.6 Timer 2 external flag set when either a capture

or reload is caused by a negative transition on

T2EX and EXEN2 = 1. When Timer 2 interrupt is

enabled, EXF2 must be cleared by software.

RCLK   T2CON.5 Receive clock flag. When set, causes the

serial port to use Timer 2 overflow pulses

for its receive clock in modes 1 and 3. RCLK

= 0 causes Timer 1 overflow to be used for

the receive clock.

TCLK    T2CON.4 Transmit clock flag. When set, causes the

serial port to use Timer 2 overflow pulses

for its transmit clock in modes 1 and 3. TCLK =

0 causes Timer 1 overflows to be used for the

transmit clock.

EXEN2   T2CON.3 Timer 2 external enable flag. When set, allows

a capture or reload to occur as a result of a

negative transition on T2EX if Timer 2 is not

being used to clock the serial port. EXEN = 0

causes Timer 2 to ignore events at T2EX.

TR2     T2CON.2 Start/stop control for Timer 2. A logic 1

starts the timer.

--

C/T2    T2CON.1 Timer or counter select. (Timer 2)

0 = Internal timer (OSC/12)

1 = External event counter (falling edge

triggered).

---

CP/RL2    T2CON.0 Capture/Reload flag. When set, captures will

occur on negative transitions at T2EX if

222

EXEN2 = 1. When cleared, auto reloads will

occur either with Timer 2 overflows or

negative transitions at T2EX when EXEN 2 = 1.

When either RCLK = 1 or TCLK = 1, this bit is

ignored and the timer is forced to auto-reload

on Timer 2 overflow.

223

1.11 8052AH SPECIAL FUNCTION REGISTERS

SCON : Serial Port Control Register

(MSB)                         (LSB)

SM0   SM1  SM2   REN  TB8   TI   RI

where SMO, SM1 specify the serial port mode, as follows:

SMO SM1 Mode Description  Baud

Rate

-TB8 is the 9th data bit that

will be transmitted in

0   0   0    shift        fosc./12       modes 2 and 3. Set or

clear by software as

0   1   1    register     variable       as desired.

1   0   2    8-bit UART   fosc./64

9-bit UART   or        -RB8 In modes 2 and 3, is the

fosc./32       9th data bit that was

received. In mode 1, if

SM2 = 0, RB8 is the stop

1   1   3    9-bit UART variable         bit that was received.

In mode 0, RB8 is not

used.

-SM2  enables the multiprocessor     -TI is transmit interrupt

communication feature in modes     flag. Set by hardware

2 and 3. In mode 2 or 3,           at the end of the 8th

if SM2 is set to 1 then RI         bit time in mode 0, or

will not be activated if the       at the beginning of the

received 9th data bit (RB8)        stop bit in the other

is 0. In mode 1, if SM2 = 1        modes, in any serial

then RI will not be activated      transmission. Must be

if a valid stop bit was not        cleared by software.

received. In mode 0, SM2

should be 0.

-REN  enables serial reception. Set  -RI is receive interrupt

software to enable reception.      flag. Set by hardware

Clear by software to disable       at the end of the 8th

reception.                         bit time in mode 0, or

halfway through the stop

bit time in the other

modes, in any serial

reception (except see

SM2). Must be cleared

by software.

224

(MSB)                            (LSB)

TF1  TR1  TF0  TRO  IE1  IT1  IE0  IT0

0    1    0    1    0    1    0    0

TCON: Timer/Counter Control Register

Symbol Position  Name and           Symbol Position  Name and

Significance                        Significance

TF1    TCON.7  Timer 1 overflow      IE1  TCON.3  Interrupt 1

Flag. Set by hardware              Edge flag. Set

on timer/counter over-            by hardware when

flow. Cleared by hard-            external inter-

ware when processor               rupt edge detec-

vectors to interrupt              ted.Cleared when

routine.                          interrupt pro-

cessed.

TR1    TCON.6  Timer 1 Run control   IT1   TCON.2 Interrupt 1

bit. Set/cleared by                Type control

software to turn                   bit. Set/

timer/counter on/off.              cleared by

software to

TF0    TCON.5  Timer 0 overflow Flag.             specify

Set by hardware on                 falling edge/

timer/counter overflow.            low level

Cleared by hardware                triggered ex-

when processor vectors             ternal inter-

to interrupt routine.              rupts.

TR0    TCON.4  Timer 0 Run control   IE0   TCON.1 Interrupt 0

bit. Set/cleared by                Edge flag.

software to turn                   Set by hard-

timer/counter on/off.              ware when ex-

ternal inter-

rupt edge de-

tected. Cleared

when interrupt

processed.

IT0   TCON.0 Interrupt 0

Type control

bit. Set/

cleared by

software to

specify falling

edge/low level

triggered ex-

225

ternal inter-

rupts.

1.11 8052AH SPECIAL FUNCTION REGISTERS

MSB                    LSB

X X PT2 PS PT1 PX1 PT0 PX0

Symbol  Position Function

-     IP.7    reserved

-     IP.6    reserved

PT2    IP.5    defines the Timer 2

interrupt priority

level. PT2 = 1 prog-

rams it to the higher

priority level.

PS     IP.4    defines the Serial Port

interrupt priority level.

PS = 1 programs it to the

higher priority level.

PT1    IP.3    defines the Timer 1

interrupt priority level.

PT1 = 1 programs it to the

higher priority level.

PX1    IP.2    defines the External

Interrupt 1 priority

level. PX1 = 1 programs

it to the higher priority

level.

PT0    IP.1    defines the Timer 0 inter-

rupt priority level. PT0 =

programs it to the higher

priority level.

PX0    IP.0    defines the External Inter-

rupt 0 priority level. PX0

= 1 programs it to the

higher priority level.

IP: Interrupt Priority Register

226

MSB                                   LSB

EA    X   ET2   ES  ET1   EX1   ET0   EX0

Symbol    Position  Function

EA       IE.7     disables all interrupts. If

EA = 0, no interrupt will be

acknowledge. If EA = 1, each

interrupt source is individually

enabled or disabled be setting or

clearing its enable bit.

-        IE.6    reserved

ET2       IE.5    enables or disables the Timer 2

overflow or capture interrupt.

If ET2 = 0, the Timer 2 interrupt

is disabled.

ES        IE.4    enables or disables the Serial Port

interrupt. If ES = 0, the Serial Port

interrupt is disabled.

ET1       IE.3     enables or disables the Timer 1

Overflow interrupt. If ET1 = 0

the Timer 1 interrupt is disabled.

EX1       IE.2     enables or disables External Interrupt

1. If EX1 = 0, External Interrupt 1

is disabled.

ET0       IE.1     enables or disables the Timer 0

Overflow interrupt. If ET0 = 0,

the Timer 0 Interrupt is disabled.

EX0       IE.0     enables or disables External Interrupt 0.

If EX0 = 0, External Interrupt 0 is

disabled.

IE: Interrupt Enable Register

227

1.12 REFERENCES

REFERENCES

J.  Sack  and J.  Meadows,  Entering BASIC,  Science Research Associates

1973.

C.  Pegels,  BASIC: A Computer Programming Language,  Holden-Day,  Inc.,

1973.

J. Kemeny and T. Kurtz BASIC Programming, People Computer Company, 1967.

Albrecht, Finkle, and Brown, BASIC People Computer Company, 1973.

T.  Dwyer, A Guided Tour of Computer Programming in BASIC, Houghton Mif-

flin Co., 1973.

Eugene H.  Barnett,  Programming Time Shared Computers in BASIC,  Wiley-

Interscience, L/C 72-175789.

Programming Language #2, Digital Equipment Corp., Maynard, Mass. 01754.

101 BASIC Computer Games, Digital Equipment Corp., Maynard, Mass. 01754.

What to do After You Hit Return. People Computer Company.

BASIC-80 REFERENCE MANUAL, Intel Corp., Sta. Clara Calif.

228

APPENDIX B

INSTRUCTION SET SUMMARY

This  appendix contains two tables (see tables B-1 and B-2);   the first

identifies all of the 8052's instructions in alphabetical  order;    the

second  table  lists the instructions according to their hexadecimal op-

codes and lists the assembly language instructions  that  produced  that

opcode.

The alphabetical listing also includes documentation of the bit pattern,

flags affected,   number of machines cycles per execution and a descrip-

tion of the instructions operation and function.  The list below defines

the conventions used to identify operation and bit patterns.

ABBREVIATIONS AND NOTATIONS USED

A                   Accumulator

AB                  Register Pair

B                   Multiplication Register

bit address         8052 bit address

page address        11-bit code address within 2K page

relative offset     8-bit 2's complement offset

C                   Carry flag

code address        Absolute code address

data                immediate

data address        On-chip 8-bit RAM address

DPTR                Data pointer

PC                  Program Counter

Rr                  Register (r= 0-7)

SP                  Stack Pointer

high                High order byte

low                 Low order byte

i-j                 Bit i through j

n                   Bit n

aaa aaaaaaaa        Absolute page address encoded in instruction and

operand byte

bbbbbbbb            Bit address encoded in operand byte

dddddddd            Immediate data encoded in operand byte

llllllll            One byte of a 16-bit address encoded in

operand byte

mmmmmmmm            Data address encoded in operand byte

oooooooo            Relative offset encoded in operand byte

r or rrr            Register identifier encoded in operand byte

AND                 Logical AND

NOT                 Logical complement

OR                  Logical OR

XOR                 Logical exclusive OR

229

+                   Plus

-                   Minus

/                   Divide

*                   Multiply

(X)                 The contents of X

((X))               The memory location address by (X)

(The contents of X)

=                   is equal to

<>                  is not equal to

<                   is less than

>                   is greater than

<-                  is replaced by

230

Table B-1, Instruction Set Summary

Mnemonic        Cycles      Binary          Flags          Function

Operation                   Code          P OV AC C

------------------------------------------------------------------------------

ACALL code addr        2    a a a 1 0 0 0 1               Push PC on stack, and

(PC)<- (PC) + 2           a a a a a a a a               replace low order 11

(SP)<- (SP) + 1                                         bits with low order

((SP))<- (PC) low                                       11 bits of code

(SP)<- (SP) + 1                                         address

((PC))<- (PC) high

(PC) 0-10 <-page address

ADD A, #data           1    0 0 1 0 0 1 0 0   P OV AC C   Add immediate data

(A) <- (A) + data         d d d d d d d d               to A.

ADD A, @Rr             1    0 0 1 0 0 1 1 r   P OV AC C   Add contents of

(A) <- (A) + ((Rr))                                     indirect address to A

ADD A, Rr              1    0 0 1 0 1 r r r   P OV AC C   Add contents of

(A) <- (A) + ((Rr))                                     indirect address to A

ADD A, data addr       1    0 0 1 0 0 1 0 1   P OV AC C   Add contents of data

(A)<-(A)+(data addr)      m m m m m m m m               address to A.

ADDC A, #data          1    0 0 1 1 0 1 0 0   P OV AC C   Add C and immediate

(A)<-(A)+(C)+data         d d d d d d d d               data to A

ADDC A,@Rr             1    0 0 1 1 0 1 1 r   P OV AC C   Add C and contents of

(A)<-(A)+(C)+((Rr))                                     indirect address to A

ADDC A,Rr              1    0 0 1 1 1 r r r   P OV AC C   Add C and register

(A)<-(A)+(C)+(Rr)                                       to A

ADDC A,data addr       1    0 0 1 1 0 1 0 1   P OV AC C   Add C and contents of

(A)<-(A)+(C)+(data address)m m m m m m m m               data address to A

AJMP code addr         2    a a a 0 0 0 0 1               Replace low order 11

(A) <- (A) + data         a a a a a a a a               bits of PC with low

with low order 11

bits of code address.

ANL A, #data           1    0 1 0 1 0 1 0 0   P           Logical AND immediate

(A)<-(A) AND data         d d d d d d d d               data to A

ANL A, @Rr             1    0 1 0 1 0 1 1 r   P           Logical AND contents

(A)<-(A) AND ((Rr))                                     of indirect address

to A

ANL A, Rr              1    0 1 0 1 0 1 1 r   P           Logical AND register

(A)<-(A) AND (Rr)                                       to A

231

Table B-1, Instruction Set Summary (Cont'd)

Mnemonic        Cycles      Binary          Flags          Function

Operation                   Code          P OV AC C

------------------------------------------------------------------------------

ANL A, data address    1    0 1 0 1 0 1 0 1           C   Logical AND contents

(A)<-(A) AND (data address) m m m m m m m m               of data address to A

ANL C, bit address     2    1 0 0 0 0 0 1 0           C   Logical AND bit to C

(C)<-(C) AND (bit address)  b b b b b b b b

ANL data addr, #data   2    0 1 0 1 0 0 1 1               Logical AND immediate

(data addr)<-              m m m m m m m m               data to contents of

(data addr)AND data      d d d d d d d d               data address

ANL data addr, A       1    0 1 0 1 0 0 1 0               Logical AND A to

(data addr)<-              m m m m m m m m               contents of

(data  addr)AND data                                      data address

232

Table B-1, Instruction Set Summary (Cont'd)

Mnemonic        Cycles      Binary          Flags          Function

Operation                   Code          P OV AC C

------------------------------------------------------------------------------

CJNE @Rr,#data,        2    1 0 1 1 0 1 1 r           C   If immediate data and

code addr          d d d d d d d d               contents of indirect

(PC)<-(PC) + 3              o o o o o o o o               address are not equal,

IF ((Rr))<> data                                          jump to code address.

THEN

(PC)<-(PC) +

relative offset

IF ((Rr))<data

THEN (C) <- 1

ELSE (C) <- 0

CJNE A,#data,          2    1 0 1 1 0 1 0 0           C   If immediate data and

code addr          d d d d d d d d               A are not equal, jump

(PC)<-(PC) + 3              o o o o o o o o               to code address

IF (A)<> data

THEN

(PC)<-(PC) +

relative offset

IF (A)<data

THEN (C) <- 1

ELSE (C) <- 0

CJNE A,data addr,      2    1 0 1 1 0 1 0 1           C   If contents of data

code addr          m m m m m m m m               address and A are not

(PC)<-(PC) + 3              o o o o o o o o               equal, jump to code

IF (A)<>(data addr)                                       address

THEN

(PC)<-(PC) +

relative offset

IF (A)<(data addr)

THEN (C) <- 1

ELSE (C) <- 0

CJNE Rr,#data addr,    2    1 0 1 1 1 r r r           C   If immediate data and

code addr          d d d d d d d d               register are not equal

(PC)<-(PC) + 3              o o o o o o o o               jump to code

IF (Rr)<>data                                             address

THEN

(PC)<-(PC) +

relative offset

IF (Rr)<data

THEN (C) <- 1

ELSE (C) <- 0

CLR A                  1    1 1 1 0 0 1 0 0   P           Set A to zero (0)

(A)<-0

233

Table B-1, Instruction Set Summary (Cont'd)

Mnemonic        Cycles      Binary          Flags          Function

Operation                   Code          P OV AC C

------------------------------------------------------------------------------

CLR C                  1    1 1 0 0 0 0 1 1           C   Set C to zero (0)

(C)<-0

CLR bit addr           1    1 1 0 0 0 0 1 0               Set bit  to  zero  (0)

(bit addr)<-0             b b b b b b b b

CPL A                  1    1 1 1 1 0 1 0 0   P           Complements each bit

(A)<-NOT(A)                                             in A

CPL C                  1    1 1 1 1 0 1 0 0           C   Complements each bit

(C)<-NOT(C)                                             in C

CPL bit addr           1    1 1 1 1 0 1 0 0               Complement bit.

(bit address)<-           b b b b b b b b

NOT (bit address)

DA A                   1    1 1 0 1 0 1 0 0   P       C   Adjust A after a

BCD add

DEC @Rr                1    0 0 0 1 0 1 1 r               Decrement contents

((Rr))<-((Rr))-1                                        of indirect address

DEC A                  1    0 0 0 1 0 1 0 0   P           Decrement A

(A)<-(A)-1

DEC Rr                 1    0 0 0 1 1 r r r               Decrement register

(Rr)<-(Rr)-1

DEC data addr          1    0 0 0 1 0 1 0 1               Decrement contents

(data address)<-          m m m m m m m m               of data address

(data address)-1

DIV AB                 4    1 0 0 0 0 1 0 0   P OV    C   Divide A by B

(AB)<-(A)/(B)                                           (multiplication

register)

DJNZ Rr, code addr     2    1 1 0 1 1 r r r               Decrement register,

(PC)<-(PC)+2              o o o o o o o o               if not zero(0), then

(Rr)<-(Rr)-1                                            jump to code address

IF (Rr) <> 0

THEN

(PC)<-(PC)+

relative offset

234

Table B-1, Instruction Set Summary (Cont'd)

Mnemonic        Cycles      Binary          Flags          Function

Operation                   Code          P OV AC C

------------------------------------------------------------------------------

DJNZ data addr,        2    1 1 0 1 1 r r r               Decrement data

code addr             m m m m m m m m               address, if zero (0),

(PC)<-(PC)+3              o o o o o o o o               then jump to code

(data addr)<-                                           address

(data addr)-1

IF (data addr) <> 0

THEN

(PC)<-(PC)+

relative offset

INC @Rr                1    0 0 0 0 0 1 1 r               Increment contents

((Rr))<-((Rr))+1                                        of indirect address

INC A                  1    0 0 0 0 0 1 0 0   P           Increment A

(A)<-(A)+1

INC DPTR               1    1 0 1 0 0 0 1 1               Increment 16-bit data

(DPTR)<-(DPTR)+1                                        pointer

INC Rr                 1    0 0 0 0 1 r r r               Increment register

(Rr)<-(Rr)+1

INC data addr          2    0 0 0 0 0 1 0 1               Increment contents of

(data addr)<-             m m m m m m m m               address

(data addr)+1

JB bit addr, code addr 2    0 0 1 0 0 0 0 0               If bit is one, n jump

(PC)<-(PC)+3              b b b b b b b b               to code address

IF (bit address)=1        o o o o o o o o

THEN

(PC)<-(PC)+

relative offset

JBC bit addr,code addr 2    0 0 0 1 0 0 0 0               If bit is one, n jump

(PC)<-(PC)+3              b b b b b b b b               to code address

IF (bit address)=1        o o o o o o o o

THEN

(bit address)<-0

(PC)<-(PC)+

relative offset

235

Table B-1, Instruction Set Summary (Cont'd)

Mnemonic        Cycles      Binary          Flags          Function

Operation                   Code          P OV AC C

------------------------------------------------------------------------------

JC code addr           2    0 1 0 0 0 0 0 0               if C is one, then jump

(PC)<-(PC)+2              o o o o o o o o               to code address

IF (C)=1

THEN

(PC)<-(PC)+

Relative offset

JMP @A + DPTR          2    0 1 1 1 0 0 1 1               Add A to data pointer

(PC)<-(A)+(DPTR)                                        and jump to that code

address

JNB bit addr,code addr 2    0 0 1 1 0 0 0 0               If bit is one, n jump

(PC)<-(PC)+3              b b b b b b b b               to code address

IF (bit address)=0        o o o o o o o o

THEN

(bit address)<-0

(PC)<-(PC)+

relative offset

236

Table B-1, Instruction Set Summary (Cont'd)

Mnemonic        Cycles      Binary          Flags          Function

Operation                   Code          P OV AC C

------------------------------------------------------------------------------

JC code addr           2    0 1 0 1 0 0 0 0               if C is zero(0),

(PC)<-(PC)+2              o o o o o o o o               n jump to code

IF (C)=0                                                address

THEN

(PC)<-(PC)+

Relative offset

JNZ code addr          2    0 1 1 1 0 0 0 0               if A is zero(0),

(PC)<-(PC)+2              o o o o o o o o               n jump to code

IF (A)<>0                                               address

THEN

(PC)<-(PC)+

Relative offset

JNZ code addr          2    0 1 1 0 0 0 0 0               if A is zero(0),

(PC)<-(PC)+2              o o o o o o o o               then jump to code

IF (A)=0                                                address

THEN

(PC)<-(PC)+

Relative offset

LCALL code addr        2    0 0 0 1 0 0 1 0               Push PC on stack, and

(PC)<- (PC) + 3           l l l l l l l l               replace entire PC

(SP)<- (SP) + 1           l l l l l l l l               value with code

((SP))<- (PC) low                                       address

(SP)<- (SP) + 1

((PC))<- (PC) high

(PC)<-code address

LJMP code addr         2    0 0 0 0 0 0 1 0               Jump to code address

(PC)<-code address        l l l l l l l l

l l l l l l l l

MOV @Rr, #data         1    0 1 1 1 0 1 1 r               Move immediate data

((Rr))<-data              d d d d d d d d               indirect address

MOV @Rr, A             1    1 1 1 1 0 1 1 r               Move A to indirect

((Rr))<-(A)                                             address

MOV @Rr, data addr     2    1 0 1 0 0 1 1 r               Move contents of data

((Rr))<-(data addr)       m m m m m m m m               address to indirect

address

MOV A, #data           1    0 1 1 1 0 1 0 0   P           Move immediate data

(A)<-data                 d d d d d d d d               to A

237

Table B-1, Instruction Set Summary (Cont'd)

Mnemonic        Cycles      Binary          Flags          Function

Operation                   Code          P OV AC C

------------------------------------------------------------------------------

MOV A,@Rr              1    1 1 1 0 0 1 1 r   P           Move contents of

(A)<-((Rr))                                             indirect address to A

MOV A,Rr               1    1 1 1 0 1 r r r   P           Move register to A

(A)<-(Rr)

MOV A, data addr       1    1 1 1 0 0 1 0 1   P           Move contents of data

((Rr))<-(data addr)       m m m m m m m m               address to A

MOV C, bit addr        1    1 0 1 0 0 0 1 0           C   Move bit to C

(C)<-(bit addr)           b b b b b b b b

MOV DPTR, #data        2    1 0 0 1 0 0 0 0               Move two bytes of

(DPTR)<-data              d d d d d d d d               immediate data pointer

d d d d d d d d

MOV Rr, #data          1    0 1 1 1 1 r r r               Move immediate data

(Rr)<-data                d d d d d d d d               to register

MOV A,Rr               1    1 1 1 0 1 r r r   P           Move register to A

(A)<-(Rr)

238

Table B-2.  Instruction Opcodes in Hexadecimal

Hex    Number of   Mnemonic           Operands

Code    Bytes

-------------------------------------------------------------------------

00        1          NOP

01        2          AJMP                code addr

02        3          LJMP                code addr

03        1          RR                  A

04        1          INC                 A

05        2          INC                 data addr

06        1          INC                 @R0

07        1          INC                 @R1

08        1          INC                 R0

09        1          INC                 R1

0A        1          INC                 R2

0B        1          INC                 R3

0C        1          INC                 R4

0D        1          INC                 R5

0E        1          INC                 R6

0F        1          INC                 R7

10        3          JBC                 bit addr,code addr

11        2          ACALL               code addr

12        3          LCALL               code addr

13        1          RRC                 A

14        1          DEC                 A

15        2          DEC                 data addr

16        1          DEC                 @R0

17        1          DEC                 @R1

18        1          DEC                 R0

19        1          DEC                 R1

1A        1          DEC                 R2

1B        1          DEC                 R3

1C        1          DEC                 R4

1D        1          DEC                 R5

1E        1          DEC                 R6

1F        1          DEC                 R7

20        3          JB                  bit addr,code addr

21        2          AJMP                code addr

22        1          RET

23        1          RL                  A

24        2          ADD                 A,#data

25        2          ADD                 A,data addr

26        1          ADD                 A,@R0

27        1          ADD                 A,@R1

28        1          ADD                 A,R0

29        1          ADD                 A,R1

2A        1          ADD                 A,R2

2B        1          ADD                 A,R3

2C        1          ADD                 A,R4

2D        1          ADD                 A,R5

2E        1          ADD                 A,R6

239

Hex    Number of   Mnemonic           Operands

Code    Bytes

-------------------------------------------------------------------------

2F        1          ADD                 A,R7

30        3          JNB                 bit addr,code addr

31        2          ACALL               code addr

32        1          RETI

33        1          RLC                 A

34        2          ADDC                A,#data

35        2          ADDC                A,data addr

36        1          ADDC                A,@R0

37        1          ADDC                A,@R1

38        1          ADDC                A,R0

39        1          ADDC                A,R1

3A        1          ADDC                A,R2

3B        1          ADDC                A,R3

3C        1          ADDC                A,R4

3D        1          ADDC                A,R5

3E        1          ADDC                A,R6

3F        1          ADDC                A,R7

40        2          JC                  code addr

41        2          AJMP                code addr

42        2          ORL                 data addr,A

43        3          ORL                 data addr,#data

44        2          ORL                 A,#data

45        2          ORL                 A,data addr

46        1          ORL                 A,@R0

47        1          ORL                 A,@R1

48        1          ORL                 A,R0

49        1          ORL                 A,R1

4A        1          ORL                 A,R2

4B        1          ORL                 A,R3

4C        1          ORL                 A,R4

4D        1          ORL                 A,R5

4E        1          ORL                 A,R6

4F        1          ORL                 A,R7

50        2          JNC                 code addr

51        2          ACALL               code addr

52        2          ANL                 data addr,A

53        3          ANL                 data addr,#data

54        2          ANL                 A,#data

55        2          ANL                 A,data addr

56        1          ANL                 A,@R0

57        1          ANL                 A,@R1

58        1          ANL                 A,R0

59        1          ANL                 A,R1

5A        1          ANL                 A,R2

5B        1          ANL                 A,R3

5C        1          ANL                 A,R4

5D        1          ANL                 A,R5

5E        1          ANL                 A,R6

5F        1          ANL                 A,R7

240

Hex    Number of   Mnemonic           Operands

Code    Bytes

-------------------------------------------------------------------------

60        2          JZ                  code addr

61        2          AJMP                code addr

62        2          XRL                 data addr,A

63        3          XRL                 data addr,#data

64        2          XRL                 A,#data

65        2          XRL                 A,data addr

66        1          XRL                 A,@R0

67        1          XRL                 A,@R1

68        1          XRL                 A,R0

69        1          XRL                 A,R1

6A        1          XRL                 A,R2

6B        1          XRL                 A,R3

6C        1          XRL                 A,R4

6D        1          XRL                 A,R5

6E        1          XRL                 A,R6

6F        1          XRL                 A,R7

70        2          JNZ                 code addr

71        2          ACALL               code addr

72        2          ORL                 C,bit addr

73        1          JMP                 @A+DPTR

74        2          MOV                 A,#data

75        3          MOV                 data addr,#data

76        2          MOV                 @R0,#data

77        2          MOV                 @R1,#data

78        2          MOV                 R0,#data

79        2          MOV                 R1,#data

7A        2          MOV                 R2,#data

7B        2          MOV                 R3,#data

7C        2          MOV                 R4,#data

7D        2          MOV                 R5,#data

7E        2          MOV                 R6,#data

7F        2          MOV                 R7,#data

80        2          SJMP                code addr

81        2          AJMP                code addr

82        2          ANL                 C,bit addr

83        1          MOVC                A,@A+PC

84        1          DIV                 AB

85        3          MOV                 data addr,data addr

86        2          MOV                 data addr,@R0

87        2          MOV                 data addr,@R1

88        2          MOV                 data addr,R0

89        2          MOV                 data addr,R1

8A        2          MOV                 data addr,R2

8B        2          MOV                 data addr,R3

8C        2          MOV                 data addr,R4

8D        2          MOV                 data addr,R5

8E        2          MOV                 data addr,R6

8F        2          MOV                 data addr,R7

90        3          MOV                 DPTR,#data

241

Hex    Number of   Mnemonic           Operands

Code    Bytes

-------------------------------------------------------------------------

91        2          ACALL               code addr

92        2          MOV                 bit addr,C

93        1          MOVC                A,@A+DPT

94        2          SUBB                A,#data

95        2          SUBB                A,data addr

96        1          SUBB                A,@R0

97        1          SUBB                A,@R1

98        1          SUBB                A,R0

99        1          SUBB                A,R1

9A        1          SUBB                A,R2

9B        1          SUBB                A,R3

9C        1          SUBB                A,R4

9D        1          SUBB                A,R5

9E        1          SUBB                A,R6

9F        1          SUBB                A,R7

A0        2          ORL                 C,/bit addr

A1        2          AJMP                code addr

A2        2          MOV                 C,bit addr

A3        1          INC                 DPTR

A4        1          MUL                 AB

A5                   reserved

A6        2          MOV                 @R0,data addr

A7        2          MOV                 @R1,data addr

A8        2          MOV                 R0,data addr

A9        2          MOV                 R1,data addr

AA        2          MOV                 R2,data addr

AB        2          MOV                 R3,data addr

AC        2          MOV                 R4,data addr

AD        2          MOV                 R5,data addr

AE        2          MOV                 R6,data addr

AF        2          MOV                 R7,data addr

B0        2          ANL                 C,/bit addr

B1        2          ACALL               code addr

B2        2          CPL                 bit addr

B3        1          CPL                 C

B4        3          CJNE                A,#data addr,code addr

B5        3          CJNE                A,data addr,code addr

B6        3          CJNE                @R0,#data,code addr

B7        3          CJNE                @R1,#data,code addr

B8        3          CJNE                R0,#data,code addr

B9        3          CJNE                R1,#data,code addr

BA        3          CJNE                R2,#data,code addr

BB        3          CJNE                R3,#data,code addr

BC        3          CJNE                R4,#data,code addr

BD        3          CJNE                R5,#data,code addr

BE        3          CJNE                R6,#data,code addr

BF        3          CJNE                R7,#data,code addr

C0        2          PUSH                data addr

C1        2          AJMP                code addr

242

Hex    Number of   Mnemonic           Operands

Code    Bytes

-------------------------------------------------------------------------

C2        2          CLR                 bit addr

C3        1          CLR                 C

C4        1          SWAP                A

C5        2          XCH                 A,data addr

C6        1          XCH                 A,@R0

C7        1          XCH                 A,@R1

C8        1          XCH                 A,R0

C9        1          XCH                 A,R1

CA        1          XCH                 A,R2

CB        1          XCH                 A,R3

CC        1          XCH                 A,R4

CD        1          XCH                 A,R5

CE        1          XCH                 A,R6

CF        1          XCH                 A,R7

D0        2          POP                 data addr

D1        2          ACALL               code addr

D2        2          SETB                bit addr

D3        1          SETB                C

D4        1          DA                  A

D5        3          DJNZ                data addr,code addr

D6        1          XCHD                A,@R0

D7        1          XCHD                A,@R1

D8        2          DJNZ                R0,code addr

D9        2          DJNZ                R1,code addr

DA        2          DJNZ                R2,code addr

DB        2          DJNZ                R3,code addr

DC        2          DJNZ                R4,code addr

DD        2          DJNZ                R5,code addr

DE        2          DJNZ                R6,code addr

DF        2          DJNZ                R7,code addr

E0        1          MOVX                A,@DPTR

E1        2          AJMP                code addr

E2        1          MOVX                A,@R0

E3        1          MOVX                A,@R1

E4        1          CLR                 A

E5        2          MOV                 A,data addr

E6        1          MOV                 A,@R0

E7        1          MOV                 A,@R1

E8        1          MOV                 A,R0

E9        1          MOV                 A,R1

EA        1          MOV                 A,R2

EB        1          MOV                 A,R3

EC        1          MOV                 A,R4

ED        1          MOV                 A,R5

EE        1          MOV                 A,R6

EF        1          MOV                 A,R7

F0        1          MOVX                @DPTR,A

F1        2          ACALL               code addr

F2        1          MOVX                @R0,A

243

Hex    Number of   Mnemonic           Operands

Code    Bytes

-------------------------------------------------------------------------

F3        1          MOVX                @R1,A

F4        1          CPL                 A

F5        2          MOV                 data addr,A

F6        1          MOV                 @R0,A

F7        1          MOV                 @R1,A

F8        1          MOV                 R0,A

F9        1          MOV                 R1,A

FA        1          MOV                 R2,A

FB        1          MOV                 R3,A

FC        1          MOV                 R4,A

FD        1          MOV                 R5,A

FE        1          MOV                 R6,A

FF                1                    MOV                               R7,A

244

INDEX

A

ABS

Accumulator

ADD

Argument Stack

Arithmetic Overflow

Arithmetic Underflow

Array Size

ASC

Assembly Language Linkage

ATN

Auto_Baud

B

BAUD Rate

C

CALL

Carry Bit

CBY

CHR

CLEAR

CLEARI

CLEARS

CLOCK0

CLOCK1

Command Mode

Command/Statement Extension

Constants

CONT

Control Stack

COS

CR

D

DATA

Data Format

DBY

DIM

DIMUSE

Direct Memory Access (DMA)

DIVIDE

DO_UNTIL

DO_WHILE

DPTR

E

END

EPROM Programming

EQUAL

245

Error Messages

EXCLUSIVE OR

EXP

EXPONENT

Expression

F

Floating Point Numbers

FOR_TO_{STEP}_NEXT

FPROG

FPROG1

FPROG2

FPROG3

FPROG4

FPROG5

FPROG6

FREE

G

GET

GOSUB

GOTO

GREATER THAN

GREATER THAN OR EQUAL

I

IDLE

IE

IF_THEN_ELSE

Illegal Direct

INPUT

Input Buffer

INT

Integers

INTELligent Algorithm

Internal Stack

Interrupts

IP

L

LD@

LEN

LESS THAN

LESS THAN OR EQUAL

LET

Line Editor

LIST

LIST#

LIST@

LOG

LOGICAL AND

LOGICAL EXCLUSIVE OR

246

LOGICAL OR

M

MTOP

MULTIPLY

N

NEGATION

NEW

NOT

NOT EQUAL

NULL

O

ON GOSUB

ON GOTO

ONERR

ONEX1

ONTIME

ON_GOSUB

ON_GOTO

Opbyte

Operators

P

PCON

PGM

PH0.

PH0.#

PH0.@

PH1.

PH1.#

PH1.@

PI

POP

PORT1

PRINT

PRINT#

PRINT@

PROG

PROG1

PROG2

PROG3

PROG4

PROG5

PROG6

Programming Error

PSW

PUSH

PWM

R

247

RAM

RAM Only Mode

RAM/EPROM Mode

RCAP2

READ

REM

Reset

RESTORE

RETI

RETURN

RND

ROM

RROM

Run Mode

Run Trap

S

SCON

Serial Port

SGN

Sign-On

SIN

SMOD

SPC

SQR

ST@

Stack Pointer

STOP

STRING

SUBTRACT

T

T2CON

TAB

TAN

TCON

Text Pointer

TIME

TIMER0

TIMER1

TIMER2

TMOD

U

UI

UNTIL

UO

USING

V

Variables

VARTOP

248

VARUSE

X

X-OFF

X-ON

XBY

XFER

XTAL

249